SA WG2 Meeting #S2-136	S2-1911445

18 - 22 November, 2019, Reno, Nevada, USA

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0096 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UE Communication Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BUPT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some changes are needed in order to be inline with the terminology of other clauses of TS 23.288, to limit the signalling overhead caused by notifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution proposes to add the UL/DL data packet length to the input data of the UE Communication Analytics. Accordingly, the Output Analytics can provide the UL/DL average data packet length in the UE Communication Analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Discussion

In the current specification, the NWDAF can collect service data from AF related to UE communication in the UE Communication Analytics.

In some circustances, it needs NWDAF to provide the UL/DL data packet length for analysing user traffic or resource allocation, but the analytic results of UE communication don’t include it.

This contribution proposes to add the UL/DL data packet length to the input data of the UE Communication Analytics. Accordingly, the Output Analytics can provide the UL/DL average data packet length in the UE Communication Analytics.

* * * Start of Changes * * *

6.7.3	UE Communication Analytics

#### 6.7.3.1	General

In order to support some optimized operations, e.g. customized mobility management or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF and SMF that corresponds to this application ID.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytic Filter Information containing optional list of analytics subset(s)

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

#### 6.7.3.2	Input Data

The NWDAF supporting data analytic on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.7.3.2-1.

Table 6.7.3.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be GPSI or external UE ID |
| Internal Group ID |  | To identify UE group if available |
| Correlation information | AF | Identifying the application generating the traffic |
| Application ID | AF | Identifying the application providing this information |
| Communication pattern parameters | AF | Same as communication pattern specified in TS 23.502 [3] |
| UE communication (1..max) | AF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication expected by the application |
| >DL data rate |  | DL data rate of this communication expected by the application |
| >UL data packet length |  | UL data packet length of this communication |
| >DL data packet length |  | DL data packet length of this communication |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

The correlation information may be used to correlate the communication information collected from an AF with the communication information collected from SMF(s). Other correlations can be made within the NWDAF using UE communication time pattern.

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE communication information).

#### 6.7.3.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF includes the UE communication statistics or predictions.

UE communication statistics are defined in the Table 6.7.3.3-1.

Table 6.7.3.3-1: UE Communication Statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication (average and variance). Example: Time: 13:00-20:00, Day: Monday |
| > Start time | Start time observed (average and variance) |
| > Duration time | Duration interval time of communication (average and variance). |
| > Traffic characterization | DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > UL average data packet length | average data packet length in the UL direction |
| > DL average data packet length | average data packet length in the DL direction |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

UE communication predictions are defined in the Table 6.7.3.3-2.

Table 6.7.3.3-2: UE Communication Predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication. Example: Time: 13:00-20:00, Day: Monday. |
| > Start time | Start time predicted (average and variance). |
| > Communication duration time | Duration interval time of communication. |
| > Traffic characterization | DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > UL average data packet length | average data packet length in the UL direction |
| > DL average data packet length | average data packet length in the DL direction |
| > Confidence | Confidence of the prediction. |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group for a given communication at a given time and duration.

The number of UE communication entries (1..max) is limited by the maximum number of results provided as input parameter. The communications shall be provided by order of time, possibly overlapping.

Depending on the list size limitation, the least probable communications on a given observation period may not be provided.

The analytics subset Periodic communication information shall be provided only if the parameter Periodicity Detection was requested.

* * * End of changes * * *