SA WG2 Meeting #136 	S2-1911453

18 - 22 Nov 2019, Reno, USA	 (revision of S2-190xxxx)

Source:	CATT

Title:	Add new term and key issue about MBS Coverage Area

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: this paper proposes to add a new term and a new key issue about MBS Coverage Area.

Discussion

The following terms are introduced in SA2 meeting #135:

Broadcast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

Multicast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data) .

Broadcast/multicast coverage area is present in the terms above, but the meaning of coverage area is not clear. So it is proposed to add the term of MBS Coverage Area in clause 3.1.

How to use the MBS Coverage Area in 5GS should also be considered, for example, which NF determines the area, how to update the area, whether the MBS Coverage Area is impacted by network slicing. So it is proposed to add a new key issue about the MBS Coverage Area.

Proposal

It is proposed to add a new Term and a new key issue about MBS Coverage Area.

First change

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Broadcast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

Multicast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data) .

NOTE:	The transport in the 5GC for broadacst and multicast services will be determined as part of this study. A multicast communication service could for instance use a lower layer unicast or multicast transfer in the 5GC and the access interfaces.

Broadcast session: A session to deliver the broadcast communication service. A broadcast session is characterised by the content to send and the geographical area where to distribute it.

Multicast session: A session to deliver the multicast communication service. A multicast session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

MBS session: A multicast session or a broadcast session.

MBS Coverage Area: The area within which data of a specific MBS session are sent.

Second change

5.X	Key Issue #X: MBS Coverage Area support

5.X.1	Description

MBS Coverage Area is the area within which data of a specific MBS session are sent. This key issue aims at studying:

-	Which NF determines the MBS Coverage Area?

-	Which entity enforces the MBS Coverage Area and how?

-	How to update the MBS Coverage Area?

-	Whether and how MBS Coverage Area is impacted by network slicing?

End of changes