SA WG2 Meeting #136 	S2-1912744

18 - 22 Nov 2019, Reno, USA	 (revision of S2-1911453, 1912446, 1912697)

Source:	CATT, Samsung

Title:	Add new term and key issue about MBS Coverage Area

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: this paper proposes to add a new term and a new key issue about MBS Coverage Area.

Discussion

The following terms are introduced in SA2 meeting #135:

Broadcast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

Multicast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data) .

Broadcast/multicast coverage area is present in the terms above, but the meaning of coverage area is not clear. So it is proposed to add the term of MBS Coverage Area in clause 3.1.

How to use the MBS Coverage Area in 5GS should also be considered, for example, which NF determines the area, how to update the area, whether the MBS Coverage Area is impacted by network slicing. So it is proposed to add a new key issue about the MBS Coverage Area.

Proposal

It is proposed to add a new Term and a new key issue about MBS Coverage Area.

First change

5.1	Key Issue #1: MBS session management

5.1.1	Description

To transfer the service data flow of multicast and broadcast communication services, MBS sessions need to be established and maintained. This key issue will study the following aspects:

-	When and how to trigger the establishment of an MBS session?

-	Which network function(s) is/are responsible for establishing and maintaining the MBS session (including interaction between 5GC and 3rd party)?

-	Which information/parameters are used to establish an MBS session (e.g., session related IDs, QoS information, MBS service area, etc.)?

-	Which information/parameters can be updated for an established MBS session (e.g. QoS information, MBS service area, etc.)?

-	Which network function(s) is/are responsible for determining the MBS service area used for MBS sessions? How is the MBS service area determined (e.g. pre-defined, determined during an on-going MBS session, etc.)?

End of changes