SA WG2 Meeting #S2-137E	S2-2001830

24 - 27 February, 2020, Electronic meeting	(revision of S2-2001508)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2069 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying UL configuration issue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the 5GS supporting TSN, there might be many PDU sessions sharing a single egress port in uplink (i.e., NW-TT port/UPF side). Therefore, scheduled traffic [IEEE 802.1Q-2018] at egress port is not sufficient to identify which PDU session(s) need to be configured. This indicates what is required to identify which PDU session(s) to configure in Uplink based on per-stream filtering and policing parameters [IEEE 802.1Q-2018]. Refer to paper S2-1909403 for details. Moreover, in order to support the aforementioned calculations and for general correct use of PSFP parameters, the following is mandatory to be supported: - Identifiers of stream filters (StreamHandleSpec) and stream gates (StreamGateInstance) – and the set of parameters per stream (Stream Filter Instance tables), and the set of parameters per gate (Stream Gate Instance tables). Clause 8.6.5.1 IEEE 802.1Q-2018 – otherwise it is impossible to identify to which stream filter or gate instance table of parameters we are referring to. - “Support of PSFP requires implementation of the Stream identification function specified in Clause 6 of IEEE Std 802.1CB-2017 [B14], as the stream_handle provided by this function is used in the policing and queuing decisions taken by PSFP.” Clause 8.6.5.1 IEEE 802.1Q-2018. This is a mandatory condition by IEEE, otherwise it is impossible to identify the Stream. - Maximum stream parameters should be exposed to the TSN system, by using the corresponding managed object “Stream Parameter Table”, clause 12.31.1 IEEE 802.1Q-2018. Necessary for the TSN System to know the maximum of number of streams and gates that a bridge can handle. It is required to be exposed/reported by every bridge (including the 5GS virtual bridge). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a clear indication on the identification of the PDU session to be configured in UL, using PSFP. Adding stream parameters to be exposed in clause 5.28.2. Addition of support for the Stream Identification function as specified by 802.1CB-2017. Adding stream filter table, stream gate table, and parameter StreamGateInstance in the Port Managemet Information container. Changes for rev3: Clarified that the use of PSFP is mandatory only at the TSN AF and can optionally be used at the DS-TT (if supported) and NW-TT (if supported) for the purpose of per-stream policing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL issue solution is not specified. Impossible to identify information related to a stream. Impossible to support PSFP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1, 5.28.2, 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***BEGIN CHANGES***

### 5.28.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the ports on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the logical TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side, a port on UPF/NW-TT side, and a PDU Session during reporting of logical TSN bridge information.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single virtual bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5G virtual bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 1:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based virtual bridge

NOTE 2:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple logical TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge Address (unique MAC address that identifies the bridge used to derive the bridge ID);

-	Bridge Name;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97]:

- Chassis ID subtype and Chassis ID of the 5GS Bridge.

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

-	Stream Parameters as defined in clause 12.31.1 in IEEE 802.1Q [98], in order to support PSFP information:

- Maximum number of filters, which defines the maximum number of streams that the bridge can handle;

- Maximum number of gates, which can be equal or less than the maximum number of filters;

- Maximum number of meters (optional) if meassurements are required;

- Maximum length of the PSFPAdminControlList parameter that can be handled.

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Bridge ID of the 5GS Bridge, port numbers of the Ethernet in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment and stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports in DS-TT and NW-TT of the related PDU session to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released, then SMF informs TSN AF accordingly.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

***NEXT CHANGE***

### 5.28.2	QoS parameters mapping for 5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session. The QoS parameters mapping for TSN is described in TS 23.503 [45] clause 6.1.3.23.

The configuration information of 5GS Bridge as defined in IEEE 802.1Qbv [96] and in IEEE 802.1Q [98], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes and their priorities.

NOTE 1:	In this Release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS.

The configuration information of 5GS Bridge as defined in 802.1Q [98], includes the following:

-	Chassis ID of 5GS Bridge;

-	Traffic forwarding information as defined in IEEE 802.1Q [98] clause 8.8.1:

-	Destination MAC address and VLAN ID of TSN stream;

-	Port number in the Port MAP as defined in IEEE 802.1Q [98] clause 8.8.1.

-	Configuration information per stream according to IEEE 802.1Q [98] clause 8.6.5.1:

-	Ingress port number of 5GS Bridge, i.e., ports on DS-TT/NW-TT;

-	Stream priority.

NOTE 2:	In order to support 802.1Q [98] clause 8.6.5.1, it is required to support the Stream Identification function as specified by 802.1CB-2017 [83].

The SMF report the MAC address of the DS-TT port of the related PDU Session to AF via PCF as the MAC address of the PDU Session. The association between the MAC address used by the PDU Session, 5GS Bridge ID and port number on DS-TT is maintained at AF and further used to assist to bind the TSN traffic with the UE's PDU session.

In the case of provisioning traffic routing information based the information above, the TSN AF determines the MAC address used by the PDU Session for the TSN traffic based on ingress port number or egress port number, and requests to PCF influence traffic routing for TSN traffic as defined in clause 5.6.7 with the MAC address of the PDU Session, traffic class, destination MAC address and VLAN ID.

***NEXT CHANGE***

#### 5.28.3.1	General

Bridge management information is exchanged between CNC and TSN AF. A subset of bridge management information, referred to as port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively inside a Port Management Information Container. Table 5.28.3.1-1 lists standardized port management information.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see Note 6) |  | Supported operations by TSN AF (see Note 1) | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT |  |  |
| General |  |  |  |  |
| Port management capabilities (see Note 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminBaseTime | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminControlList | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminCycleTime (see Note 3) | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminControlListLength (see Note 3) | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| Tick granularity | X | X | R | IEEE 802.1Qbv [96] Table 12-28 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | (x) | X | RW | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | (x) | (X) | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | (x) | (X) | RW | IEEE 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| TTL | (x) | X | R | IEEE 802.1AB[97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (see Note 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE 802.1AB [97] clause 8.5.4.1 |
| Per-Stream Filtering and Policing information (NOTE 9) |  |  |  |  |
| Stream Filter Instance Table (NOTE 7) |  |  |  | IEEE 802.1Q [98] Table 12-32 |
| StreamHandleSpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PrioritySpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| StreamGateInstanceID | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 8) |  |  |  | IEEE 802.1Q [98] Table 12-33 |
| StreamGateInstance | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT and NW-TT performs neighbor discovery on behalf on DS-TT. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, which performs neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; (x) = applicable when validation and generation of LLDP frames is processed at the DS-TT; (X) = applicable when validation and generation of LLDP frames is processed centrally at NW-TT. NOTE 7: There is a Stream Filter Instance Table per Stream. NOTE 8: There is a Stream Gate Instance Table per Gate. NOTE 9: The use of PSFP information is mandatory at the TSN AF and is optional at both DS-TT and NW-TT. TSN AF uses the PSFP information at TSN bridge configuration time to identify the PDU Session as described in clause 5.28.2 and for determination of the TSCAI information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in IEEE 802.1Q [98] clause 8.6.5.1. |  |  |  |  |

Exchange of port management information between TSN AF and NW-TT or DS-TT allows TSN AF to:

1)	retrieve port management information for a DS-TT or NW-TT Ethernet port;

2)	send port management information for a DS-TT or NW-TT Ethernet port;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT Ethernet port changes.

Exchange of port management information between TSN AF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF indicates inside the Port Management Information Container whether it wants to retrieve or send port management information or intends to (un-)subscribe for notifications.

***END OF CHANGES***