3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002219

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2147 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NSSAA for two AMFs serving UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR proposes the following changes: - Add the scenario of two AMFs serving UE for NSSAA The UE can be registered to two serving AMFs in different PLMNs via different accesses. During Network Slice-Specific Re-authentication and Re-authorization or Revocation, when the AUSF requests the serving AMF ID from UDM, the UDM may return both AMF IDs to the AUSF in case of two AMFs serving UE in order to ensure both AMFs can be involved in the Re-authentication and Re-authorization or Revocation and then determine whether to update Allowed NSSAI. - Update the Nudm_UECM_Get service operation The Access Type is currently required as the input for the Nudm_UECM_Get service operation. However, in case of Network Slice-Specific Re-authentication and Re-authorization or Revocation, the AUSF need not to provide any Access Type to the UDM when querying the serving AMF ID. So the Access Type should be optional as the input for the Nudm_UECM_Get. In addition, the AUSF as the Consumer and the identities of two AMFs serving UE if available as the outputs should be supported for this service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The scenario that UE registered to two serving AMFs in different PLMNs via different accesses is missing in the NSSAA procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to handle the NSSAA procedures in case of UE registered to two serving AMFs in different PLMNs via different accesses is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9.3, 4.2.9.4, 5.2.3.1, 5.2.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.9.3	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure

Figure 4.2.9.3-1: AAA Server initiated Network Slice-Specific Re-authentication and Re-authorization procedure

1.	The AAA-S requests the re-authentication and re-authorization for the Network Slice specified by the S-NSSAI in the AAA protocol Re-Auth Request message, for the UE identified by the GPSI in this message. This message is sent to a AAA-P, if the AAA-P is used (e.g. the AAA Server belongs to a third party), otherwise it is sent directly to the AUSF.

2.	The AAA-P, if present, relays the request to the AUSF.

3a-3b.	AUSF gets AMF ID from UDM using Nudm_UECM_Get with the GPSI in the received AAA message. If there are two AMFs serving the UE, both AMF IDs may be returned by the UDM.

4.	The AUSF notifies Re-auth event to the AMF(s) received from UDM to re-authenticate/re-authorize the S-NSSAI for the UE using Nausf_NSSAA_Notify with the GPSI and S-NSSAI in the received AAA message. The callback URI of the notification for the AMF is derived via NRF as specified in TS 29.501 [62].

5.	The AMF triggers the Network Slice-Specific Authentication and Authorization procedure defined in clause 4.2.9.1.

# * * * * Third change * * * *

4.2.9.4	AAA Server triggered Slice-Specific Authorization Revocation

Figure 4.2.9.4-1: AAA Server-initiated Network Slice-Specific Authorization Revocation procedure

1.	The AAA-S requests the revocation of authorization for the Network Slice specified by the S-NSSAI in the AAA protocol Revoke Auth Request message, for the UE identified by the GPSI in this message. This message is sent to AAA-P if it is used.

2.	The AAA-P, if present, relays the request to the AUSF.

3a-3b.	AUSF gets AMF ID from UDM using Nudm_UECM_Get with the GPSI in the received AAA message. If there are two AMFs serving the UE, both AMF IDs may be returned by the UDM.

4.	The AUSF notifies Revoke Auth event to the AMF(s) received from UDM to revoke the S-NSSAI authorization for the UE using Nausf_NSSAA_Notify with the GPSI and S-NSSAI in the received AAA message. The callback URI of the notification for the AMF is derived via NRF as specified in TS 29.501 [62].

5.	The AMF updates the UE configuration to revoke the S-NSSAI from the current Allowed NSSAI, for any Access Type for which Network Slice Specific Authentication and Authorization had been successfully run on this S-NSSAI. The UE Configuration Update may include a request to Register if the AMF needs to be re-allocated. The AMF provides a new Allowed NSSAI to the UE by removing the S-NSSAI for which authorization has been revoked. The AMF provides new rejected NSSAIs to the UE including the S-NSSAI for which authorization has been revoked. If no S-NSSAI is left in Allowed NSSAI for an access after the revocation, and a Default NSSAI exists that requires no Network Slice Specific Authentication or for which a Network Slice Specific Authentication did not previously fail over this access, then the AMF may provide a new Allowed NSSAI to the UE containing the Default NSSAI. If no S-NSSAI is left in Allowed NSSAI for an access after the revocation, and no Default NSSAI can be provided to the UE in the Allowed NSSAI or a previous Network Slice Specific Authentication failed for the Default NSSAI over this access, then the AMF shall execute the Network-initiated Deregistration procedure for the access as described in clause 4.2.2.3.3, and it shall include in the explicit De-Registration Request message the list of Rejected S-NSSAIs, each of them with the appropriate rejection cause value.

# * * * * Third change * * * *

5.2.3.1	General

The following table illustrates the UDM Services and Service Operations.

Table 5.2.3.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscriber Data | Get | Request/Response | AMF, SMF, SMSF, NEF |
| Management | Subscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF |
| (SDM) | Unsubscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF |
|  | Notification | Subscribe/Notify | AMF, SMF, SMSF, NEF |
|  | Info | Request/Response | AMF, NEF |
|  | Update | Request/Response | NEF |
| UE Context | Registration | Request/Response | AMF, SMF, SMSF |
| Management | DeregistrationNotification | Subscribe/Notify | AMF |
| (UECM) | Deregistration | Request/Response | AMF, SMF, SMSF |
|  | Get | Request/Response | NEF, SMSF, GMLC, NWDAF, AUSF |
|  | Update | Request/Response | AMF, SMF |
|  | PCscfRestoration | Subscribe/Notify | AMF, SMF |
| UE | Get | Request/Response | AUSF |
| Authentication | ResultConfirmation | Request/Response | AUSF |
| EventExposure | Subscribe | Subscribe/Notify | NEF (NOTE), NWDAF |
|  | Unsubscribe |  | NEF (NOTE), NWDAF |
|  | Notify |  | NEF (NOTE), NWDAF |
| Parameter | Update | Request/Response | NEF, AMF |
| Provision | Create | Request/Response | NEF |
|  | Delete | Request/Response | NEF |
|  | Get | Request/Response | NEF |
| NIDDAuthorisation | Get | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| NOTE: Other NFs are allowed to consume the service based on roaming agreement or operator policy. |  |  |  |

# * * * *Fourth change* * * *

5.2.3.2.4	Nudm_UECM_Get service operation

Service operation name: Nudm_UECM_Get.

Description: The NF consumer requests the UDM to get the NF ID or SMS address of the NF serving the UE.

Inputs, Required: UE ID, NF Type.

Inputs, Optional: Access Type.

-	Access Type may be included when the NF type indicates AMF.

-	Access Type is included when the NF type indicates SMSF.

Outputs, Required: SUPI, NF ID(s) or SMS address of the NF corresponding to the NF type requested by NF consumer.

If this service operation is invoked during AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure or AAA Server triggered Slice-Specific Authorization Revocation, then both AMF IDs may be provided if there are two AMFs serving the UE.

Outputs, Optional: None.

# * * * * End of changes * * * *