SA WG2 Meeting #S2-137E	S2-2002088

# 24 - 28 February, 2020, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2126 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF provides URC format to UCMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UCMF is expected to provide UE radio capability(URC) to the AMF in 38.xxx format and to the MME in 36.xxx format. To perform this UCMF should know the format in which it has received the URC. When provisioning happens from AF its not clear to UCMF which format of URC is provide. Thus its proposed that AF should indicate the Rat type(format of URC) to the UCMF. During normal operation when MME/AMF sends the URC to the UCMF, UCMF can identify the URC format based on the source node from which it is receiving the URC. i.e. if its AMF URC is always in 38.xxx and if its MME URC is always in 36.xxx. However to follow service based API principles and expectation that this APIs can be used by any CN node and explicit Coding format is included in the APIs indicating which format the URC is provided and which format the URC is expected from the UCMF. A wording error on the AMF and NB-IoT RAC is corrected |  |  |  |  |  |  |  |  |
| Summary of change: |  | . AF should indicate the Rat type(format of URC whether its 38.331 or 36.331 format) to the UCMF The NF while assigning the URC(and receiving the RAC ID) will indicate to UCMF in which format this URC is. The NF while receiving the URC will indicate to UCMF in which format this URC is expected to be received. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UCMF will not know the incoming URC format and hence will not be able to provide to the AMF/MME in the required format. 5GC principles of API reusability are not maintained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.18.2.1, 5.2.18.3.1, 5.2.18.3.2, 5.2.18.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.2.18.2.1	Nucmf_Provisioning_Create service operation

Service operation name: Nucmf_Provisioning_Create.

Description: The consumer creates a UCMF dictionary entry(s) for a Manufacturer-assigned UE Radio Capability ID(s).

Inputs (required): UE Radio Capability ID(s), set(s) of UE radio access capabilities, Type Allocation Code(s), Coding format(s).

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

The Coding format(s) indicates the format of the respective UE radio access capabilities as defined in TS 36.331 [16] or TS 38.331 [12].

NEXT CHANGES

##### 5.2.18.3.1	Nucmf_UECapabilityManagement Resolve service operation

Service Operation name: Nucmf_UECapabilityManagement_Resolve

Description: Consumer NF gets the UE radio access capability corresponding to a specific UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned) and Coding Format.

Inputs, Required: UE Radio Capability ID, Coding format.

Inputs, Optional: None.

Outputs, Required: UE radio access capability.

Outputs, Optional: None.

The Coding format indicates the format of the UE radio access capabilities as defined in TS 36.331 [16] or TS 38.331 [12].

NEXT CHANGES

##### 5.2.18.3.2	Nucmf_UECapabilityManagement_Assign service operation

Service or service operation name: Nucmf_UECapabilityManagement_Assign

Description: The NF consumer sends the UE radio access capability (and its Coding Format) to the UCMF and obtains a PLMN-assigned UE Radio Capability ID in return.

Inputs, Required: UE radio access capability, Type Allocation Code, Coding format.

Inputs, Optional: None.

Outputs, Required: UE Radio Capability ID.

Outputs, Optional: None.

The AMF does not send NB-IoT radio access capability to the UCMF.

The Coding format indicates the format of the UE radio access capabilities as defined in TS 36.331 [16] or TS 38.331 [12].

NEXT CHANGES

##### 5.2.18.3.5	Nucmf_UECapabilityManagement_Notify service operation

Service Operation name: Nucmf_UECapabilityManagement_Notify

Description: Producer NF provides notifications about changes in UCMF to subscribed consumer NF.

Inputs, Required: Notification Type ("creation", "deletion", "Added TAC in Manufacturer Assigned operation requested list", "Removed TAC from Manufacturer Assigned operation requested list", "Removed UE Radio Capability ID from Manufacturer Assigned operation requuested list").

Inputs, Optional: If Notification Type is set to "creation": One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID, the corresponding UE radio access capability and Coding format. If Notification Type is set to "deletion": One or more UE Radio Capability IDs. If Notification Type is set to "Added TAC in Manufacturer Assigned operation requested list" or "Removed TAC from Manufacturer Assigned operation requested list", one or more TACs, if Notification type is "Removed UE Radio Capability ID from Manufacturer Assigned operation requested list, one or more PLMN assigned UE radio Capability IDs.

Outputs, Required: None.

Outputs, Optional: None.

The Manufacturer Assigned operation requested list is defined in clause 5.11.3a of TS 23.401 [13] and clause 5.4.4.1a of the present document.

The Coding format indicates the format of the UE radio access capabilities as defined in TS 36.331 [16] or TS 38.331 [12].

END OF CHANGES