SA WG2 Meeting #S2-138E	S2-2002923

20 - 24 April, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2226 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE or network requested MA PDU Session Modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - When the AMF determines that the UE is unreachable over 3GPP access (e.g. the AMF receives no response from the UE to the Paging) or over non-3GPP access (e.g. the UE is in CM-IDLE on non-3GPP access), the AMF notifies the SMF that the UE is unreachable if the SMF subscribes to such an event. Based on that, the SMF may decide to move the GBR QoS Flow to the other access. This is missing in current specification. - When the SMF decides to move the GBR QoS Flow to the other access, to achieve service continuity, the SMF should trigger the release of AN resources in the source access after the establishment of AN resources in the target access. - The bullets describing that the SMF provides QoS profile to the AN are misformatted. - When the SMF decides to move the GBR QoS Flow to the other access, the SMF shall provide the updated N4 rules to the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add that the SMF may decide to move the GBR QoS Flow to the other access based on the report from the AMF (i.e. UE reachability report), besides the report from the UPF. 2. Clarify that the SMF triggers the release of the AN resources in the source access after the establishment of the AN resources in the target access. 3. Change the bullets describing that the SMF provides QoS profile to the AN to sub-bullets. 4. Change that the SMF “may” provide the updated N4 rules to the UPF to “shall”, for moving the GBR QoS flow to the other access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and incorrect procedures for QoS flow handling during the MA PDU session modification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.8.2 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

4.22.8.2	UE or network requested MA PDU Session Modification (non-roaming and roaming with local breakout)

The signalling flow for a MA PDU Session Modification when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.3.2-1 with the following differences and clarifications:

-	In step 1b, the SMF may decide to update ATSSS rules and/or N4 rules based on updated PCC rules.

-	In step 1d, if the UPF determines that it cannot send GBR traffic over the current ongoing access e.g. based on the N4 rules and access availability and unavailability report from the UE, the UPF shall send Access Availability report to the SMF. If the AMF determines that the UE is unreachable over 3GPP access (e.g. the AMF receives no response from the UE to Paging) or over non-3GPP access (e.g. the UE is in CM-IDLE on non-3GPP access), the AMF notifies the SMF if the SMF has subscribed to the event of UE reachability. When the SMF receives the Access Availability report from the UPF or the UE reachability report from the AMF, the SMF may decide to move the GBR QoS Flow to the other access as described in clause 5.32.4 of TS 23.501 [2]. If the SMF decides to move the GBR QoS Flow, the SMF triggers this procedure and moves the GBR QoS Flow to the target access.

-	In step 3, if the SMF decides to move the GBR QoS Flow to the other access, the SMF sends N2 SM information to the target AN. The PDU Session Modification Command message is sent to the UE to update ATSSS rule of the UE so that the UE sends uplink GBR traffic over the target access.

-	In step 3, when the SMF establishes user plane resources for a QoS flows, the SMF provides QoS profile to the AN as follows:

-	for Non-GBR QoS Flow, steps 3 to 8 are performed over each access for which the user plane resources are activated.

-	for GBR QoS Flow allowed in a single access, steps 3 to 8 are performed in the allowed access.

-	for GBR QoS Flow allowed in both accesses, steps 3 to 8 are performed in the access according to the decision by the SMF (as described in clause 5.32.4, TS 23.501 [2]).

-	In step 3, if the SMF wants to update ATSSS rules, the SMF includes updated ATSSS rules in the N1 SM container (PDU Session Modification Command). When the SMF provides N1 SM container and/or N2 SM information, the SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the AMF.

-	In step 8, if the SMF decides to moves GBR QoS Flow to the other access, the SMF shall send updated N4 rules to the UPF. The SMF triggers the release of user plane resources of the GBR QoS Flow in the source access if not already released.

******************** End of Change ********************