3GPP TSG-WG SA2 Meeting #138E e-meeting  	S2-2003118

# Elbonia, April 20 – 24, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2348 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of 5G LAN-type service under ETSUN architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical LAN, ETSUN |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5G LAN-type service is limited to served by a single SMF in the PLMN. However, UE may access to the 5G LAN-type service in an area not served by the SMF. Like ETSUN architecture, an I-SMF is inserted to serve the UE location and connected to the SMF. If multiple UEs access to the same (5G LAN) DNN, and are served by the same I-SMF, to optimize the traffic routing path, it is expected that the traffic between these UEs can be routed by the UPF controlled by the I-SMF. This is achieved by SMF to provide the DNAI of interest for this PDU Session for local traffic steering to the I-SMF. I-SMF can indicate the UPF(s) to perfrom local switch or N19-based routing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When an I-SMF is inserted to serve a PDU Session, and SMF provides the DNAI of interest for this PDU Session for local traffic steering to the I-SMF, I-SMF may indicate the UPF(s) (controlled by I-SMF) to apply local switch or N19-based traffic forwarding, as described in clause 5.29.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How UE accesses 5G LAN-type service via I-SMF is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.34.X	Support for 5G LAN-type service

This clause defines the specific impacts of deployments topologies with specific SMF Service Areas on how 5GS supports 5G LAN-type service as defined in clause 5.29.

For multiple UE PDU sessions served by the same I-SMF, and SMF provides the DNAI of interest for this PDU Session for local traffic steering to this I-SMF, I-SMF may indicate the UPF(s) (controlled by I-SMF) to apply local switch or N19-based traffic forwarding, as described in clause 5.29.4.

# * * * * End of changes * * * *