SA WG2 Meeting #1378E (e-meeting)	S2-2002684r03update

- April 20 - 24, 2020, Elbonia  	 									            (revision of S2-XXXXXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2223 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the supported and non-supported features and services for SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SNPN is a kind of 5GS and doesn’t support all of the 5GS services and features are supported by SNPN. Some non supported services and features are specifed in 23.501, e.g. emergency services, roaming between SNPNs. - So far, it is specified that roaming between SNPNs is not supported for SNPN. Actually, all kinds of roaming is not supported for SNPN in this release, e.g. roaming between SNPN and PLMN or PNI SNPN is also not supported. - Some non-supported feature and services are under study to support in release17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify roaming for SNPN is not supported in this release, including roaming between SNPN and PLMN or PNI SNPN; Clarify the non supported features and services are not supported for SNPN in this release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The supported and non-supported features and services are incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

## 5.30	Support for non-public networks

### 5.30.1	General

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2]. An NPN is either:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public Network Integrated NPN (PNI-NPN), i.e. a non-public network deployed with the support of a PLMN.

NOTE:	An SNPN and a PLMN can share NG-RAN as described in clause 5.18.

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3, the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.8.2.1-1) for access to SNPN services via a PLMN (and vice versa) and the additional functionality covered in clause 5.30.2. In this Release, direct access to SNPN is specified for 3GPP access only.

Interworking with EPS is not supported for SNPN in this release. Also, emergency services are not supported for SNPN in this release. Furthermore, roaming is not supported for SNPN in this release, e.g. roaming between SNPNs, roaming between SNPN and PLMN or PNI NPN. Handover between SNPNs, between SNPN and PLMN or PNI NPN are not supported for SNPN in this release.

Public Network Integrated NPNs are described in clause 5.30.3.

* * * * End of Change * * * *