3GPP TSG-SA2 Meeting #138-E	S2-2003193

# Online, 20th Apr 2020 – 23rd Apr 2020		     							   (was S2-2002857)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2208 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enablers for multiple SCPs (23.502) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation, Verizon UK Ltd, Nokia, Nokia Shanghai-Bell, T-Mobile, Inter Digital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR addresses the 23.502 aspects of the issue of handling Multiple SCPs (This covers both model C and model D defined in 23.501 Annex E). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SCP can register its profile information in the NRF. Consumer or/and SCP can then use it to discover the next hop SCP. This can be useful to route a service request from a consumer to a producer via one or more SCPs. 23.501 related aspects are covered in a corresponding CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be a way to route a service request from a consumer to a producer via multiple SCPs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.1, 5.2.7.2.2, 5.2.7.3.1, 5.2.7.3.2., new Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.2.7.1	General

The following table shows the NRF Services and Service Operations:

Table 5.2.7.1-1: NF services provided by the NRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_NFManagement | NFRegister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP |
|  | NFUpdate | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP |
|  | NFDeregister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP |
|  | NFStatusSubscribe | Subscribe/Notify | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM |
|  | NFStatusNotify |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM |
|  | NFStatusUnSubscribe |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM |
| Nnrf_NFDiscovery | Request | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, AF (NOTE 2) |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, UDM, NWDAF, I-CSCF, S-CSCF, IMS-AS, HSS |

NOTE 1:	HSS_IMS services are defined in TS 23.228 [55].

NOTE 2:	The AF is a trusted AF by an operator.

2nd change

##### 5.2.7.2.2	Nnrf_NFManagement_NFRegister service operation

Service Operation name: Nnrf_NFManagement_NFRegister.

Description: Registers the consumer NF in the NRF by providing the NF profile of the consumer NF to NRF, and NRF marks the consumer NF available.

Inputs, Required: NF type, NF instance ID, FQDN or IP address of NF, Names of supported NF services (if applicable), and PLMN ID e.g. if NF needs to be discovered by other PLMNs.

NOTE 1:	for the UPF, the addressing information within the NF profile corresponds to the N4 interface.

NOTE 2:	For the purpose of the Nnrf_NFManagement service, the SCP is treated by the NRF in the same way as NFs. Specifically, the SCP is designated with a specific NF type and NF instance ID. However, the SCP does not support services.

Inputs, Optional:

-	If the consumer NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s). If the consumer is BSF: Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes.

NOTE 32:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the consumer is UDM, UDR, PCF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, AUSF Group ID respectively.

-	For UDM and AUSF, Routing Indicator.

-	If the consumer is AMF, it includes list of GUAMI(s). In addition, AMF may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the consumer is CHF, it may include Range(s) of SUPIs, Range(s) of GPSIs, or Range(s) of PLMNs as defined in TS 32.290 [42].

-	If the consumer is P-CSCF, the P-CSCF IP address(es) to be provided to the UE by SMF.

-	If the consumer is HSS, IMPI range, IMPU range, HSS Group ID (as defined in TS 23.228 [55]) can be used as optional input parameters.

-	For the UPF Management: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	Information about the location of the NF consumer (operator specific information, e.g. geographical location, data center).

-	TAI(s).

-	NF Set ID.

-	NF Service Set ID.

-	If the consumer is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the consumer is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the consumer is NWDAF, it includes Analytics ID(s) and NWDAF Serving Area information. Details about NWDAF specific information are described in clause 6.3.13, TS 23.501 [2].

-	If the consumer is NEF, it may include range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF.

-	Notification endpoint for default subscription for each type of notification that the NF is interested in receiving.

-	Endpoint Address(es) of instance(s) of supported service(s).

-	NF capacity information.

-	NF priority information.

-	If the consumer is SCP, it may include SCP load information, locality, served PLMN(s) (see plmnList in 29.510 [37] clause 6.1.6.2.2), NF sets of NFs served by the SCP, serving scope (see servingScope in 29.510 [37] clause 6.1.6.2.2), Address Domain(s) reachable through SCP, NF types of NFs served by the SCP, Service names of services of NFs served by the SCP, Inter Connect SCP Domain(s).

Outputs, Required: Result indication.

Outputs, Optional: None.

See clause 5.21.2.1 in TS 23.501 [2], the AMF registers itself to NRF.

3rd change

#### 5.2.7.3	Nnrf_NFDiscovery service

##### 5.2.7.3.1	General

Service description: This service enables one NF or SCP to discover a set of NF/SCP instances with specific NF service or a target NF type. The service also enables one NF service or SCP to discover a specific NF service. The service operations defined below allow the NF/NF services or SCP to communicate with NRF.

This service also enables an NF service consumer or SCP to discover SCPs.

NOTE x:	For the purpose of the Nnrf_NFDiscovery service, the SCP is treated in the same way as NFs. It is designated with a specific NF type. However, the SCP does not support services.

4th change

##### 5.2.7.3.2	Nnrf_NFDiscovery_Request service operation

Service operation name: Nnrf_NFDiscovery_Request

Description: provides the IP address or FQDN of the expected NF instance(s) and, if present in NF profile, the Endpoint Address(es) of NF service instance(s) to the NF service consumer or SCP.

Inputs, Required: one or more target NF service Name(s), NF type of the target NF, NF type of the NF service consumer.

If the NF service consumer intends to discover an NF service producer providing all the standardized services, it provides a wildcard NF service name.

Inputs, Optional:

-	S-NSSAI and the associated NSI ID (if available), DNN, target NF/NF service PLMN ID, NRF to be used to select NFs/services within HPLMN, Serving PLMN ID, the NF service consumer ID, preferred target NF location, TAI.

NOTE 1:	For network slicing the NF service consumer ID is a required input.

-	FQDN for the S5/S8 interface of the PGW-C+SMF, to discover the N11/N16 interface of the PGW-C+SMF in the case of EPS to 5GS mobility.

-	If the target NF stores Data Set(s) (e.g., UDR, BSF): SUPI, IMPI, IMPU, Data Set Identifier(s). (UE) IPv4 address, IP domain or (UE) IPv6 Prefix.

NOTE 2:	If the request includes a subscriber identifier the NRF may need to use the association between the supplied subscriber identifier and the appropriate NF Group ID as described in TS 23.501 [2] clause 6.3.1 to determine the applicable set of NF instances for the response.

NOTE 3:	The (UE) IPv4 address or (UE) IPv6 Prefix is provided for BSF discovery: in that case the NRF looks up for a match within one of the Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes provided by BSF(s) as part of the invocation of Nnrf_NFManagement_NFRegister operation. The NRF is not meant to store individual (UE) IPv4 addresses or (UE) IPv6 prefixes.

-	If the target NF is UDM or AUSF, the request may include the UE's Routing Indicator.

-	If the target NF is AMF, the request may include AMF region, AMF Set, GUAMI and Target TAI.

-	If the target NF is UDR or UDM or AUSF or PCF, the request may include UDR Group ID or UDM Group ID or AUSF Group ID or PCF Group ID respectively.

NOTE 4:	It is assumed that the corresponding NF service consumer is either configured with the corresponding Group ID or it received it via earlier Discovery output.

-	If the target NF is UDM, the request may include SUPI, GPSI, Internal Group ID and External Group ID.

If the target NF is UPF, the request may include SMF Area Identity, UE IPv4 Address/IPv6 Prefix, supported ATSSS steering functionality

NOTE 5:	The (UE) IPv4 address or (UE) IPv6 Prefix is provided for UPF discovery: in that case the NRF looks up for a match within one of the Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes provided by UPF as part of the invocation of Nnrf_NFManagement_NFRegister operation. The NRF is not meant to store individual (UE) IPv4 addresses or (UE) IPv6 prefixes.

NOTE 6:	Discovering UPF at PDU Session Establishment time and creating the N4 association assumes full connectivity between SMF and UPFs.

-	If the target NF is CHF, the request may include SUPI or GPSI as specified in TS 32.290 [42].

-	If the target NF is PCF or SMF, the request may include the MA PDU Session capability to indicate that a NF instance supporting MA PDU session capability is requested.

-	If the target NF is PCF, the request may include the DNN replacement capability to indicate that a NF instance supporting DNN replacement capability is preferred.

-	If the target NF is NWDAF, the request may include Analytics ID(s) and TAI(s). Details about NWDAF discovery and selection are described in clause 6.3.13, TS 23.501 [2].

-	If the target NF is HSS, the request may include IMPI, and/or IMPU and/or HSS Group ID.

-	If the NF service consumer needs to discover NF service producer instance(s) within an NF instance, the request includes the target NF Instance ID and NF Service Set ID of the producer.

-	If the NF service consumer needs to discover NF service producer instance(s) in an equivalent NF Service Set within an NF Set, the request includes the identification of the equivalent NF service Set and NF Set ID of producer.

NOTE 7:	TS 29.510 [37] specifies the mechanism to identify equivalent NF Service Sets.

-	If the NF service consumer needs to discover NF service producer instance(s) in the NF Set, the request includes the target NF Set ID of the producer.

-	If the target NF is SMF, the request may include the UE location (TAI).

-	If the target NF is P-CSCF, the request may include UE location information, UE IP address/IP prefix, Access Type.

-	If the target NF is NEF, the request may include Event ID(s) provided by AF, and optional AF identification as described in TS 23.288 [50], clause 6.2.2.3. When the consumer is an AF, the request may include an External Identifier, External Group Identifier, or a domain name.

-	If the target NF is SMF, the request may include the Contorl Plane CIoT 5GS Optimisation Indication or User Plane CIoT 5GS Optimisation Indication.

-	If the target NF is SCP, the request may include information about target PLMN, NF types of NFs served by the SCP, Service names of services of NFs served by the SCP, preferred locality and Served Location(s) (see serving scope in 29.510[37]), Address Domain(s) reachable through SCP, Inter Connect SCP Domain(s).

Outputs, Required: A set of NF instances, a validity period for the discovery result, containing per NF Instance: NF type, NF instance ID, FQDN or IP address(es) of the NF instance and if applicable, a list of services instances, where each service instance has a service name, a NF service instance ID, and optionally Endpoint Address(es)

Endpoint Address(es) may be a list of IP addresses or an FQDN for the NF service instance.

NOTE x:	For the SCP as target NF no service instances are returned.

Outputs, Optional: Per NF instance, other information in the NF profile listed in clause 6.2.6 in TS 23.501 [2] related to the NF instance, such as:

-	NF load information.

-	NF capacity information.

-	NF priority information.

-	If the target NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s). If the target NF is BSF or P-CSCF: Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes.

NOTE y:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the target NF is UDM, UDR, PCF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, AUSF Group ID respectively.

-	If the target NF is HSS, it can include HSS Group ID.

-	For UDM and AUSF, Routing Indicator.

-	If the target NF is AMF, it includes list of GUAMI(s). In addition, it may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the target NF is CHF, it includes primary CHF instance and the secondary CHF instance pair(s).

-	For the UPF Management: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	Information about the location of the target NF (operator specific information, e.g. geographical location, data center).

-	TAI(s).

-	PLMN ID.

-	If the target is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the target is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the target NF is NWDAF, it includes the Analytics ID(s) and NWDAF Serving Area information. Details about NWDAF specific information are described in clause 6.3.13, TS 23.501 [2].

-	NF Set ID.

-	NF Service Set ID.

-	If the target NF is SMF, it may include the SMF(s) Service Area.

NOTE y:	If no SMF Service Area is provided, the AMF assumes that a SMF can serve the whole PLMN.

-	If the target NF is P-CSCF, it includes P-CSCF FQDN(s) or IP address(es) and optional Access Type(s) associated with each P-CSCF.

-	If the target NF is NEF, it may include Event ID(s) provided by AF.

-	If the target NF is SCP, Served Location(s), Remote PLMN reachable through SCP, Address Domain(s) reachable through SCP, NF types of NFs served by the SCP, Service names of services of NFs served by the SCP, Inter Connect SCP Domain(s).

See clause 4.17.4 and 4.17.5 for details on the usage of this service operation.

5th change

Annex x (informative):
SCP to SCP routing

# x.1	General

Annex x describes how maintaining and querying SCP profile in an NRF can be used to route message in a network.

NOTE:	We assume in this paper either a centric NRF or distributed NRFs, which share their information with each other.

## X.2	Multi-SCP path without delegated discovery

Figure x.2-1: Example flow with 3 SCPs

The prerequisite for this flow is that the NF consumer (denoted NFc) has done service instance selection of NF producer (denoted NFp), all SCPs register successfully in the NRF and all SCPs have discovered the profiles of the other SCPs by querying or/and subscribing to the NRF. Hence these steps are not explicitly shown in the call flow.

1.	NFc has selected the target address of wanted service. It has the SCP1 as a configured SCP. NFc sends the service request to SCP1. This includes the target address of NFp. This can include the Routing-Binding Indication and discovery and selection parameters.

2.	SCP1 checks if it has the received target address in its routing cache. In this case it does not have it, so it checks local configuration to determine whether the target address can be reached directly, and otherwise compares the target address with Address Domains in the SCP profiles and selects an SCP based on that information. If it received a Routing Binding Indication and/or discovery and selection parameters, it can invoke Nnrf_NFDiscovery (target-nf-type=NFp and target address of NFp) to get the corrresponding NFp profile.

3.	If step 2 was executed. the NRF responds with the profile of NFp.

4.	If step 2 was executed, SCP1 retrieves from the profile of NFp parmeters to compare with the SCP profiles, for instance the the Locality. It matches those parameters with parameters in the SCP profile, for instance it matches locality  with serving scope of other SCPs.
Based on that the selected target SCP3 and based on the Inter Connect SCP Domain (s) of the other SCPs, SCP1 derives the next hop SCP; in this case, SCP 1 realizes it needs to choose SCP2 as the next hop as SCP2 shares the same Inter Connect SCP Domain  with SCP 1 and SCP 3 but SCP1 and SCP3 do not share an Inter Connect SCP Domain.

5.	SCP1 forwards the service request to SCP2 and includes the target address and can include discovery and selection parameters, for instance the locality of NFp. SCP1 also includes the Routing-Binding Indication, if one was received.

6.	SCP2 matches received discovery and selection parameters with SCP profile information. For instance, it matches the locality against the serving scope of other SCPs. If it did not receice discovery and selection parameters, it applies procedures similar to step 2 and 3.  Based on that, it identifies SCP 3 as the next hop; SCP3 being in SCP2’s Inter Connect SCP Domain, SCP2 knows it can send the request to SCP3 directly and chooses it as its next hop.

7.	SCP2 forwards the service request to SCP3 and includes the target address and can include discovery and selection parameters, for instance the locality of NFp. SCP2 also includes the Routing-Binding Indication, if one was received.

8.	SCP3 checks local configuration to determine whether the target address can be reached directly. Since SCP3 can directly reach the target address, it forwards the service request directly to NFp. SCP3 can use information in Routing-Binding Indication and discovery and selection parameters, to reselect NFp.

## X.3	Multi-SCP path notification without delegated discovery

Figure x.3-1: Example flow with 3 SCPs

The prerequisite for this flow is that the NF producer (denoted NFp) is aware of NF consumer (denoted NFc)( e.g. it has received Binding Indication during the subscription), all SCPs register successfully in the NRF and all SCPs have discovered the profiles of the other SCPs by querying or/and subscribing to the NRF. Hence these steps are not explicitly shown in the call flow.

1.	NFp has selected the target address of a wanted service. It has the SCP3 as a configured SCP. NFp sends the notification request to SCP3. This can include the Routing-Binding Indication (including at least NFc instance ID or NF Set ID of NFc).

2.	SCP3 checks if it has the received target address in its routing cache. It checks local configuration to determine whether the target address can be reached directly, and otherwise compares the target address with Address Domains in the SCP profiles and selects an SCP based on that information.  Alternatively, it can use the locality of the NFc along with serving Scope and Inter Connect SCP Domains to determine the next hop as SCP2; SCP2 being in SCP3’s Inter Connect SCP Domain, SCP3 knows it can send the request to SCP2 directly and chooses it as its next hop..

3.	SCP3 forwards the service request to SCP2 and includes Routing-Binding Indication (including at least NFc instance ID or NF Set ID of NFc), if it was received.

4.	SCP2 checks if it has the received target address in its routing cache. It checks local configuration to determine whether the target address can be reached directly, and otherwise compares the target address with Address Domains in the SCP profiles and selects an SCP based on that information.  Alternatively, it can use the locality of the NFc along with serving Scope and Inter Connect SCP Domains to determine the next hop as SCP1; SCP1 being in SCP2’s SCP group, SCP2 knows it can send the request to SCP1 directly and chooses it as its next hop.

5.	SCP2 forwards the service request to SCP1 and includes Routing-Binding Indication (including at least NFc instance ID or NF Set ID of NFc), if it was received.

6.	If SCP1 received a Routing Binding Indication, SCP1 can decide to reselect the notification target and then invoke the Nnrf_NFDiscovery (target-nf-instance-id=NFc) to get the corrresponding NFc profile.

7.	If step 6 was executed, the NRF responds with the profile of NFc.

8.	SCP3 checks local configuration to determine whether the target address can be reached directly. Since SCP3 can directly reach the target address, it forwards the service request directly to NFc.

End of changes