3GPP TSG-WG SA2 Meeting #138E e-meeting  	S2-2002754

# Elbonia, April 20 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0439 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gp.p.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of BDT policy re-negotiation description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current description of background data transfer policy renegotiation lists the removal of the no longer valid background data transfer policy from the UDR as an independent action which happens before the potential AF selection of a new background data transfer policy. This removal triggers an immediate URSP rule update to potentially many UEs as other PCFs have subscribed to notifications on changes in background data transfer policy in UDR (see Note 8). After the AF has selected another background data transfer policy, the UDR is updated again and this will trigger another URSP rule update to potentially many UEs. Given that the background data transfer policy re-negotiation is only executed by one PCF while other PCFs may be resposible for the URSP rule update to the UEs, the no longer valid backgroud data transfer policy should not be only removed after the AF selected a new one (or did not select a new one). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description is updated so that the background data transfer policy removal in the UDR happens after the AF selection. This ensures that there is only one URSP rule update to the UEs. It is also clarified that the previously negotiated BDT policy shall be kept if the PCF does not find an alternative BDT policy to offer to the AF. In addition, the terminology is corrected in several places by adding the word “data” to achieve a consistent usage of the term “background data transfer”. Furthermore, the network performance analytics related description is generalized to allow for other output parameters of the network performance analytics. A statement about the PCF subscribing to the network performance analytics is also added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Triggering of URSP rule update to many UEs followed by another URSP rule update when a background data transfer policy is re-negotiated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.3, 6.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.1.1.3	Policy decisions based on network analytics

Policy decisions based on network analytics allow PCF to perform policy decisions taking into account the analytics information provided by the NWDAF. The PCF subscribes/unsubscribes to Analytics information as defined in TS 23.288 [24].

The following Analytics IDs are relevant for Policy decisions: "Load level information", "Service Experience", "Network Performance" and "Abnormal behaviour". The PCF may subscribe to notifications of network analytics related to "Load Level Information" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Load level information", the Analytics Filter "S-NSSAI and NSI ID" and the Analytics Reporting Information set to a load level threshold value. The PCF is notified when the load level of the Network Slice Instance reaches the threshold, and then the PCF may verify if the RFSP index value needs to be modified for a SUPI for which an AM Policy Association is created; this is based on operator policies in the PCF, as defined in clause 6.1.2.1.

The PCF may subscribe to notifications of network analytics related to "Service Experience" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Service Experience", the Target of Analytics Reporting "any UE" and the Analytics Filter including one or more "Application ID(s)". The PCF is notified on the Service Experience statistics or predictions including, for each Application Id, the list of SUPIs for which Service Experience is provided. In addition, both spatial and time validity may be provided as well as the confidence of the prediction. The PCF may check the 5QI values assigned to the Application, the number of UEs affected and may use this as input to calculate and update the authorized QoS for a service data flow template.

The NWDAF service to retrieve the service experience (i.e. the average observed Service MoS) is described in clause 6.4 of TS 23.288 [24].

The PCF may subscribe to notifications of network analytics related to "Network Performance" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Network Performance", the Target of Analytics Reporting "Internal Group Id" and the Analytics Filter including the Area of Interest. The PCF is notified on the Network Performance statistics or predictions including the Area of Interest and both the gNB status information and the predicted number of UEs in the Area of Interest. In addition, the confidence of the prediction may be provided. The PCF may use this information as input to calculate the background data transfer policies that are negotiated with the ASP, as defined in clause 6.1.2.4.

The NWDAF services to retrieve "Network Performance" as described in clause 6.6 of TS 23.288 [24].

The PCF may subscribe to notifications of network analytics related to "Abnormal behaviour" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Abnormal behaviour", the Target of Analytics Reporting "SUPI", "Internal Group Id" and the Analytics Filter including the list of Exceptions IDs and per each Expception Id a possible threshold. The list of Exception IDs is specified in TS 23.288 [24]. The PCF may use "Unexpected UE location" as input to determine the Service Area Restrictions defined in clause 6.1.2.1, "Suspicion of DDoS attack" to request the SMF to terminate the PDU session as defined in clause 6.1.3.6, "Wrong destination address" to perform gating of a service data flow as defined in clause 6.1.3.6 and "Unexpected long-live/large rate flows" to perform QoS related policies such as gating or policing as defined in clause 6.2.1.1.

The NWDAF services to retrieve UE related analytics are described in clause 6.7 of TS 23.288 [24].

The PCF may also use the network analytics as input to its policy decision to apply operator defined actions for example for the UE context(s) or PDU session(s).

# * * * * Second change * * * *

#### 6.1.2.4	Negotiation for future background data transfer

The AF may contact the PCF via the NEF (and Npcf_BDTPolicyControl_Create service operation) to request a time window and related conditions for future background data transfer.

NOTE 1:	The NEF may contact any PCF in the operator network.

The AF request shall contain an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window, the External Group Identifier and optionally, Network Area Information, request for notification. The AF provides as Network Area Information either a geographical area (e.g. a civic address or shapes), or an area of interest that includes a list of TAs or list of NG-RAN nodes and/or a list of cell identifiers. When the AF provides a geographical area, then the NEF maps it based on local configuration into of a short list of TAs and/or NG-RAN nodes and/or cells identifiers that is provided to the PCF. The NEF may map the ASP id based on local configuration into the DNN, S-NSSAI that is provided to the PCF. The request for notification is an indication that BDT warning notification should be sent to the AF. The BDT warning notification indicates to the ASP that the BDT policy needs to be re-negotiated.

NOTE 2:	A 3rd party application server is typically not able to provide any specific network area information and if so, the AF request is for the whole operator network.

The PCF shall first retrieve all existing background data transfer policies stored for any ASP from the UDR. The PCF may retrieve analytics on "Network Performance" from NWDAF in the area where the UEs of this ASP are expected to be located at a certain time period. The "Network Performance" Analytics includes the tuple-(expected load in the area of interest, expected number of UEs of this ASP in the Area of Interest) following the procedure and services described in TS 23.288 [24]. Afterwards, the PCF shall determine, based on the information provided by the AF, the analytics on "Network Performance" (if available) and other available information (e.g. network policy and existing background data transfer policies) one or more background data transfer policies. The PCF may be configured to map the ASP identifier into a target DNN and slicing information (i.e. S-NSSAI), that is used if the NEF did not provide the DNN, S-NSAAI to the PCF.

A background data transfer policy consists of a recommended time window for the background data transfer, a reference to a charging rate for this time window, network area information and optionally a maximum aggregated bitrate (indicating that the charging according to the referenced charging rate is only applicable for the aggregated traffic of all involved UEs that stays below this value). Finally, the PCF shall provide the candidate list of background data transfer policies or the selected background data transfer policy to the AF via NEF together with the Background Data Transfer Reference ID. If the AF received more than one background data transfer policy, the AF shall select one of them and inform the PCF about the selected background data transfer policy.

NOTE 3:	The maximum aggregated bitrate (optionally provided in a background data transfer policy) is not enforced in the network. The operator may apply offline CDRs processing (e.g. combining the accounted volume of the involved UEs for the time window) to determine whether the maximum aggregated bitrate for the set of UEs was exceeded by the ASP and charge the excess traffic differently.

NOTE 4:	It is assumed that the 3rd party application server is configured to understand the reference to a charging rate based on the agreement with the operator.

The selected background data transfer policy is finally stored by the PCF in the UDR as part of the Policy Data Set. The background data transfer policy contains the Background Data Transfer Reference ID, the volume of data to be transferred per UE, the expected amount of UEs, the one or more instances of the tuple (ASP id, DNN, S-NSSAI) and if the AF subscribed to notifications on changes of the negotiated BDT policy. The same or a different PCF can retrieve this background data transfer policy and corresponding related information from the UDR and take them into account for future decisions about background data transfer policies related to the same or other ASPs.

When the AF wants to apply the Background Data Transfer Policy to an existing session, then the AF will, at the time the background data transfer is about to start, provide, for each UE, the Background Data Transfer Reference ID together with the AF session information to the PCF (via the N5 interface). The PCF retrieves the corresponding background data transfer policy from Policy Data Set in the UDR and derives the PCC rules for the background data transfer according to this transfer policy.

When the AF wants to apply the Background Data Transfer Policy to a future session, then the AF provides, to the NEF, the Background Data Transfer Reference ID together with the External Identifier (i.e. GPSI) or External Group Identifier of the UE(s) that are to be subject to the policy. The NEF translates the External Group Identifier into the Internal Group Identifier or the External Identifier into a SUPI. The NEF stores the Background Data Transfer Reference ID, in the UDR as Application Data Set and Background Data transfer data Subset for an Internal Group Identifier or a SUPI and the ASP id requesting to apply the Background Data transfer Policy to a future session for the UE(s). A PCF that serves the UE(s) (i.e. the PCF that serves the UE for AM Policies) may retrieve the Background Data Transfer Reference ID by retrieving the UE's Application Data from the UDR or by subscribing to notifications of changes to the UEs' Application Data in the UDR. Furthermore, the PCF retrieves the specific Background Data Transfer Policy based on the received Background Data Transfer Reference ID stored as Policy Data Set from the UDR.

When the PCF determines to send the Background Data Transfer Policy information to the UE as part of a URSP rule, the PCF will store the policy in the UDR as part of the UE's Policy Set Entry and will use the associated S-NSSAI and DNN associated with the ASP id stored in the Application Data to store the Background Data Transfer Reference ID in the UE's PDU Session policy control subscription information (see clause 6.2.1.3). The PCF uses local policies to decide if and when the Background Data Transfer Policy information is going to be sent to the UE as Validation Criteria in the RSD part of the URSP rule (see clause 6.6.2.1). The UE uses Validation Criteria to determine whether or not a PDU Session should be established. The Time Window and Location Criteria are not required to be checked again during the lifetime of the PDU Session.

The PCF may, based on operator configuration, trigger the UE Configuration Update procedure when the policy is selected, or the PCF may wait until receiving a notification from the AMF that the UE has entered the Tracking Area or Presence Area where the policy applies, and/or the PCF may wait until the time window when the policy applies is approaching. The UE's support of the Validation Criteria in a URSP rule is optional.

NOTE 5:	If a non-supporting UE receives Validation Criteria, it ignores the URSP rule.

When the PDU Session is established, the PCF that serves the PDU session will use the Background Data Transfer Reference ID in the UE's PDU Session policy control subscription information (see clause 6.2.1.3) to retrieve the corresponding background data transfer policy (i.e. Time Window and/or Location Criteria) from the UDR and derives the PCC rules for the background data transfer according to this transfer policy.

NOTE 6:	The AF will typically contact the PCF for the individual UEs to request sponsored connectivity for the background data transfer.

NOTE 7:	A transfer policy is only valid until the end of its time window. The removal of outdated transfer policies from the UDR is up to implementation.

The PCF may reject corresponding SM Policy Association, as described in clause 4.16.4 of TS 23.502 [3], if Validation condition is not satisfied. And based on this feedback, SMF will reject the PDU session setup.

After successful PDU session setup, PCF may trigger PDU session release when Validation condition is not satisfied.

The PCF may subscribe to analytics on "Network Performance" from NWDAF for the area of interest and time window of a background data transfer policy following the procedure and services described in TS 23.288 [24]. When the PCF gets a notification from the NWDAF that the network performance in the area of interest and time window goes below the criteria set by the operator, the PCF  may try to re-negotiate the time window for the BDT policy. To do this, the PCF retrieves all the background data transfer policies from the UDR, identifies the BDT policy that is not applicable anymore due to the degradation of the network performance and calculates a list of new candidate BDT policies for the ASP to select from. If the PCF does not find any new candidate BDT policy, the previously negotiated BDT policy shall be kept and no interaction with the ASP shall occur. If the PCF finds at least one new candidate BDT policy, the PCF notifies the ASP on both the BDT policy that is not valid any longer and the candidate BDT policies via NEF if requested by the ASP.

The PCF removes the BDT policy stored in the UDR for the corresponding background data transfer reference ID.

When the AF receives the notification, the AF may select one of the background data transfer policies included in the candidate list, and then informs the PCF about the selected background data transfer policy. The PCF stores the newly selected background data transfer policy into the UDR for the corresponding Background Data Transfer Reference ID As a consequence, the PCF identifies the UEs for which the background data transfer policy was already applied and updates URSP rules with the new Validation Criteria as described in clause 4.16.12.2 of TS 23.502 [3].

NOTE 8:	A PCF can subscribe to notifications on changes in background data transfer policy in UDR. Upon reception of such notification the PCF has also to identify the UEs for which the background data transfer policy was already applied and update URSP rules with the new Validation Criteria as described in clause 4.16.12.2 of TS 23.502 [3].

.

# * * * * End of changes * * * *