SA WG2 Meeting #S2-138E	S2-2003032

20 - 24 April, 2020, Electronic, Elbonia	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2320 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the bridge model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During moderated emial discussion, the working assumption is, there is one or more Ports for each NW-TT. However, how many NW-TTs in the UPF is pending. If there are more than one NW-TT in the UPF for one bridge, how to handle is not clear. Example, it assumes there are 2 NW-TTs in the UPF for one bridge. And there are two ports in each NW-TT. - To convey port management information from NW-TT to TSN AF. The UPF combines the Port Management Information Container from each NW-TT and send to TSN AF, or UPF send the Port Management Information Container from each NW-TT to TSN AF separately? - To convey port management information from TSN AF to NW-TT. How the UPF determine which NW-TT will be sent? So far, the Port Management Information Container is transparent to UPF. - Qbv, traffic forwarding and PSFP configuration to NW-TT. These configuration are per port. How the UPF know which NW-TT is related to the configuration. All these issue requires the UPF to look into the conrainer or configuration. So to simplify, it proposes, there is one NW-TT per UPF for one bridge, and there is one or more ports in the NW-TT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding, there is one NW-TT per UPF for one bridge, and there is one or more ports in the NW-TT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to handle the case if there are more than one NW-TT in UPF for one bridge.5.28.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.28.1	5GS TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the ports on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the 5GS TSN bridge is per UPF. There is one NW-TT per UPF for one 5GS bridge, and each NW-TT can support one or more ports. The bridge ID of the 5GS TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side and a PDU Session during reporting of 5GS TSN bridge information. The TSN AF also stores the information about ports on the UPF/NW-TT side. The UPF/NW-TT forwards traffic to the appropriate egress port based on the traffic forwarding information.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single 5GS bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5GS Bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 3:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based 5GS bridge

NOTE 4:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple 5GS TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Configuring the bridge information in 5GS as defined at clause 5.28.2.

-	Report the bridge information of 5GS Bridge to TSN network after PDU session establishment.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS Flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge Address (unique MAC address that identifies the bridge used to derive the bridge ID);

-	Bridge Name;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97]:

-	Chassis ID subtype and Chassis ID of the 5GS Bridge.

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

-	Stream Parameters as defined in clause 12.31.1 in IEEE 802.1Q [98], in order to support PSFP information:

-	Maximum number of filters, which defines the maximum number of streams that the bridge can handle;

-	Maximum number of gates, which can be equal or less than the maximum number of filters;

-	Maximum number of meters (optional) if meassurements are required;

-	Maximum length of the PSFPAdminControlList parameter that can be handled.

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Bridge ID of the 5GS Bridge, port numbers of the Ethernet port in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT Ethernet port for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports in DS-TT of the related PDU session and port numbers and MAC addresses of the Ethernet ports in NW-TT to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released, then SMF informs TSN AF accordingly.

The TSN AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

END OF CHANGES