SA WG2 Meeting #S2-139E	S2-2003544

01 - 12 June, 2020, Electronic, Elbonia

3GPP TSG-CT WG3 Meeting #109e	C3-202507

# E-Meeting, 16th – 24th April 2020

Title:	LS on Clarification on eNA

Response to:

Release:	Rel-16

Work Item:	eNA

Source:	CT3

To:	SA2

Cc:

Contact Person:

Name:	Yali Yan

Tel. Number:	+ 86 15800958428

E-mail Address:	yanyali@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

# 1	Overall description

As defined in TS 23.288, the network slice instance (NSI ID) associated with the S-NSSAI is introduced as optional parameter in clause 6.3 for slice load level analytics type and observed service experience analytics type in clause 6.4, since the network may deploy multiple network slice instances for one S-NSSAI, depending on the deployment choices of the operator.

Clause 6.1.3 of TS 23.288 specifies the maximum number of objects and maximum number of SUPIs as two parameters of Analytics Reporting Information.

========   TS 23.288 clause 6.1.3  =========

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	(Only for Nnwdaf_AnalyticsInfo_Request) Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shal return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

However, in many clauses in TS 23.288, e.g. clauses 6.4.1 and 6.7.5, both two parameters also are specified as Analytics Filter Information.

========   TS 23.288 clause 6.4.1  =========

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1 and, optionally, maximum number of objects and maximum number of SUPIs;

========   TS 23.288 clause 6.7.5  =========

The consumer of this analytics shall indicate in the request:

-	Analytics ID set to "Abnormal behaviour";

……

-	maximum number of objects;

-	maximum number of SUPIs;

As defined in clause 6.1.3 of TS 23.288, Analytics Filter Information is used to indicate the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

CT3 asks SA2 to clarify the following questions:

Q1:  Is the network slice instance also applicable for other analytics types which may provide the S-NSSAI during the analytics request, e.g. QoS Sustainability Analytics?

Q2: 	Does the maximum number of objects and/or the maximum number of SUPIs can be provided as both Analytics Reporting Information and Analytics Filter Information in one Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request request sent from the NF consumer to the NWDAF?

# 2	Actions

To SA WG2 group:

ACTION: 	CT3 kindly asks SA2 to answer above questions and update their specifications if necessary.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#110e	02nd – 11th June 2020	E-Meeting

3GPP TSG CT3#111	24th – 29th August 2020	US, US