SA WG2 Meeting #S2-139E	S2-2004288

01 - 12 June, 2020, Electronic, Elbonia

Source:	Orange, MediaTek Inc., Telecom Italia, AT&T, China Mobile

Title:	KI #4, New Sol: UE onboarding using PLMNs

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This is a solution to Key Issue #4; it addresses the UE onboarding for the SNPN with non-AKA credentials with the use of PLMNs.

# Proposal

It is proposed to add the below text into the TR 23.700-07.

# *** Start of changes ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 | X |  |  |  |  |  |
| 3 | X |  |  |  |  |  |
| 4 | X |  |  |  |  |  |
| 5 |  |  |  | X |  |  |
| 6 |  |  |  | X |  |  |
| 7 |  |  |  | X |  |  |
| X |  |  |  | X |  |  |

# *** Next change (all new text) ***

## 6.X	Solution #<X>: UE onboarding using PLMNs

### 6.X.1	Introduction

This is a solution to Key Issue #4; it addresses the UE onboarding for the SNPN with non-AKA credentials.

The UE is configured with Default credentials in USIM to attach to any PLMN in which the UE can register with the Default credentials, i.e. an Onboarding Network (ON). This enables UEs to be provisioned with necessary network credentials and configuration for the SNPN that will own the UE's subscription.

Figure 6.X.1-1: UE onboarding using PLMNs

The following assumptions are considered:

-	The UE is equipped with a USIM containing Default credentials belonging to a PLMN operator.

-	The UE is not provisioned with NPN credentials.

-	As part of the onboarding process the UE is eventually provisioned with NPN credentials that grant access to an SNPN.

-	No relationship is assumed between the PLMNs used for UE onboarding and the SNPN.

-	The PLMNs provides the UE with connectivity to a Provisioning Server that allows UEs to retrieve their NPN credentials and other personalized configuration.The Provisioning Server can be owned by the device manufacturer or an entity affiliated with the device manufacturer.

-	The SNPN owning the subscription has a list of GPSI identifying the UEs for which a subscription will be provisioned using the UE onboarding procedure and provides this list to the Provisioning Server.

### 6.X.2	Functional Description

The UE is manufactured with Default credentials that consist in a USIM profile that will be used only for UE onboarding. This USIM profile can be issued by any PLMN operator the manufacturer has an agreement with. In what follows, this PLMN is referred to as "HPLMN". The UDM (or HSS) of the HPLMN plays the role of Default Credential Server (DCS) and the Provisioning Server (PS) is in a DN that the HPLMN provides access to.

Any PLMN available at the location where the UE is when performing UE onboarding can be used as Onboarding Network (ON), as long as it has a roaming agreement with the HPLMN.

At power-up, the UE does not have information for SNPN selection and therefore performs normal PLMN selection, registers and establishes a PDU Session using normal procedures. The UE subscription corresponding to the USIM profile contains a default Subscribed S-NSSAI and a default DNN that are appropriate to reach the Provisioning Server; consequently a PDU Session that provides access to the Provisioning Server is established without the UE having to indicate any NSSAI or DNN information.

The UE connects to the Provisioning Server through the established PDU Session. The HPLMN provides the GPSI of the UE subscription to the Provisioning Server e.g. using header enrichment. Upon establishment of connectivity to the Provisioning Server, the UE is provisioned with NPN credentials (for the SNPN that will own the UE's subscription) and additional configuration data. Then the UE deregisters from the PLMN and, using the information acquired during UE onboarding, performs a new network selection and registers using the provisioned NPN credentials with the SNPN owning the UE's subscription.

### 6.X.3	Procedures

Figure 6.X.3-1: High-level flow for onboarding of the UE into an SNPN

The procedure consists in the following steps:

A)	UE pre-configuration: the UE is equipped with a USIM containing Default credentials that allows for successful authentication of the device during the PLMN access step (step B).

B)	PLMN access: In this step, the UE discovers and selects the PLMN to be used as ON using regular procedures. The UE performs a Registration procedure as specified in TS 23.502 [6] clause 4.2.2.2.2, without providing any Requested NSSAI. The AMF uses the S-NSSAI marked as default in the Subscribed S-NSSAIs of the UE subscription.

C)	PDU Session Establishment: The UE establishes a PDU session as specified in TS 23.502 [6] clause 4.3.2.2, without providing any DNN. The AMF determines the DNN for the requested PDU Session by selecting the default DNN present in the UE's Subscription Information.

D1)	The device discovers and connects, at application level, to a provisioning server address (that was preconfigured in the UE in step A or is derived from the application identifier and/or Service Provider Identifier provided by the user in step B) for retrieving its own personalized information. The HPLMN provides the GPSI of the UE subscription to the Provisioning Server e.g. using header enrichment and the provisioning server relies on the GPSI to identify the UE.

D2)	The Provisioning Server retrieves the NPN credentials for the future SNPN owning the subscription, as well as other UE configuration parameters (e.g. PDU session parameters, such as SNSSAI, DNN, URSPs, QoS rules, and other required parameters to access the SNPN and establish a regular PDU session).

D3)	The Provisioning Server pushes the UE's NPN credentials for the SNPN and other configuration information into the UE.

E)	De-registration: Upon a successful provisioning in the previous step, the device releases the PDU Session and deregisters from the ON.

F)	Normal service: Upon a successful de-registration as per step E, the device initiates a regular procedure, including selection of an SNPN, Registration using the provisioned NPN credentials with the SNPN owning the subscription, and PDU Session establishment(s).

### 6.X.4	Impacts on services, entities and interfaces

This solution has no network impact and minimal UE impact.

The UE impact is to be pre-configured with provisioning server address or derive it, and initiate a connection to this address.

# *** End of changes ***