3GPP TSG SA WG2 Meeting #139-e		S2-2004248r08

Electronic, 01 – 12 June 2020

Title:	LS on Potential RAN impact for support of Multi-USIM devices

Response to:

Release:	Rel-17

Work Item:	FS_MUSIM

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR?

# 1	Overall description

SA2 would like to inform RAN2 and RAN3 that they have progressed the study on FS_MUSIM (TR 23.761) and would like to provide an early overview of solution types under consideration by SA2 that have RAN impact.

SA2 would like to point out that they have not yet completed the evaluation of the solutions in TR 23.761, many of them having RAN dependency and requiring closer examination and feedback from RAN WGs.

According to the current SA2 work plan, the FS_MUSIM study and normative work are scheduled for completion in Q3 and Q4, respectively.

The table below provides a high-level overview of FS_MUSIM solution types in TR 23.761 that require RAN impact or require RAN feedback in order to conclude:

|  | Solution’s distinguishing mark | Reference | Likely RAN WG impact |
| --- | --- | --- | --- |
| Key Issue 1: Handling of Mobile Terminated service with Multi-USIM device | Paging Cause in the Uu Paging message for RRC_Idle and RRC_Inactive. | Solution #1 | RAN2, RAN3 |
|  | Negotiated “absence” | Solution #2 | RAN2, RAN3 |
|  | RRC Busy indication as a response to Uu Paging | Solution #3 | RAN2, RAN3 (?) |
|  | Handling of Service “whitelist” / “blacklist” in RAN for UEs in RRC_Inactive | Solution #A | RAN3 |
| Key Issue 2: Enabling Paging Reception for Multi-USIM Device |  |  |  |
|  | UE negotiates “scheduling gap” in RAN A so that UE can listen to paging in RAN B. No NAS impact | Solution #C | RAN2 |
|  | NAS-only solutions requiring RAN WG’s feedback (1/2): - UE detects potential collisions and requests new GUTI. | Solution #D | RAN2, RAN3. The feedback in particular is related to the type of assistance information (if any) that UE can provide to 5GC to assist the new GUTI assignment |
|  | NAS-only solutions requiring RAN WG’s feedback (2/2): - CN and RAN use an Alternative UE ID (different from UE’s GUTI) for paging. | Solution #E | RAN2, RAN3 |
|  | Implementation-based (e.g. alternating USIMs when monitoring POs) |  | RAN2 |
| Key Issue 3: Coordinated leaving for Multi-USIM device | Local RRC release in NG-RAN based on N2 request. | Solution #4 | RAN2, RAN3 |
|  | UE-requested Leave and Resumption via RRC. | Solution #5, Solution #6 | RAN2, RAN3 |

SA2 would like RAN WGs, if feasible, to provide any early feedback in Q3 that would potentially help to reduce the number of candidate solutions in SA2.

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 kindly asks RAN2 and RAN3 to take into consideration the information above and provide any feedback in Q3, if feasible, on the solution proposals with RAN impacts in TR 23.761 in addition with any other solutions that RAN2/RAN3 would like SA2 to consider.

# 3	Dates of next TSG SA WG 2 meetings

SA2#140E		19 August - 2 September			Electronic meeting