3GPP TSG-SA WG2 Meeting #140 E (e-meeting)	S2-2005182

August 19 – September 02, 2020, Elbonia	(Revision of S2-200xxxx)

Source:	Lenovo, Motorola Mobility

Title:	KI #7, Sol #30: Update to remove Ed. Notes

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes an update to solution #30 to resolve the Editor's Notes and to clarify open points.

# 1. Introduction

Solution #30 is for KI#7 and on a high-level the proposal is that the preferred frequency bands information is included in the Configured NSSAI. The UE uses this preferred frequency info when a new Requested NSSAI is created for a new Registration procedure.  The main problem solved by Solution #30 is how does the UE select an appropriate cell (e.g. in an appropriate frequency layer), when the UE initiates a Registration procedure in order to register to Network Slice(s).

There is the following Editor's Note in clause 6.30.1:

Editor's note:	it is FFS how to cover the case where S-NSSAIs are not provided over the same frequencies network-wide. It is FFS how to align with the current mechanism where the slice availability itself is provided from RAN to CN, but if a slice is available, its frequency is configured separately on access and core.

The first part of this Editor's note says: "it is FFS how to cover the case where S-NSSAIs are not provided over the same frequencies network-wide".  The resolution of this FFS is already in the solution:

"The parameter preferred frequency bands may include a list of target carrier frequencies containing one or more entries, and for each entry a carrier frequency priority index may be associated as well. For example, the priority index indicating the priority for scanning/selecting of a carrier frequency."

With this, if an S-NSSAI is deployed in several frequencies in the PLMN, then the parameter Preferred Frequency Bands may include a list of all carrier frequencies where the S-NSSAI is deployed.

The second part of the Editor's note says: "It is FFS how to align with the current mechanism where the slice availability itself is provided from RAN to CN, but if a slice is available, its frequency is configured separately on access and core".  We believe that this FFS is not relevant to our solution, as this FFS is about the steering of the UE for Idle mode mobility (i.e. cell selection process), which is steered from the RAN when the RRC Connection is released.  To clarify that Solution #30 targets the case of cell selection during the Registration procedure, it is proposed to update the solution #30 principles.

Proposal 1: it is proposed to remove the Editor's Note in clause 6.30.1 and to clarify the use case of the solution #30. Namely, the main feature is that the UE in Idle state, when initiating a Registration procedure with a new Requested NSSAI, should apply the frequency band priority from the Configured NSSAI during the cell selection procedure.

There is one further Editor's Note in clause 6.30.2:

Editor's note:	it is FFS how this scales if availability of S-NSSAIs over frequency bands is not uniform across the network.

In our view, this is similar to the FFS from clause 6.30.1 "it is FFS how to cover the case where S-NSSAIs are not provided over the same frequencies network-wide".  The resolution to this Editor's Note is as described above, i.e. if an S-NSSAI is deployed in several frequencies in the PLMN, then the parameter Preferred Frequency Bands may include a list of all carrier frequencies where the S-NSSAI is deployed.

Proposal 2: it is proposed to delete the Editor's Note in clause 6.30.2 and to highlight the details for the use case when an S-NSSAI operate in multiple frequency bands.

# Proposal

It is proposed to update the following solution in 3GPP TR 23.700-40.

1st  CHANGE

## 6.30	Solution #30: Preferred frequency bands in Configured NSSAI

### 6.30.1	Introduction

This solution is for Key Issue #7, Support of 5GC assisted cell selection to access network slice. This solution solves the particular problem in KI#7, "How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band)", i.e. how to steer the UE to cell(s) where the registration with the S-NSSAI(s) from the Requested NSSAI can be successful.

The solution is based on the following architectural assumptions:

-	The Rel-15 concept is retained that homogenous availability of S-NSSAIs from the Allowed NSSAI is provided within a Registration Area.

-	The UE may simultaneously register to S-NSSAIs that are accessible on different operating frequency band(s).

-	If the UE is about to register to one or more network slices identified by S-NSSAIs which are accessible on some preferred frequency band(s), then the UE in Idle state considers the list of preferred band(s) of the S-NSSAIs for cell selection when triggering the Registration request message.

-	If the Allowed NSSAI  contains one or more S-NSSAIs which are accessible on different frequency band(s), then the UE is provided with configuration from the RAN (e.g. during RRC Connection Release procedure) to follow carrier frequencies for Idle mode mobility.

-	It is assumed that the AMF (or NSSF, or both) and the NG-RAN are configured with the preferred frequency band(s) per S-NSSAI. With this, signalling enhancements to the N2 MM and N2 SM signalling are not needed.

### 6.30.2	High Level Description

The high-level principle of this solution is that the UE is provided with preferred frequency band(s) (PFB) information per network slice (e.g. target carrier frequencies per S-NSSAI) in the Configured NSSAI.  The PFBs are applicable to the S-NSSAIs of the Serving PLMN.  The UE in Idle state, which initiates a Registration procedure with a new Requested NSSAI, uses the PFBs from the Configured NSSAI for cell selection procedure in order to select the appropriate cell where to establish the RRC connection.

If an S-NSSAI is available over more than one frequency bands, the PFB parameter  may include a list of target carrier frequencies containing one or more entries, and for each entry a carrier frequency priority index may be associated as well. For example, the priority index indicating the priority for scanning/selecting of a carrier frequency, if multiple carrier frequencies are available.

After the Configured NSSAI has been updated and the UE is in Idle state (e.g. the 5GC may initiate the release of N1 connection, if the Configured NSSAI has been updated), the UE determines which S-NSSAIs it wants to register with, i.e. the S-NSSAIs to be included in the Requested NSSAI. Then, the UE in Idle state performs cell selection procedure considering the frequency priority (and the priority index) from the PFB for the S-NSSAI(s) to be included in the Requested NSSAI. The NAS layer provides the PFB information to the AS layer to assist the cell selection procedure.

### 6.30.3	Procedures

The Configured NSSAI is provided to the UE during the Registration Accept message and UE Configuration Update Command from the AMF. The AMF or NSSF when creating the Configured NSSAI for the UE may include the preferred frequency band (PFB) per S-NSSAI. If the AMF performs update of the Configured NSSAI, the AMF may trigger N1 connection release procedure in order to allow the UE to apply the new configuration when the UE is in Idle state.

With respect to the cell selection procedure in Idle state, the UE may first create a Requested NSSAI (e.g. based on the Configured NSSAI) to be included in the Registration Request message. The UE may then consider the list of PFBs for the S-NSSAI values included in the Requested NSSAI.  If there are more than one S-NSSAIs in the Requested NSSAI, the UE may prioritize the S-NSSAIs, and consequently, prioritize the PFBs to be used for cell selection.  The prioritization of S-NSSAIs is up to UE internal configuration (e.g. from higher layers) or user priorities. The NAS layer provides the PFB information to the AS layer for cell selection, i.e. the PFB may overwrite the frequency configuration which has been stored in the AS layer from the last RRC Connection Release procedure.

If a Service Request procedure or activation of UP resourced for a PDU Session procedure result in use of multiple network slices operated in different frequency bands, the NG-RAN node may decide to activate Dual Connectivity in order to used different frequency bands simultaneously.

### 6.30.4	Impacts on services, entities and interfaces

-	To AMF (or NSSF):

-	Takes into account the preferred frequency bands per S-NSSAI when generating the Configured NSSAI, Allowed NSSAI and Rejected S-NSSAIs;

-	Provides to the UE preferred frequency bands per S-NSSAI within the Configured NSSAI.

-	To UE:

-	During Registration procedure with a new Requested NSSAI, the UE takes into account the preferred frequency bands per S-NSSAI as provided in the Configured NSSAI for the cell selection procedure.

End of CHANGES