3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005387

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, Hisilicon

Title:	KI#2: EAS relocation for SSC mode 3

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes a new solution for EAS relocation for SSC mode 3.

- Discussion

In the TR, solution#23 is for SSC mode 3 PDU Session. In this solution, the UE reselects another EAS when a new PDU session to the same DN has been established. The UE provides the new EAS IP address to the old EAS. And context migration is triggered by old EAS. After the completion of the context migration, old EAS indicates UE to connect to new EAS. All this is done via the application layer signaling

In Solution#23 EAS reallocation is controlled by the UE. This is not suitable. It is more suitable to do the EAS reallocation per the 5GC network notification. It is proposed a solution for EAS relocation for SSC mode 3 PDU Session triggered by the 5GC and determined by the EAS. In the solution, the target EAS is selected by old EAS.

- Proposal

It is proposed to have the following changes in TR 23.748-040:

/************************** 1st Changes *************************/

6.x	Solution #x: EAS relocation for SSC mode 3 PDU Session

This solution is for Key Issue #2 on EAS relocation after PDU session re-establishment for SSC mode 3 PDU session.

6.x.1	Description

The AF (e.g. Old EAS) subscribes to UP path change notification. After the PDU Session re-establishment of the SSC mode 3 PDU Session, the SMF sends UP Path change notification to the AF, providing the new DNAI of the PDU Session Anchor UPF of the new PDU Session. The AF reselects target EAS based on the new DNAI and triggers context relocation between old EAS and new EAS.

6.x.2	Procedures

Figure 6.x.2-1: EAS relocation after SSC mode 3 PDU Session re-establishment

- The AF (e.g. Old EAS) subscribes to UP path change notification regarding the PDU session to SMF. This may be triggered by the UE establishing connection with the old EAS.

- When UE moves, the SMF decides to re-establish PDU Session to the same DNN for the SSC mode 3 PDU Session. This may be because the UE has moved out of the service area of the UPF of the old PDU Session.

- After the new PDU Session to the same DNN has been established, the SMF sends UP Path change notification to the AF that subscribed for this event. The notification includes the DNAI corresponding to the UPF of the new PDU Session.

NOTE: This only applies to the case in which the SMF does not change for the new PDU Session.

- The AF reselects a target EAS if it is available based on the DNAI.

- The AF starts context migration between old EAS and the target EAS.

- The old EAS redirects the UE to the target EAS based on application mechanism. The UE establishes connection with target EAS. Service continuity can be achieved as follows:

- The old EAS terminates transmitting downlink packets and indicates the termination of traffic transmitting in downlink by sending a TCP FIN packet to the UE.

- The UE acknowledges the TCP FIN packet and indicates the termination in uplink by sending a TCP FIN packet to the old EAS.

- The old EAS finishes the context migration with the target EAS and indicates the successful of EAS relocation by sending an acknowledgement back to the UE.

- The UE starts to send uplink packets of the application to the target EAS. Based on the received acknowledgement, the UE can continue sending the packets that have not been acknowledged by the old EAS.

6.X.3	Impacts on services, entities and interfaces

UE:

- Continue sending the packets in the new TCP connection for the packet which have not been acknowledged by the old EAS via the old TCP connection.

Old EAS：

- The old EAS acknowledges the termination of the uplink transmission of the TCP connection until it has finished the context migration with the target EAS.

- Optionally, the new EAS continues sending the packets that have not been acknowledged in the old connection.

/*********************** End of Changes **********************/