SA WG2 Meeting SA2-140e	S2-2005685

19th August – 2nd September, 2020, E-meeting	(revision of S2-201xxxx)

Source:	Intel

Title:	KI #1, Proposed way forward

Document for:	Discussion / Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes to interim conclusions on KI#1.

# 1	Discussion

During the FS_MUSIM conference call Intel proposed an evaluation of solutions and a way forward proposal on KI#1:

https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_140e_Electronic/INBOX/CCs/2020_08_06_FS_MUSIM_PreSA2%23140E/INT%20S2-200xxxx_FS_MUSIM_wf.pptx

This contribution updates the way forward proposal and formats it to fit TR 23.761.

# 2	Proposal

It is proposed to update TR 23.761 as indicated below.

* * * * Next Change * * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

* * * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

Based on the evaluation in clause 7.1, the following interim concusions are agreed for the baseline functionality:

Among the solution principles with RAN impact:

-	A Paging Cause (as in Solution #1) shall be considered for further study for both EPS and 5GS, pending confirmation from SA3 about any privacy issues with sending the Paging Cause as cleartext. The content of the Paging Cause is FFS.

-	A Negotiated Short Absence (as in Solution #2) shall be considered for further study for 5GS together with the KI#3 solutions on RRC-based coordinated leaving, pending RAN WGs confirmation on feasibility.

-	A Busy indication (as in Solution #3) should be considered for further study for 5GS, pending feedback from RAN WGs.

-	MUSIM Assistance Information for paging filtering (as in Solution #10) should be considered for further study only as part of the solutions for KI#3.

Editor’s note:	The decision related to the other add-ons (Sol#9, Sol#11) is postponed until Q4.

Editor’s note:	It is FFS whether there is a need for both solution with and without RAN impact. The decision will be made in Q4 based on the received feedback from other WGs.

* * * * End of Change * * * *