3GPP TSG-SA/WG2 Meeting #141E	S2-2007368

Elbonia, 12 – 23 Oct. 2020, Electronic meeting

Source:	ZTE

Title:	KI#3 evaluation and conclusion

Document for:	Agreement

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation for KI#3

1	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

7.3	Evaluation on solutions of KI#3

Editor's note:	This clause will provide some interim evaluation based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

High level aspects of the solutions:

-	Solution 22 has RAN impact. It lets RAN to enforce the SMBR (Slice Maximum Bitrate). Currently, RAN is able to be aware of the S-NSSAI of the PDU Session. And RAN is able to be enforce the UE AMBR per UE and GFBR/MFBR per QoS Flow.

Editor´s note:	Solution#22 needs to be validated with RAN WG2 and RAN WG3, due to RAN impacts.

-	Solution 13 &37 uses UPF to enforce the DL slice level bitrate. They have no RAN impact. These solutions will require to select the same SMF/PCF and UPF for all the PDU Sessions within the slice. Solution 20 method 1 also required to select same PCF for all PDU sessions within the slice. It is not necessary to introduce such limitation.

-	Distribution based solutions, i.e. Solution 20 method 2 & Solution 21, let a centralized NF distribute the SMBR into pieces (i.e. Session AMBR and/or MFBR). They have no RAN impact. However, solutions do not explain how to resolve the fact that since the SMBR is distributed into Session-AMBRs, the aggregated SMBR enforced may be smaller than the SMBR, as such the SLA would not be fulfilled, as the UE will be throttled while SMBR is not fully consumed. The situation could be worse when a large amount of PDU Sessions exist as the SMBR is distributed over more Session AMBR.

9	Conclusions

9. x	Interim Conclusion for Key Issue #3

RAN is enhanced to enforce the SMBR (Slice Maximum Bitrate) for both GBR QoS flow and non GBR QoS flow of the UE within the slice. Solution#22 is used as the baseline for normative work.

Editor´s note:	Final conclusion depends on feedback from RAN WG2/RAN WG3.

/*******************End of Change****************/