SA WG2 Meeting #141E (e-meeting)	S2-2007941

Oct 12 – Oct 23, 2020, Elbonia	(merger of S2-2006833 and S2-2006893)

Source:	CATT, vivo

Title:	KI #4 evaluation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the evaluation for Key Issue #4.

- Discussion

For Key Issue #4 "QoS level support for Multicast and Broadcast communication services", there are the following solutions in the TR:

-	Solution #3, #4, #17, #35, #36 and #37.

Evaluations are made on these solutions.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change (all new text) *****

7.x	Key Issue #4: QoS level support for Multicast and Broadcast communication services

Solution #3, #4, #17, #35, #36 and #37 are proposed to address Key Issue #4.

Solution #3 and #4 mainly address KI#1 "MBS session management", but also address the following aspects for KI#4:

- Multicast QoS flow(s) management within a multicast session context, and mapping to radio bearer.

- Association between multicast QoS flows and unicast QoS flows.

Solution #17 and #36 propose the QoS model and general handling (incl. policy provision, QoS enforcement, etc.) for 5G MBS services based on the QoS framework of TS 23.501 [2] clause 5.7. Both solutions propose that the PCF performs policy control for MBS (e.g. QoS control per MBS session) if dynamic PCC is deployed. Solution #36 also introduces new QoS parameter (i.e. MBS Session-AMBR) for MBS.

Solution #35 proposes that the AF can set specific QoS requirements (e.g. priority) for the MBS service data flows of a specific UE, which is sent to the 5GC and then forwarded to the NG-RAN.

Solution #37 proposes that 5GC and NG-RAN enforce MBS QoS by performing UEs and MBS QoS flow association based on UE interests in MBS services.

Solution #3 and #4 propose to use one QoS flow for a MBS session, and can use multiple QoS flows for a MBS session, and if individual delivery method is used, propose to use dedicated QoS flow for multicast in the PDU session associated with the MBS session. Solution #4 also proposes that shared QoS flow for multicast and unicast can be used in the PDU session.

Support one QoS flow for a MBS session is mandate, while support multiple QoS flows for a MBS session has advantage that allows a MBS session has multiple IP traffic with different QoS requirement.

***** End of Changes *****