3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-200xxxx

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #4, evaluations and conclusions

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces evaluations and conclusions on boarding and provisioning for SNPN subscription for KI#4

1. Introduction/Discussion

This contribution aims to provide the evaluations and conclusions for the Solution #5, #6, #27, #29, #30, #31, #33, #34, #35, #36, #37, #39 and #40. These solutions address the key issue #4, especially for provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access. The following table provides an overall analysis and comparison for these solutions in the following dimensions:

- UP based solution or CP based solution or support both

- Primary authentication and authorization: whether this is specified and if yes, which entity is responsible for the primary AA

- Onboarding registration: whether the functionality is specified to restrict only for onboarding and remote provisioning

- Configuration PDU session: whether the PDU session is specified to provide restricted connectivity to the PS

- O-SNPN discovery: whether discovery and selection of the onboarding SNPN is specified

- subscription provisioning at UE: whether this is specified

- subscription provisioning at SNPN: whether subscription at UDM of the SO-SNPN is specified

| Solution | UP or CP based or both | primary AA | Onboarding registration | configuration PDU Session | O-SNPN discovery | subscription provisioning at UE | subscription provisioning at SNPN | remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol#5: UE Onboarding and provisioning for an SNPN | UP-based solution | By DCS | Support | Support | Support | Support | Not specified | Address PS discovery |
| Sol#6: Control Plane-Based UE Onboarding and Provisioning Solution | CP-based solution | Not specified | Support | NA | Not specified | Support | Support |  |
| Sol#27: Common UP/CP onboarding solution for SNPN | both | By DCS/PS | Support | Support | Support | Support | Support | UP or CP decision |
| Sol#29: Discovery of the Onboarding Network | Not specified | Not specified | Not specified | Not specified | Support | Not specified | Not specified |  |
| Sol#30: UE Onboarding via Control Plane | CP-based solution | By DCS | Support | NA | Not specified | Support | Support | PS needs subscribe to UE events at O-SNPN |
| Sol#31: UE onboarding and provisioning for SNPN subscription | UP-based solution. | By DCS or PLMN | Support | Support | Support | Support | Not specified | the non-3GPP Identities/credentials are provisioned |
| Sol#33: UE Onboarding and remote provisioning based on eSIM | UP-based solution | By PLMN | Support | Support | NA | Support | Not specified | UICC is used at UE |
| Sol#34: PLMN assisted On-boarding Procedures | both | By PLMN | NA | NA | NA | Support | NA | SNPN subscription is updated at the PLMN UDM, this is trigged by UE location event |
| Sol#35: Solution for On Boarding for SNPN Compatible with SO's Existing Provisioning Server | both | By SO-SNPN | Support | Support | Support | Support | Support | SO-SNPN can also support the DCS |
| Sol#36: Providing provisioning details to UEs using local and central provisioning server | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Architecture for PS (Local PS, Central PS) |
| Sol#37: UE Onboarding and remote provisioning for SNPN | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Architecture for OPF (merging PS and DCS) (vOPF, default OPF, target OPF) |
| Sol#39: UE onboarding using PLMNs | UP-based solution | By PLMN | Support | Support | NA | Support | Support |  |
| Sol#40: Solution for UE onboarding and remote provisioning | UP-based solution | Not specified | Not specified | Support | Support | Support | Support |  |

Proposal: Provides evaluations and updates to the conclusions

2. Text Proposal

It is proposed to capture the following changes vs. T R 23.700-07 V0.5.0.

# * * * * First change * * * *

7.4	Key Issue #4: UE Onboarding and remote provisioning

7.4.X	Evaluations for SNPN case

This clause provides evaluations for the solutions that address the SNPN case: provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access, among these solutions:

-	Sol#31 proposes to use the UP-based approach, it allows the UEs without subscription to the desired SO-SNPN to temporarily register to an ON that supports UE onboarding and to establish a restricted PDU Session for provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access.

-	In the case of ON is PLMN or PNI-NPN, UE selects and registers to the onboarding PLMN using default operator profile (subscription and credentials to the onboarding PLMN for the UE to be "uniquely identifiable and verifiably secure"), and the AMF restricts this registration only for onboarding service only based on UDM indication, SMF restricts the PDU session for remote provisioning only.

-	In the case of onboarding 5G is SNPN, UE selects and registers to the onboarding SNPN using broadcast system information and default configuration (including default UE credentials for the UE to be "uniquely identifiable and verifiably secure"), and the AMF restricts this registration only for onboarding service only based on local configuration and UE indication, SMF restricts the PDU session for remote provisioning only based on local configuration.

In both cases, the UE initiates the de-registration at onboarding 5G network after finishing the remote provisioning or the onboarding 5G network initiates the de-registration, e.g. when the time is up.

-	Sol#35 has following benefits for on-boarding participants:

-	For device vendor: the solution does not mandatory ask for a DCS built by device vendor which reduces cost for device vendor. The solution supports DCS if device vendor has capability to maintain it, device vendor only need to have an agreement with SO-SNPN and does not need an agreement with O-SNPN. The solution support manual or automatic network selection without limitation of terminal model, the UEs could have input interface or not.

-	For O-SNPN: The solution has little impact on RAN side for broadcasting on-boarding information. The solution has little configuration on core network for restrict PDU session establishment.

-	For SO-SNPN: If DCS is maintained by device vendor, the SO-SNPN has interface with DCS, the architecture is similar with architecture of key issue#1. Thus, the similar logic can be reused. If DCS is co-located in SO-UDM, the similar logic for primary authentication can be reused. The SO-SNPN provides configuration to the UE via CP using UPU which reuses existing R15 method, since the configuration is protected by AUSF located in SO-SNPN, the O-SNPN cannot modify that.

-	For vertical side: Vertical can reuse existing IT PS (e.g. CA), and can reuse existing CMPv2 protocol to provide non-3GPP credential to the UE via UP without update of existing IT network entity

-	Sol#5: FFS

-	Sol#6: FFS

-	Sol#27: FFS

-	Sol#29: FFS

-	Sol#30: FFS

-	Sol#33: FFS

-	Sol#34: FFS

-	Sol#36: FFS

-	Sol#37: FFS

-	Sol#39: FFS

-	Sol#40: FFS

This following table provides an overview for the solutions that address the SNPN case:

| Principle | What it solves | System impacts | Pros/Cons | Solutions |
| --- | --- | --- | --- | --- |
| Control Plane remote provisioning | Use the Control Plane remote provisioning | Impacts on AMF receiving data to be provisioned and forwarding it to the UE | Pros: efficient Cons: | Sol#6, #27, #30, #34, #35 |
| User Plane remote provisioning | Use the User Plane remote provisioning | A PDU session is established only for provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access | Pros: Cons: | Sol#5, #27, #31, #33, #34, #35, #39, #40 |
| Primary authentication and authorization towards DCS via O-SNPN | O-SNPN supports the primary authentication and authorization towards DCS for UE using default credential | O-SNPN determines “onboarding registration” and interacts with the DCS to validate the UE | Pros: Cons: | Sol#5, #27, #30, #31 |
| Primary authentication and authorization towards DCS via SO-SNPN | SO-SNPN supports the primary authentication and authorization itself or towards DCS for UE using default credential | SO-SNPN interacts with the DCS to validate the UE | Pros: Cons: | Sol#35 |
| Primary authentication and authorization at PLMN | PLMN performs the primary authentication and authorization for UE using default operator profile | default operator profile is used | Pros: reuse existing mechanism Cons: | Sol#31, #33, #34, #39 |
| PS discovery | Discovery and selection of the target PS/SO-SNPN | Onboarding network discovers and selects the target PS or SO-SNPN | Pros: Cons: | Sol#5, #27, #30, #31, #36, #37, #39 |
| DCS discovery | Discovery and selection of the target DCS | Onboarding network discovers and selects the target DCS | Pros: Cons: | Sol#5, #27, #30, #31, #35 |
| O-SNPN discovery | Discovery and selection of the O-SNPN | UE discovers and selections the O-SNPN based on RAN broadcast information | Pros: Cons: | Sol#5, #27, #29, #31, #35, #40 |
| Subscription provisioning at UE | Store the NPN credentials (i.e. for primary authentication) and other information to enable SNPN access at ME or USIM/UICC | Once the UE receive the new NPN credentials (i.e. for primary authentication) and other information to enable SNPN access, it stores it at ME or USIM/UICC | Pros: Cons: | Sol#5, #6, #27, #29, #30, #31, #33, #34, #35, #39, #40 |
| Subscription provisioning at SNPN | UDM of SO-SNPN is provisioned with UE credentials/subscription data when remote provisioning is successfully performed | PS provides the new NPN credentials (i.e. for primary authentication) and other information to enable SNPN access to SNPN UDM | Pros: Cons: | Sol#6, #27, #30, #35, #39, #40 |

7.4.Y	Evaluations for PNI-NPN case

This clause provides evaluations for the solutions that address the SNPN case: provisioning of NPN credentials for access to specific slice(s) and/or PDU Sessions offering NPN services, i.e. for Network Slice Specific Authentication and Authorization and/or secondary authentication for PDU Sessions, among these solutions:

# * * * * Second change * * * *

8.4	Key Issue #4: UE onboarding and remote provisioning

Editor's note:	These are *INTERIM* conclusions for Key issue #4.

8.4.1	Conclusions for SNPN case

UE onboarding for SNPN (Component 1 of KI#4)

-	It should be possible to support a registration procedure that enables support for UE onboarding using Default UE credentials and with an O-SNPN as the Onboarding Network (ON).

Editor's note:	In order to support UE onboarding using Default UE credentials and O-SNPN as the Onboarding Network (ON) the distribution of security functions when primary authentication is used should be decided by SA WG3, e.g. whether and how to support the primary authentication based on default credential in case DCS is deployed or not.

-	It should be possible that one SNPN can take the role of both Onboarding Network (ON) and SO (Subscription Owner), and it should be possible that the ON and SO are different SNPNs i.e. O-SNPN and SO-SNPN.

Editor's note:	DCS is potentially introduced to authenticate a UE with default UE credentials or provide means to another entity to do it. There are two potential mechanisms for DCS to authenticate the UE. 1) DCS interacts with O-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is not involved in the authentication procedure. As a result, the SO-SNPN is not aware of the result of authentication procedure but perform remote provisioning. 2) DCS interacts with SO-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is involved in the authentication procedure. As a result, the SO-SNPN is aware of the result of authentication procedure and perform remote provisioning. SA WG3 needs to evaluate the two above mechanisms from security perspective and provide feedback.

Editor's note:	The decision on whether primary authentication is required during initial access to the O-SNPN is dependent on SA WG3 feedback; until this feedback is received, it is assumed that such authentication is required.

-	The SNPN, which directly interacts with DCS, determines the corresponding DCS identity or address/domain based on the input from the UE.

Editor's note:	It is FFS whether some alternative way to identify the DCS is to be added.

-	The NG-RAN of the Onboarding network includes information in the SIB so that the UE can discover and select an appropriate O-SNPN.

Editor's note:	whether an explicit indication towards the network from the UE is used indicating that the registration is for onboarding is FFS.

Editor's note:	Whether new RRC information is needed or existing RRC information can be used (e.g. NSSAI information) for an AMF selection during Registration procedure is FFS.

-	Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) is already possible.

-	Onboarding network should support functionality to restrict usage to only on-boarding service.

-	When Onboarding network is a PLMN, the functionality to restrict usage to only on-boarding service is activated for the UE by AMF based on received default operator profile from the UDM;

-	When Onboarding network is a O-SNPN, the functionality to restrict usage to only on-boarding service is activated for the UE by AMF based on UE indication (e.g., onboarding registration type) or local policy.

-	The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of onboarding or based on timer configured for on-boarding service.

Remote provisioning for SNPN credentials (Component 2 of KI#4)

-	Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SNPN credentials can be transmitted to UE via UP connectivity. The UE shall be configured with Default credentials in USIM to register with a PLMN where the UE can register with the Default credentials in order to communicate with the provisioning server;

-	When User Plane is used for provisioning of SO-SNPN credentials, the User plane remote provisioning protocol used and how the UE downloads the SNPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA2 and uses industry developed mechanisms;

-	Control Plane remote provisioning based on UE Parameters Update Procedure as defined in TS 23.502 [6] can be used for provisioning of credentials and other information to enable access to SO-SNPN;

Editor's note:	SA WG3 feedback will need to be taken into account for including of the CP based provisioning.

-	For the provisioning of IMSI accompanied by AKA credentials, GSMA RSP is used, Provisioning Server (PS) can provision the credential to UE over User Plane (UP) connectivity;

-	For the provisioning of Non-3GPP credentials, the credentials can be provided to UE over UP connectivity;

-	It shall be possible to pre-configure the Provisioning Server (PS) address, SO-SNPN identity, neither or both DNN/NSSAI used to access PS on the UE, and it shall also be possible that the O-SNPN provides the PS address and DNN/NSSAI used to access PS to the UE after successful authentication and authorization. The PS address and DNN/NSSAI used to access PS from the O-SNPN shall be integrity protected. The PS address and DNN/NSSAI used to access PS provided by the network is prioritized, if configured and overrides any PS address, and/or DNN/NSSAI used to access PS stored in the UE. Configuration of PS address and DNN/NSSAI used to access PS from the onboarding configuration data to the UE can be supported using one of the following methods:

Editor's note:	the need to pre-configure the S-NSSAI and DNN in the UE and to send it to the UE is FFS as default DNN and S-NSSAI could be used for provisioning to an SNPN. Also, if needed, whether to progress one or more options into normative phase is FFS.

a.	SMF may deliver onboarding configuration data as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10).

b.	Alternatively, onboarding configuration data may be configured in the UE during Registration Procedure using UE Route Selection Policy (URSP) that may be subject UE capabilities. As part of UE initial registration (based on received UE capability information) AMF indicates to PCF that UE has requested restricted/provisioning registration. The PCF may initiate UE Policy delivery using UE Route Selection Policies (URSP), for instance to trigger UE after successful registration to request establishment of specific type of PDU Session limited to onboarding purposes only.

c.	In addition, onboarding configuration data may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area.

-	It is assumed that the UDM (or AAA) of SO-SNPN is provisioned with UE credentials/subscription data when remote provisioning is successfully performed.

Editor's note:	Whether the specific procedure for provisioning of UDM (or AAA) with the UE credentials/subscription data is in scope of 3GPP is FFS.

-	When User Plane is used for provisioning of SO-SNPN credentials, a restricted PDU session is supported to be dedicated for the remote provisioning based on local configuration or UDM subscription. Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address, the device uses a well-known FQDN to perform PS discovery.

NOTE:	SA WG3 may evaluate these mechanisms and provide guidance on appropriateness of use for SNPNs.

8.4.2	Conclusions for PNI-NPN case

UE Onboarding for PNI-NPN (Component 1 of KI#4)

-	No enhancement for the UE onboarding (component 1 of KI#4) with PLMN credentials used for primary authentication and PLMN network selection are needed for the case of PNI-NPN credentials provisioning.

Remote provisioning for PNI-NPN credentials (Component 2 of KI#4)

-	At least network initiated remote provisioning of credentials to allow access to PNI-NPN services should be supported in Rel-17;

-	Both procedures using Control Plane and using User Plane protocols after establishing PDU session shall be enabled for remote provisioning the PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication;

Editor's note:	SA WG3 feedback for the suitability of the procedure will need to be taken into account.

Editor's note:	whether an extra security layer for protection of credentials between PS and UE is needed should be decided by SA WG3.

-	For User Plane remote provisioning:

-	The protocol for provisioning of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication, i.e. how the UE download the NPN credential from the PS after PDU session establishment in PNI-NPN, is out of scope of SA WG2;

-	The PS address and DNN/NSSAI used to access PS may be provided to the UE during or after the Registration procedure;

Editor's note:	How the PS address is provided to the UE is FFS.

Editor's note:	The vertical may verify the UE before PNI-NPN credential is provisioned to UE, and how this is done should be decided by SA WG3.

-	Upon successful remote provisioning of the UE, the UE Subscription Data in the UDM/UDR may be updated to enable the access to the PNI-NPN.

Editor's note:	for PNI-NPN credentials remote provisioning, whether the 3GPP operator could decide to update the UE Subscription Data (e.g., S-NSSAI, DNN, CAG information) in the UDM/UDR used to access to the PNI-NPN based on the input from the vertical which may be outside 3GPP operator domain should be decided by SA WG3.

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *