SA WG2 Meeting #141E (e-meeting)	S2-2006835r01

Oct 12 – Oct 23, 2020, Elbonia	(merger of S2-2006835, S2-2006895 and eval in S2-2007465/6973)

Source:	CATT

Title:	KI #7 evaluation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the evaluation for Key Issue #7.

- Discussion

For Key Issue #7 "Reliable delivery method switching between unicast and multicast", there are the following solutions in the TR:

-	Solution #11, #12, #18, #22~#31, #39 and #40.

These solutions can be categorized as follows:

- Switching between 5GC Shared and Individual MBS traffic delivery methods: Solution #23, #24, #25, #28, #30, #31 and #40 (single RAN case).

- Switching between PTP and PTM delivery methods: Solution #18, #22 and #25.

- Inter-RAN handover / UE mobility related delivery method switching: Solution #11, #12, #26, #27, #29, #39 and #40 (dual RANs case).

Evaluations are made on the solutions for each category.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change (all new text) *****

7.X	Key Issue #7: Reliable delivery method switching between unicast and multicast

Solution #11, #12, #18, #22~#31, #39 and #40 are proposed to address Key Issue #7 in the present document. These solutions can be categorized as follows:

- Switching between unicast and multicast: Solution #23, #28, #39 and #40.

- Switching between 5GC Shared and Individual MBS traffic delivery methods: Solution #23, #24, #25, #28, #30, #31 and #40 (single RAN case).

- Switching between PTP and PTM delivery methods: Solution #18, #22 and #25.

- Inter-RAN handover / UE mobility related delivery method switching: Solution #11, #12, #26, #27, #29, #39 and #40 (dual RANs case).

Following are the evaluations on the solutions for each category:

1) Switching between unicast and multicast

Solution#23, #39, and #40 propose that UE and AF interacts via application layer for the unicast and multicast mode switching, solution#23 proposes that UE and AF interacts via NAS signalling and N6 interface while solution#39 and #40 propose to use user plane.

Solution#28 and 40 propose that UE and 5GS interacts for unicast and multicast mode switching.

2) Switching between 5GC Shared and Individual MBS traffic delivery methods

A comparison between solutions for switching between 5GC Shared and Individual MBS traffic delivery methods is shown in Table 7.X-1.

Table 7.x-1: Comparison of solutions for switching between 5GC Shared and Individual MBS traffic delivery methods

|  | Sol#23 | Sol#24 | Sol#25 | Sol#28 | Sol#30 | Sol#31 | Sol#40 (single RAN case) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Delivery method switching triggers/ criteria | - Application level trigger. - QoS requirements. - UE MBS subscription changes. - Handover or reselection to target RAN supporting or not supporting MBS. - Predefined threshold (i.e. UE number) | - Content/ Application Server decisions. - Consumption reports sent by the UE. - Handover. | - Number of the UEs receiving the multicast service in a certain multicast group | - Number of devices receiving the specific content. - Number of devices in a specific location area related to the specific content. -Measurement of multicast signalling strength | Whether there're multiple PDU Sessions corresponding to the same TMGI according to the multicast session information in all UE AS Context | - MBS service related network policy, (e.g. predefined traffic threshold, network status /performance changes); - UE subscription for MBS service changes. - UE moving in or out of MBS Service Area; - Change of user preferences | - UE MBS subscription changes. - Number of UEs requesting the same content in a specific area |
| NFs triggering delivery method switching | UE, SMF, MSF | RAN, MBSF, Content/Application Server | (MB-)SMF | UE, (MB-)SMF | RAN | UE, RAN, AMF, SMF, UDM, PCF, AF | UE, SMF |
| Procedures for delivery method switching | UE or network initiated PDU Session Establishment or PDU Session Modification procedure | MBSF initiated session update procedure | SMF/UPF initiated new procedure | UE initiated User Multicast Session Join / User Multicast Context Update procedure | RAN triggered delivery method switching from individual to shared | PDU Session Modification procedure | - UE triggered PDU Session Modification procedure. - SMF triggered new procedure |
| Baseline architecture used | 1 | 2 (NOTE 2) | 2 (NOTE 2) | 1 | 1 & 2 | 1 | 1 |
| NOTE 1: Though different architectures are used, architecture 1 and 2 share common functionalities, e.g. in AF, RAN, etc. |  |  |  |  |  |  |  |

Based on which NF(s) triggers the switching between 5GC Shared and Individual MBS traffic delivery methods, the solutions are further evaluated as follows:

- Triggered by the UE

Solution #23, #28, #31 and #40 propose that the UE can trigger the switching between 5GC Shared and Individual MBS traffic delivery methods, due to trigger conditions including changes of user preferences or UE MBS subscription, moving into/out of the multicast area, 5MBS capability of target gNB during handover, radio signalling condition, etc.

- Triggered by the RAN

Solution #24, #28, #30 and #31 propose that the RAN can trigger the switching between 5GC Shared and Individual MBS traffic delivery methods, due to trigger conditions including UE consumption reports, 5MBS capability of target gNB during handover, number of UEs/PDU Sessions for the same MBS service, etc.

- Triggered by the (MB-)SMF/PCF/UDM/AMF

Solution#23, #25, #28, #31 and #40 propose that the (MB-)SMF, PCF, UDM or AMF triggers the switching between 5GC Shared and Individual MBS traffic delivery methods.

- Solution #23, #25, #28 and #40 proposes that the (MB-)SMF triggers the switching between 5GC Shared and Individual MBS traffic delivery methods based on number of the UEs receiving the multicast service in a multicast group or in a specific location area (i.e. a UE count threshold).

- Solution #31 proposes that switching between 5GC Shared and Individual MBS traffic delivery methods is done via (MB-)SMF initiated session modification procedure, triggered by the (MB-)SMF’s detection on changes of network status or performance, by the PCF based on its policy decision or upon AF requests, by the UDM based on change of delivery method for the subscribed service associated with the MBS session, or by the AMF upon detecting that the UE moves in or out of MBS service area.

- Triggered by the MSF/MBSF

- Solution #23 proposes that the MSF can trigger the switching between 5GC Shared and Individual MBS traffic delivery methods, based on the count of UE subscribed and utilizing the same content

- Solution #24 introduces the MBSF can do the delivery method switch based on different optimization criteria (e.g. consumption reports, UE population in certain area, etc.).

- Triggered by the AF

Solution #24 proposes that the content/application server can send requests to the (MB-)SMF via the MBSF/NEF to trigger the switching between 5GC Shared and Individual MBS traffic delivery methods. Similarly, Solution #31 proposes that the PCF can, based on AF request, send requests to the (MB-)SMF to trigger such switching.

2) Switching between PTP and PTM delivery methods

All the three solutions, i.e. Sol #18, #22 and #25 propose that the NG-RAN decides on using PTP or PTM delivery method over radio interface for MBS data, and may dynamically switch between these two delivery methods.

In addition, Sol#18 proposes that the CN provides MBS assistance information to the NG-RAN to assist in the selection of PTP or PTM delivery method. This is pending for the confirmation of RAN WG.

3) Inter-RAN handover / UE mobility related delivery method switching

A comparison between solutions for inter-RAN handover / UE mobility related delivery method switching is shown in Table 7.X-2.

Table 7.X-2: Comparison of solutions for Inter-RAN handover / UE mobility related delivery method switching

|  | Sol#11/#12 | Sol#26 | Sol#27 | Sol#29(/#15) | Sol#39 | Sol#40 (dual RANs case) |
| --- | --- | --- | --- | --- | --- | --- |
| Procedures for delivery method switching | Enhanced Xn HO: MB Session resource setup during Xn Handover preparation or execution phase Enhanced N2 HO: MB Session resource setup during N2 Handover preparation phase May involve switching between 5GC Shared and Individual MBS traffic delivery methods based on 5MBS capability of target gNB. | Enhanced Xn/N2 HO: - Forwarding multicast data via mapped QoS flow over PDU session from source to target RAN. - Associated PDU session is used for lossless handover. | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, depending on whether target RAN supports MBS. - Forwarding MBS data to Target RAN supporting MBS. - Associated PDU session is used for lossless handover or unicast fallback when source RAN or target RAN does not support MBS. May involve switching between 5GC Shared and Individual MBS traffic delivery methods based on 5MBS capability of target gNB. | Enhanced Xn/N2 HO with UE initiated NAS multicast session join. May involve switching between 5GC Shared and Individual MBS traffic delivery methods based on 5MBS capability of target gNB. | Multicast->Uni-cast:UE initiated PDU Session establishment or activation; Unicast->Multi-cast: UE initiated PDU session release or deactivation. | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, based on UE request of unicast or multicast mode in application layer. - Forwarding multicast data over PDU session |
| Baseline architecture used | 2 | 2 | 1 | 1 |  | 1 |

NOTE:	The evaluation may depend on the feedback from RAN WGs and the conclusion of relevant solutions to other KIs (esp. KI#1).

Following list illustrates the technical aspects from candidate solutions addressing KI#7 delivery method switching.

Consideration on Unicast-multicast mode change:

Proposal:	Unicast-multicast mode change is needed

Alternatives:	Alt 1: UE and AF interacts via application layer (Sol#23, 39, 40)
The interaction may over UP or CP

Alt 2: UE and 5GS interacts for unicast-multicast mode change (Sol#28, 40)
MB-SMF or gNB notifies the UE to perform the mode change operation

Consideration on RAN specific delivery method switching:

Proposal:	Support RAN specific delivery method switching

Alternatives:	Alt 1: PTP/PTM switching without CN involved (Sol#22)

Alt 2: CN provides assistant information to RAN for PTP/PTM switching determination (Sol#18)
Need RAN coordination

Editor's note: Whether gNB needs assistant information from CN for PTP/PTM switch needs RAN coordination.

Consideration on 5GC specific delivery method switching:

Proposal:	Support 5GC specific delivery method switching

Alternatives:	Alt 1: RAN triggers the switch

> Alt 1.1: Based on the UE count under the RAN node (Sol#24, 28)

> Alt 1.2: Based on 5MBS capability of target gNB during handover (Sol#27, 29)

Alt 2: MB-SMF triggers the switch

> Alt 2.1: Based on the UE count per a MB session (Sol#25, 28)

> Alt 2.2: Based on 5MBS capability of target gNB during handover (Sol#27, 29)

Alt 3: MBSF triggers the switch based on the UE count per a MB session (Sol#24)

Alt 4: AF triggers the switch based on the UE count per a MB session (Sol#24)

Alt 5: UE triggers the switch based on the radio signalling condition (Sol#28, 31)

Consideration on 5GC specific delivery method switching during handover:

Proposal:	Support 5GC specific delivery method switching during handover between RAN node support 5MBS and RAN node not supporting 5MBS

Alternatives:	Alt 1: For 5GS Shared to Individual MBS traffic delivery method switching, uses previously activated PDU session associated with the MB session at target (Sol#27)

Alt 2: For 5GS Shared to Individual MBS traffic delivery method switching, uses any previously activated PDU session for MB traffic forwarding if needed (Sol#29)
UE establishes or activates the PDU session associated with the MB session at target during handover execution phase

Alt 3: For 5GS Individual to Shared MBS traffic delivery method switching during Xn handover when source gNB is legacy, uses MBS specific QoS information (e.g. range of QFI specific to MBS) for the QoS flows for MB sessions to make target gNB aware of that user is interesting in some MB sessions (Sol#29)
Target gNB according to the indication sends N2 message to 5GC for pulling downlink MB traffic during handover preparation/execution phase if needed

Alt 4: For 5GS Individual to Shared MBS traffic delivery method switching during Xn handover when source gNB is legacy, target gNB indicates MBS capability to UE in HO CMD for indicating UE to initiate session join via target gNB after handover (Sol#29)

Editor's note: it is FFS whether inter-RAN handover without delivery method switching is evaluated in KI#1 or KI#7.

Editor's note: further evaluation is needed.

***** End of Changes *****