3GPP TSG-SA WG2 Meeting #141E	S2-2006943

12 - 23 October, 2020, Electronic

Source:	China Mobile，

Title:	KI #3A, Sol #13: Updates to remove ENs.

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract: This paper updates solution #13 and proposes to remove several editor’s note from the solution #13.

1. Introduction

Some modifications are introduced to make this solution more workable.

In R16, the RAN WG has decided that the average delay between UE and RAN can be provided to SMF. And for solution13, in order for making the jitter calculation workable for R17, the per-packet calculation is removed and the RAN calculation is depending on RAN WG. Even the jitter accuracy depending on the delay measurement and frequency, and per-packet calculation is more precious than using the averge delay for RAN side, but we need a method to support the jitter calculation. Therefore it is suggested that the current R16 QoS Monitoring for per QoS flow is supported for jitter measurement and no enhancement for RAN side.

Also for UPF, in R16 the per packet delay measurement for UPF is already supported if the reporting frequency (TS 23.503 6.1.3.21)is small than the packet sending frequency. Therefore still no new UPF impact of this solution.

Based on the above discussion, the solution 13 is updated to remove the ENs and clarify the impact for RAN,UPF.

- removing the first EN:

Editor's note:	NG-RAN impacts need to be checked with RAN WGs before conclusions are reached.

Because there is no RAN impact.

- removing the EN:

Editor's note:	Whether the NWDAF can be introduced to do jitter calculation is FFS.

There is no need to introduce NWDAF to do jitter calculation therefore this EN is removed.

- Removing the EN:Editor's note:	It is FFS whether there are other parameters should be considered for jitter calculation.

Some parameters from TS 23.503 6.1.3.21 e.g. reporting threshold or reporting frequency can be added.

- Removing the EN:

Editor's note:	Based on what the enhancement of the solution may become, AMF, SMF, UPF and NG-RAN are impacted

There is no enhancement for RAN and UPF, the AMF is impact for transferring the N2 meessage, the SMF is impacted for sending N4 rule for jitter calculation.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-20.

# * * * * First change * * * *

## 6.13	Solution #13: Mechanism for AF requesting 5G network jitter

### 6.13.1	Description

This solution is for key issue#3A, which addresses Exposure of deterministic QoS including aspects related to:

-	Ability for AF to request absolute delay and jitter requirements, and mechanisms to enable the PCF to determine the 5GS QoS parameters based on the requirements received from AF.

-	Ability for AF to indicate periodicity, burst size and burst arrival time (as defined in Rel-16 for TSC Assistance information), optionally burst spread (variation of burst arrival time for DL traffic resulting from jitter on N6, if applicable) along with Time Domain (reference for these parameters) associated with these parameters to the NEF.

This solution proposes a mechanism for the AF to request jitter and the PCF is required (via NEF if untrusted AF) to  calculate an estimation of jitter based on delay values using QoS monitoring.

The main idea is that the AF provides request for jitter, and 5G system calculate the jitter based on a group of E2E packet delay between UE and PSA UPF. After PCF getting a group of E2E delay data in measurement period, the PCF may use the IETF RFC1889 method to calculate the jitter and send the jitter value to AF. Considering R16 QoS monitoring mechanism, only average packet delay measurement in RAN has been defined and tha the PSA UPF calculates the UL/DL packet delay between UE and PSA UPF based on the received RAN part of UL/DL packet delay result and the calculated UL/DL packet delay between RAN and PSA UPF.  , in order to introduce the jitter calculation mechanism in R17, there is no enhancement for RAN and UPF, and the average delay from RAN can be used to calculate an estimation of the jitter..

AF sends a request for jitter calculation by using Nnef_AFsessionWithQoS_Create request message to NEF, the indication of jitter requirement, jitter measurement period, sample frequency, reporting threshold or reporting frequency(TS 23.503 6.1.3.21) may be included in the request, and are sent to PCF via NEF.

When PCF accepts the jitter requirement, PCF generates the QoS Monitoring policy for a service data flow, based on the request from AF. In the QoS Monitoring PCC rule, the QoS monitoring period can be the same as jitter measurement period. Then PCF provides the QoS Monitoring policy to the SMF.

When SMF gets the PCC rules for jitter calculation, the SMF sends the QoS Monitoring request to the PSA UPF via N4 and NG-RAN via N2 signalling to request the QoS monitoring between PSA UPF and NG-RAN, as described in TS 23.501 [2] clause 5.33.3. When the reporting trigger(s) is satisfied, e.g. the reporting period expires, or the PDU Session is released, the UPF reports the calculated packet delay value(s) to the SMF, and the SMF sends the reports to the PCF. PCF may use the IETF RFC1889 method to calculate an estimation of jitter based on some delay measurement samples, using QoS monitoring.  , and then sends the estimated jitter value back to AF.

The sample packet frequency for jitter calculation may comply with Poisson sampling, because it ensures an unbiased and uniformly distributed sampling during jitter measurement period. However, alternate sampling methodologies are possible. For example, continuous sampling of a constant bit rate stream (i.e., periodic packet transmission) is a possibility. (IETF RFC 3393)

### 6.13.2	Procedures

The mechanism for AF requesting jitter requirements can be described in the Figure 6.13.2-1.

Figure 6.13.2-1: AF requesting jitter requirements

0.	The PDU session has been established.

1.	The AF sends a QoS monitoring request to the NEF by Nnef_AFsessionWithQoS_Create request message. The request message includes indication of jitter, jitter measurement period, sample frequency.

2.	The NEF authorizes the AF request and sends the jitter calculation parameter in the AF request message to the PCF through the Npcf_PolicyAuthorization_Create request.

3.	PCF generates the authorized QoS Monitoring policy based on the AF request, measurement period, sample frequency, and triggers SMF to initiate PDU Session Modification procedure by sending Npcf_SMPolicyControl_UpdateNotify request.

4a.	SMF activates the end to end UL/DL packet delay measurement between UE and PSA UPF for the QoS Flow or GTP-tunnel during the PDU Session Modification procedure. SMF sends QoS Monitoring request to AMF by Namf_Communication_N1N2MessageTransfer message, and AMF sends QoS monitoring request to NG-RAN through N2 interface.

4b.	The SMF sends a QoS Monitoring request to the PSA UPF via N4

5.	AMF sends QoS monitoring request to NG-RAN through N2 interface.

6.	The NG-RAN initiates the measurement of UL/DL packet delay on Uu interface based on the QoS Monitoring request from SMF, and NG-RAN performs delay measurement based on the existing procedures in 5.33.3.2 in TS 23.501 for QoS monitoring for URLLC

7.	NG-RAN reports the UL/DL packet delay result on Uu interface to the UPF in the UL packet data as defined in TS 23.501 clause 5.33.3.2 .

8.	UPF calculates the end to end packe delay between NG-RAN and PSA UPF, as described in TS 23.501 [2] clause 5.33.3, and reports the packe delay value(s) to SMF.

9.	The SMF reports the obtained packet delay value(s) to the PCF by sending an Npcf_SMPolicyControl_UpdateNotify response message.

10.	When PCF gets the packet delay value(s), it calculate delay jitter based on packet delay value(s), jitter measurement period, sample frequency. Then PCF replies the delay jitter value to NEF by using Npcf_PolicyAuthorization_Notify message. The jitter value provided to the AF is “estimated jitter”.

11.	NEF sends jitter and other QoS monitoring data to AF through Nnef_AFsessionWithQoS_Notify.

### 6.13.3	Impacts on services, entities and interfaces

PCF:

-	Receiving the AF estimation of jitter requirement indication.

-

-	Calculating the estimated jitter value based on QoS monitoring E2E latency value(s).

There is no new functionality requirement for RAN and UPF.

# * * * * End of changes * * * *