3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2006990

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #6: Conclusion

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution contains the conclusion of KI#6.

1. Introduction/Discussion

According to the evaluation in S2-2006990, it is proposed to document the related interim conclusion for KI #6.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

S2-2006898 (vivo) and S2-2006983 (LGE) are merged into this paper.

# * * * * First change (All Text New)* * * *

8.X	Conclusions for Key Issue #6: Local MBS service

8.X.1	Interim conclusions

Editor's Note: This conclusion may not be complete, additional concept/details/characteristics could be added.

To support local MBS services for both multicast and broadcast, the following architecture deployment options can be considered.

-	To locate the user plane of MBS CN functions (e.g. MB-UPF) closer to the NG-RAN.

-	To locate the the user plane of MBS CN functions (e.g. MB-UPF) as well as the control plane of MBS CN functions (e.g. MB-SMF) closer to the NG-RAN.

The following interim agreements are considered to support local MBS services as baseline for normative work:

-	For the case that the multicast service is only available within a limited area, the UE shall be able to get the service area information of the multicast service via NAS signalling, to enable the UE to trigger the session join procedure when needed.

-	The case that PLMN topology is agnostic to AF (e.g., AF is not aware of cell or TA information) shall be supported.

-	The 5GC shall be able to enforce the area restriction for accessing a local MBS service.

Editor's Note: The conclusion of KI#6 should align with the conclusion of KI#1.

Editor's Note: Alignment based on progress/decision in RAN WGs will be performed as part of the normative phase.

# * * * * End of changes * * * *