3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2- 2007029

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Update of Sol#3 to add Rx UE initiated PC5 DRX negotiation

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / R17

Abstract: This contribution proposes to this contribution updates the Sol#3 to add Rx UE initiated PC5 DRX negotiation.

1. Introduction/Discussion

Sol#3 gives a negotiation of a PC5 DRX for Unicast communication, but it is a little misleading, as the current procedure seems that the PC5 DRX is only initiated by the Tx UE. However, the PC5 DRX is to save the power from the perspective of Rx UE, so at least the PC5 DRX is an Rx UE basic parameter, i.e. the PC5 DRX is determined by the Rx UE for its data reception and notified to the Tx UE for its data transmission.

The V2X Service Types are assumed to be same and bi-directional for the PC5 Unicast Link1 between UE1 and UE2. But the Tx UE1/Rx UE2 each may have many PC5 Unicast Links with same/different UEs and a UE can perform as both a Tx UE and Rx UE in the unicast communication, so the PC5 DRX of Unicast Link1 for Rx UE2’s reception may be different with that for Tx UE1’s reception.

Consequently, maybe two PC5 DRXs should be negotiated for one PC5 Unicast Link, i.e. one PC5 DRX on each direction. The Sol#3 proposes the following additional principles:

- The Unicast PC5 DRX may be negotiated between Tx UE and Rx UE in each direction. On each direction of PC5 Unicast Link, the Unicast PC5 DRX is determined by the Rx UE.

- The Unicast PC5 DRX in different directions may be same or different, although the PC5 Service Types are assumed to be same in two directions.

- The Unicast PC5 DRX in different directions can be negotiated in one procedure initiated by the UE that triggers the PC5 Services firstly.

- When the Unicast PC5 DRX of one direction for the Rx UE need to be updated, e.g. the Rx UE has establish other Unicast links with other UEs or new V2X services occurs, the Rx UE should initiate the update procedure toward the Tx UE.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.776.

# * * * * First change * * * *

6.3	Solution #3: Solution for KI#1 to negotiate a PC5 DRX for Unicast communication

6.3.1	Functional Description

This solution is for Key Issue #1, "Support of QoS aware NR PC5 power efficiency for pedestrian UEs".

A Pedestrian UE may have multiple V2X services with different service requirements, i.e. these different V2X services have with different QoS requirements (e.g. PDB) and service pattern. PC5 DRX should be set to fulfil the requirements and can further be adjusted to coordinate with service pattern. Current PC5 QoS model is QoS Flow-based, so PC5 DRX will be negotiated during PC5 QoS Flow establishment. If only one PC5 DRX is used, the improper setting of PC5 DRX will lead to malperformance of some V2X services. To make the PC5 DRX more flexible and enable V2X services workable at most, multiple PC5 DRX for a P-UE seems reasonable.

However, a P-UE may establish multiple Unicast links with the same/different peer UEs at the same time, therefore, different PC5 DRXs set for different unicast links could be possible. Maintaining too many PC5 DRXs would be very complicated, therefore, the PC5 DRXs should be coordinated for different Unicast links in the UE.

In addition, the PC5 DRX is more like an AS layer parameter and shall be negotiated in the AS layer (as V2X layer cannot know Tx/Rx resource pool for PC5 and Uu maintained in the AS layer).

To summarize, the introduced solution mainly proposes to negotiate the Unicast specific PC5 DRX based on the related PC5 QoS parameter of PC5 QoS Flows by AS layer, and the following solution principles are highlighted:

-	The PC5 DRX relates to the PDB and service pattern, and the PC5 DRX negotiation should be triggered by the QoS Flow establishment.

-	To make the PC5 DRX more flexible and enable V2X services workable at most, PC5 DRX configuration can be negotiated for a P-UE.

NOTE:	This solution doesn't restrict the number of PC5 DRX in the configuration. It is up to RAN WGs to decide.

-	To reduce the complexities, the PC5 DRXs should be coordinated for different Unicast links.

-	The PC5 DRX is an AS layer parameter and shall be negotiated in the AS layer

-	The Unicast PC5 DRXs are negotiated between two engaged UEs. Each UE determines its own individual Unicast PC5 DRX and use it only for data reception.

-	The individual Unicast PC5 DRX is determined based on all the ongoing traffic of the UE’s Unicast Links, other than just the traffic of this current Unicast Link. Consequently, the individual Unicast PC5 DRX of each UE, which engages in the Unicast Link communication, can be different.

-	The individual Unicast PC5 DRXs of two UEs are negotiated in one procedure, which is initiated by the UE that triggers the PC5 signaling procedure.

-	When the Unicast PC5 DRX of one UE need to be updated, e.g. the UE has establish other unicast link with another UE or new V2X services occurs, the Rx UE initiates the update procedure toward the other UE.

Editor's note:	Whether and how the AS layer of UE negotiates the PC5 DRX for unicast link is up to RAN WGs to decide.

6.3.2	Procedures

Figure 6.3.2-1 illustrates the PC5 DRX negotiation for Unicast communication.

Figure 6.3.2-1: PC5 DRX negotiation for Unicast communication

1.	The P-UE1's V2X Application provides the V2X Service Type (i.e. PSID/ITS-AID), V2X Application Requirements to the V2X layer as defined in clause 5.4.1.1.2 of TS 23.287 [5].

2.	The P-UE1's V2X layer triggers the PC5 QoS Flow establishment for the Unicast link by using the Layer-2 link establishment procedure or Layer-2 link modification procedure as defined in TS 23.287 [5].

3.	The V2X layer provides the QoS Flow parameters to the AS layer for PC5 DRX determination.

4.	The AS layer determines the individual PC5 DRX parameter for the UE based on e.g. the QoS Flow parameters of V2X service message. The UE1 uses the individual PC5 DRX parameter1 for data reception from UE2.

5-6.	The AS layer performs the PC5 DRX negotiation with peer UE of the unicast link by PC5 RRC signalling and with RAN1 by Uu RRC signalling. UE2 determines the individual PC5 DRX parameter2 for data reception from UE1and returns it to UE1. The PC5 DRX parameter2 may be same or different with the PC5 DRX parameter1, e.g. if UE2 only has one Unicast Link that is the current Unicast Link with UE1, then UE2 may generate an individual PC5 DRX parameter2 that is same with DRX parameter1; if UE2 has other Unicast Links with other UEs, then UE2 may generate an individual PC5 DRX parameter2 that is different with DRX parameter1.

Editor's note:	Whether and how the AS layer of UE and NG-RAN negotiate the PC5 DRX is up to RAN WGs to decide.

7-8.	[Optional] The AS layer may provide the PC5 DRX parameter1 and DRX parameter2 to the Application layer for adaption via the V2X layer.

9.	For subsequent V2X services of each UE, if the existing PC5 DRX cannot be reused, the UE should initiate the update procedure toward the other UE(s) to negotiate a new individual PC5 DRX parameter for its data reception (see Step 5-Step 8).

6.3.3	Impacts on services, entities and interfaces

The solution has the following impact to the existing entities:

-	Pedestrian UE:

-	the AS layer negotiate the PC5 DRX between UEs based on the QoS Flow parameters and service pattern for unicast link.

-	NG-RAN:

-	adjusts the PC5 DRX and Uu DRX.

Editor's note:	Whether and how the NG-RAN adjusts the PC5 DRX and Uu DRX is up to RAN WGs to decide.

# * * * * End of changes * * * *