SA WG2 Meeting #S2-141e	S2-2007098r06

12 – 23 October, 2020, Electronic

Source:	Ericsson

Title:	KI #2, sol #20: Addressing Editor’s notes in Solution #20

Document for:	Agreement

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: We address outstanding questions regarding the relationship to release 16 VLAN handling mechanisms.

Introduction

The paper discusses the outstanding Editor’s notes for Solution 20 on CNC controlled VLAN configuration.

Relationship to release-16 VLAN support

The SMF controlled rules in the UPF are able to filter on Ethernet header fields including the VLAN ID(s), and can add or remove VLAN ID fields for an Ethernet frame under the SMF control. This may be sufficient in 3GPP-only environments where the SMF has full knowledge and control over the whole Ethernet network.

In the general case, however, an Ethernet network (which may also support TSN) is integrated into a network that may include non-3GPP components, such as fixed Ethernet segments. As is the case with TSN, the network may be centrally controlled by a CNC. It is very important for industrial deployments to be able to control the full network domain from a single central controller, and this makes it necessary to support CNC provided VLAN configuration. Such central control from a CNC is not currently supported by release 16. Note that in common deployments, it may be necessary to support dozens of VLANs due to the need for separating devices e.g., in different security zones. UE to UE communication for certain sets of devices may also be put into separate VLANs. Also, specific TSN streams may use dedicated VLANs so that they are well separated from other traffic.

In order for the CNC to be able to manage the full Ethernet network in a consistent way, it is necessary to enable the CNC to set the VLAN configuration in all bridges using the same management interface, as defined in IEEE 802.1Q Clause 12.

Note also that the VLAN configuration is closely coupled with bridge forwarding, where VLAN IDs are explicitly indicated in the forwarding tables, and there may also be multiple forwarding tables (FIDs) whose selection depends on the VLAN ID. The CNC, acting as a central controller, needs to be able to harmonize the configuration of bridge forwarding and VLANs, and for that purpose the use and implementation of CNC provided VLAN configuration and bridge forwarding including the possibility of static filtering entries are closely integrated.

For the support of CNC provided VLAN configuration which may change dynamically, the preconfigured VLAN configuration as defined for release 16 is not sufficient, and it is necessary to also enable the 5GS to realize the VLAN configuration that is provided from the CNC as defined in IEEE Std 802.1Q.

The VLAN configuration equally applies to a set of bridge ports, including both DS-TT and NW-TT ports. The release 16 SMF provided PDR/FAR rules only apply to PDU Sessions that correspond to DS-TT ports, but there are currently no PDR/FAR rules that explicitly correspond to NW-TT ports.

Proposal

It is proposed to update the solution in 23.700-20 as follows.

* * * * Start change * * * *

6.20	Solution #20: CNC controlled VLAN configuration

6.20.1	Introduction

The solution adds general support for CNC controlled VLAN configuration. This can be also used to address UE to UE communication, Key Issue #2, in practical deployments, e.g., for logical separation of one or multiple UE to UE TSN streams, for easier management of flows or for easier setup for forwarding paths. Note that the VLAN configuration support proposed here is general for all communications, including TSN and non-TSN flows that are UE to UE as well as other flows that are between UE and network, or between multiple network ports.

It is proposed that the specification supports CNC controlled VLAN configuration only by the existing interface via TSN AF already defined for the fully centralized configuration model specified by IEEE Std 802.1Qcc; therefore, MVRP is not supported or needed at DS-TT and NW-TT.

The existing release 16 specification supports Ethernet Packet Filters set by the SMF in the PDRs. However, the release 16 specification is limited to SMF controlled cases which assume that the SMF has full knowledge and control over the Ethernet network, which is not the case in many deployments that serves TSN and non TSN traffic in parallel. As another limitation, the release 16 SMF provided rules apply to the N4 sessions corresponding to the PDU Sessions and the associated DS-TT ports, while VLAN handling rules for NW-TT ports apply according to static pre-configuration at NW-TT.

The need to manage and configure the 5GS in the same way as other bridges in the network is essential in practical industrial deployments.. This solution allows the CNC to control the VLAN configuration of IEEE 802.1Q bridges in a unified way , and thereby allows the CNC to handle the VLAN configuration for 5GS bridges in the same way as fixed bridges.The need to manage and configure the 5GS in the same way as other bridges in the network is essential in practical industrial deployments.

NOTE: CNC provided VLAN configuration co-exists with SMF provided PDR/FAR rules that also act on the VLAN IDs in the Ethernet headers.

The VLAN configuration affects the configuration of bridge forwarding: the forwarding rules may also include the VLAN IDs, and the VLAN configuration may refer to a specific forwarding table. In common deployments, the use and the implementation of the VLAN configuration and bridge forwarding is closely coupled.

The VLAN configuration provided by the CNC is set on a per port granularity. VLAN configuration may be set at any port, including DS-TT as well as NW-TT ports.

VLAN configuration implies a set of functionalities listed below based on IEEE Std 802.1Q.

Table 6.20.1-1

| Configuration | Reference in IEEE 802.1Q clause | Description |
| --- | --- | --- |
| Per Bridge VLAN Configuration |  |  |
| Read Bridge VLAN Configuration | 12.10.1.1 | To obtain general VLAN information from a Bridge. |
| PVID and VID Set values | 12.10.1.2 | Set the VID value to be added to received untagged frames at a given port. |
| Acceptable Frame Types parameters | 12.10.1.3 | Used to configure whether to admit only VLAN-tagged frames, only untagged/priority tagged frames, or all frames. |
| Enable Ingress Filtering parameters | 12.10.1.4 | Whether to enable or disable ingress filtering. With ingress filtering, only ports in the member set for the given VID can admit traffic. |
| Reset Bridge | 12.10.1.5 | To reset all statically configured VLAN-related information in the Bridge to its default state. |
| VID to FID allocation | 12.10.3 | Filtering identifier for a given VID, which determines whether different VLANs use individual or shared forwarding tables. |
| FID to MSTID allocation | 12.12.2 | The FID to MSTID allocation is required to complete the VID to FID to MSTID allocation. For TSN Streams, this is used to allocate their VID to the TE-MSTID for central CNC controlled operation. (See clause 12.32.3.1 in IEEE Std 802.1Qcc-2018 for further information.) |
| Per VLAN Configuration |  |  |
| The VLAN Configuration | 12.10.2 | For setting the VLAN name and reading VLAN specific configuration. |
| Filtering Database Configuration |  |  |
| Static Filtering Entries | 8.8.1 | Defines forwarding rules based on the combination of destination MAC address and VID. Note that the use of static filtering entries is introduced and elaborated in other solutions as well, which can be used in combination with this solution. |
| Static VLAN Registration Entries | 8.8.2 | Determines whether frames are to be VLAN-tagged or untagged when transmitted; also defines member set for a VID that is used for ingress and egress filtering. |
| Dynamic Filtering Entries | 8.8.3 | Maintains the dynamically learnt addresses respecting the VID to FID allocation as configured above, i.e., whether shared or independent learning is applied among certain VLANs. |
| Default Group filtering behavior | 8.8.6 | Specifies defaults for group-addressed frames for each VID and outbound Port regarding forwarding/filtering. |

The solution reuses the mechanisms defined in Rel. 16 for the transfer of bridge and port configuration between the UPF/NW-TT, DS-TT and the TSN AF using the already specified Management Information Containers. Note that VLAN processing functionality may be set up and used even before TSN streams are established or for non-TSN traffic.

VLAN processing is a set of IEEE 802.1Q defined functionalities for Ethernet specific processing and may also be realized using N4 rules. The detailed implementation of VLAN processing is not specified; the implementation is not restricted as long as the externally observable behavior is according to the configuration.

VLAN configuration and processing is a general Ethernet functionality that is not TSN specific; however, TSN deployments often make use of this functionality. Even if there is a dependency between both, the CNC performs the VLAN configuration independent of the TSN stream configuration. For the VLAN configuration that is needed by the TSN streams, the CNC can provide the VLAN configuration before the provision of TSN stream related configuration.

Editor's note:	It is FFS how to realize the VLAN processing using N4 rules before TSN streams are established.

The solution could be combined with other solutions.

6.20.2	Functional Description

The configuration for VLAN is exchanged between the CNC and the TSN AF as part of the 5GS bridge configuration. The VLAN configuration is part of the Bridge VLAN Configuration managed object according to IEEE specifications; there is a single such managed object per Bridge, however the configuration is flexible to support different VLANs for the different ports. The TSN AF provides the information to the UPF inside the Bridge Management Information Container. Configuration can also be read from the UPF to the TSN AF and CNC in the reverse direction using the existing signaling path.

6.20.3	Procedures

-	CNC provides VLAN configuration to the TSN AF.

-	TSN AF provides the VLAN configuration inside the BMIC PMIC using the existing signaling path to the UPF/NW-TT concerned.

-	Based on the configuration, the UPF sets up the VLAN processing. The implementation of VLAN processing is not specified.

-	The VLAN configuration can also be read from the UPF/NW-TT and provided to the TSN AF and CNC.

6.20.4	Impacts on existing services and interfaces

TSN AF:

-	Forward VLAN configuration to/from the UPF/NW-TT inside BMIC

Editor’s note: support for VLAN processing for both network side and device side ports is FFS.

* * * * End change * * * *