3GPP TSG-SA WG2 Meeting #141E 	S2-2007133r12

12-23 October 2020, Electronic meeting

Source:	Ericsson

Title:	KI#2, Conclusions Regarding Edge Relocation

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes conclusions for KI#2

1	Introduction

According to the email discussion, contributions S2-2007133 is picked as the baseline to conclude on how to trigger UE to rediscovery EAS address. This includes both SSC mode 2/3 and session breakout case. Related solutions includes: solution #23, #24, #32, #34, and #51. Contribution S2-2006925, S2-2007242, S2-2007251, S2-2007343, S2-2007460, S2-2007703.

2	Proposal

It is proposed to add the following changes in TR 23.748.

**************************** Start Change ***************************

Evaluation of solutions for Key Issue #2

7.x.y	Evaluation for Key Issue #2: UE DNS cache renewal and EAS reselection by UE

For Session breakout:

Issue 1: DNS cache renewal.

For the EASs to which the UE has no ongoing connection, the UE may has stored DNS cache for these EASs. For this case, after ULCL insertion/change/removal, the cached records may be outdated.

Solution #24 uses ICMP unreachable message to trigger UE reselection of EAS. It assumes that either the TTL of the stored DNS record is very low or only one EAS IP address is stored for the FQDN. This gives restriction to operator deployment. Besides, a short TTL will cause the UE to re-discover EAS IP address frequently and consumes bandwidth and causes application layer delay. In addition, there are concerns on usage of ICMP message in the solution for security reasons. And it’ll add delay for the connection establishment with the EAS as well. Hence, Solution #24 is not recommended for Issue 1.

Solution #32 and Solution #34 (clause 6.34.2.1) support Issue 1 too. In these 2 solutions, a DNS context control information, i.e. DNS cache flush indication and its associated information (for example, domain names and IP address range), within NAS message is sent to UE to enable the UE to remove/renew the cached records.

Issue 2: EAS relocation

For the EASs to which the UE has ongoing connection, the UE may need be triggered to reselect a new EAS address due to a new EAS is more suitable to serve the UE.

Solution #53, clause 6.35.2.1 suggests that UE reselects a new EAS after ULCL insertion.. If connectivity is available towards the source PSA-UPF when a target PSA-UPF is selected, and towards the source AS when the target AS has been selected, then there is no immediate need to change the AS. For these situations the timeout for the DNS cache (if low enough, which is reasonable to have for an application layer DNS cache and for TTL in the DNS entries). Solution #24, #31, #32 and #34 can be used as triggers to trigger UE to reselect EAS.

Solution #24 is not very friendly to support EAS relocation, it is a hard switch from old EAS to new EAS, the application may experience packet loss and delay due to the packet to old EAS is routed to SMF. Hence, it is not recommended to use ICMP to trigger the EAS reselection.

Instead, Solution #31/#32/34 can trigger the UE to reselect EAS too, and they are more friendly. The UE can still connect to the old EAS while the new EAS is selected for new connections. If the N9 tunnel is established between old ULCL and new ULCL, the application layer can have a smooth switch from old EAS to new EAS, packet loss can be avoided.

How the context is migrated between the old EAS and new EAS, and when the UE starts to send packets to the new EAS are left for conclusion in other contributions.

For SSC mode 2/3:

For SSC mode 2/3 case, the UE will receive a new IP address. The new IP address can be used as a trigger to remove the DNS cache, and to reselect a new EAS.

How the context is migrated between the old EAS and new EAS, and when the UE starts to send packets to the new EAS are left for conclusion in other contributions.

Impact of Application Layer DNS caching:

Some application may have DNS caching at the application layer. DNS caching at the application layer results in name resolution using previously cached entries that will result in no new DNS network lookups until the application cache timer expires.

Since cache timeout values on the caching UE application side can be significant, the following behavior may happen:

-	Applications may determine to flush its local caching before the cache timer expires.

-	Applications using connection oriented transport establish connection towards new EAS when:

(a)	an application layer redirect is sent with the URL/FQDN of the new EAS, or

(b)	an application uses anycast destination addresses that are routed to the new ‘closest’ EAS. No new procedures are required for KI#2 in this case, or.

(c) establish new connection to new EAS after DNS caching is flushed by the applications or cache timer expires

-	Applications not using connection oriented transport (e.g., multicast or subscribe/ notify communication patterns) should be responsible for detecting connection change (e.g., notification from UE connection manager). The application should take appropriate actions to indicate new IP address (e.g., in IGMP/MLD membership report message; new subscribe message).

/************************** 2nd Change *************************/

9.x  Conclusions for Key Issue #2

9. x.y	Conclusions  on UE DNS cache renew

For UE DNS cache renew, it is concluded:

- For session breakout case, the SMF may provide DNS cache renew indication  within NAS message to UE as described in Solution #32 and #34 (only DNS cache renew part). Based on the received DNA cache renew indication, the UE cleans the related DNS cache, which do not impact the application client.

Editor’s Note: It is FFS when SMF sends the DNS cache renew indication to the UE, i.e. before the EAS relocation or after EAS relocation.

Editor’s Note: It is FFS whether the SMF send the impacted application information to the UE so the UE can renew DNS cache only for those applications, or the UE renew all DNS cache.

NOTE 1:  	If connectivity is available to the source EAS, then the application can select a new EAS after the  DNS caching is flushed by the applications or cache timer expires

NOTE 2:	In case the UE does not support DNS renew triggered by 5GC, the application will continue to be served by the old EAS until the UE Application Layer DNS cache timer expires or the applications determines to flush the DNS cache.

NOTE 3:	This DNS cache renew indication does not impact the UE Application Layer DNS caching.

- For SSC mode 2/3 case, the UE removes the cached DNS records and reselects a new EAS after it is allocated with a new IP address.

********************************* End Change ****************************