SA WG2 Meeting #141-e 	S2-2007318r04

October 12 – October 23, 2020, Online	was S2-2007318 & S2-2007135

Source:	Spirent Communications, Ericsson

Title:		KI#19_Solution Evaluation for Dispersion analytic output provided by NWDAF

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Solution Evaluation for 23.700-91 of eNA Key Issue #9

# 1 Proposal

The aim of key issue 9 is to address use case #3: NWDAF support for dispersion analytics

-	The kind(s) of information made available to the NWDAF for performing dispersion analytics

-	The kind(s) of information provided by the NWDAF dispersion analytics output to the requesting consumers

-	The procedures enabling NFs to make use of dispersion analytics output and optimized network operation

It proposes to document the following text into TR 23.700-91.

* * * * Start of Changes * * * *

# 7	Overall evaluation

## 7.X	Evaluation of solutions for Key Issue #9 – Dispersion Analytics output provided by NWDAF

For Key Issue #9 – “Dispersion Analytics output provided by NWDAF”, two complementary solutions have been proposed, candidate solutions #30 and #66. Solution #30 describes the required input, delivered NWDAF output and provides procedures that can use the Dispersion Analytics (DA)

Solution #66 describes how the AF can store, in the UDM, the dispersion information as a AMF-Associated Expected Behaviour and SMF-Associated Expected Behaviour and be used by AMF and SMF.

Editor´s Note: Which aspects of solution#30, i.e. which input data is required, which procedure consumes dispersion analytics and which dispersion analytics apply is FFS.

Editor´s Note: Which aspects of solution#30 and #66, i.e. ,, which procedure consumes dispersion analytics and which dispersion analytics apply is FFS.

* * * * End of Changes * * * *