3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007338r01

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Qualcomm, Ericsson, Oppo?

Title:	KI#7: conclusions.

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract: This contribution proposes conclusions for key issues #7.

# 1. Introduction/Discussion

KI#7 addresses How UAV(s) and a UAV Controller establish connectivity in the 3GPP system with the UTM for UAV operation.

KI#7 is automatically solved for the UAV by the architectural assumptions already agreed in the TR.

For UAVCs, based on the incoming LS from ACJA and as described in other papers, no particular mechanisms are required for networked UAVC.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.754.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

The following principles are applied, as applicable, when developing UAV support in 3GPP:

No commercial relationship is assumed between the 3GPP Network and a USS/UTM, in order to enable an UAS operator to change the serving USS/UTM while remaining with same 3GPP Network subscription, and vice versa.  The 3GPP Network subscription for the UAV cannot be assumed to contain any information about the USS/UTM based on a commercial relationship between the 3GPP Network and the USS/UTM.

Key Issue #1:

An UAV is identified by USS/UTM using a CAA-level UAV ID assigned by USS/UTM, and identified by the 3GPP System using a 3GPP UAV ID assigned by the MNO.

The CAA-level UAV ID is used for Remote ID functionality (network or broadcast remote ID). Remote Identification support by 3GPP in the scope of this release applies to the UAV, not to UAV Controller. No assumptions are made limiting the type of information on UAV Controller provided via Remote Identification to satisfy regulatory requirements.

Various formats of CAA-level UAV ID must be supported by the UAV to support various geo-specific regulations. At least Serial Number Identification, a CAA-Issued Registration Identifier (aka Session ID), and USS/UTM-Issued UUID shall be supported

It is assumed that the mechanisms for resolution of CAA Level UAV ID to the USS serving the corresponding UAV, defined outside 3GPP, and available to entities outside the 3GPP system (e.g. the TPAE), are used in the 3GPP system to discover the USS for the UAV.  It may be also possible to use other UAV information (e.g. UAV-provided USS address or FQDN) sent by the UAV to 3GPP system, to be used by the 3GPP System, to discover the USS for the UAV.3GPP system is provided the CAA-level UAV ID by the UAV, and it may provide the CAA-level UAV ID to the UTM/USS when providing MNO services towards the UTM/USS.

The USS/UTM is made aware of the 3GPP UAV ID of the UAV during procedures of UAV authorization supported by the 3GPP network. .

The USS/UTM uses the 3GPP UAV ID to invoke MNO services (e.g. exposure function or location services) or during authorization or authorization revocation. The 3GPP UAV ID is in the format of a GPSI, and at least the External Identifier is supported.

The External Identifier is allocated by the 3GPP network without interaction with the USS/UTM, and must be unique within the geography (e.g. at least country) of the 3GPP network.

Key Issue #2:

An UAV may be authenticated and authorized by USS/UTM with the support of the 3GPP system before connectivity for UAS services (e.g. UAS-USS connectivity for NRID) is enabled. Existing authentication and authorization framework is leveraged as much as possible to minimize the impact on 5GS and EPS system protocols

A UAV includes a CAA Level UAV identity to the 3GPP system. The 3GPP system determines whether to initiate UAV authentication/authorization based on request from UAV, subscription, local policies, and results of previous authentication/authorization. The USS/UTM can revoke such UAV authorization.

NOTE:	The details of how the CAA Level UAV ID is provided (e.g. a specific parameter or a transparent container) will be defined during normative work.

UAV authentication and authorization by USS/UTM is conditional on the UE having performed successfully a primary 3GPP authentication and with Aerial UE function as part of the subscription.

An UAV is authenticated and authorized by USS/UTM using a CAA-level UAV ID. The credentials and related authentication method used by the UAV and UTM/USS are outside of the 3GPP scope.

The 3GPP network shall be informed of the UAV authentication and authorization result and enforce the result accordingly. Upon successful UAV authentication and authorization by USS/UTM, UAV is authorized to establish limited connectivity to communicate with USS/UTM.

A UAV request for user plane connectivity to the 3GPP system for UAV operations (i.e. C2 between a UAV and a networked UAV controller and/or flight authorization request) may also require additional authorization by the UTM/USS.

Key Issue #7:

Based on the architectural assumptions, a UAV is authorized for connectivity to USS over UAV9 based on existing MNO policies, and is allowed to establish connectivity with an appropriate DNN/APN to exchange traffic with the USS without explicit USS authorization for the establishment of the user place connectivity. Once the UAV has succeeded the authentication/authorization of key issue 2, connectivity between the UAV and the USS will be established.

Only after a UAV has registered to the 3GPP system with a UE's MNO subscription can the UAV initiate procedures to request for user plane connectivity for C2 communications between UAV and UAVC.

The request for connectivity between UAV and UAVC shall be initiated by the UAV towards the USS/UTM over PDU Session(s) in the 3GPP system and interacting between the 3GPP CN and the USS/UTM through a UAVF or using NEF/SCEF functions. Such request for user plane connectivity will require UAV getting authenticated and authorized by the USS/UTM utilizing either NSSAA or secondary authentication methods.

Once the USS/UTM has authenticated the UAV and authorized the request for user plane connectivity, this authorization is informed back to the 3GPP system via the UAVF or NEF/SCEF functions and only then will the 3GPP system allocate the required resources to the UAV and UAVC.

USS/UTM provided traffic routing policies will be used by the 3GPP NFs for data traffic over the allocated user plane resources.

Other Aspects:

Single PDU session/PDN connection for USS and C2 connectivity, and separate PDU sessions/PDN connections for USS and C2 connectivity are supported. The mechanism that may be used is up to deployment.

The USS/UTM is not assumed to have knowledge of PDU sessions or PDN connections: the USS/UTM authorizes connectivity requests sent from the 3GPP system for a UAV or UAV controller, can revoke such authorization, and can provide information to control such connectivity (e.g. ACL, QoS information, etc.).

The functionality to support authorization of UAV and UAV controller pairing applies to networked UAV Controllers and non-networked UAV controllers that are connected to UAV via internet.

For geofencing, enable both the "direct query from USS" model, the "direct USS subscription" model, and the "area of interest subscription" model.

For geo-caging, both the option of the 3GPP system providing the UAV location to the USS during procedures, and the option where the USS retrieves it on demand, are supported.

Enable a USS UAV discovery mechanism where the USS/UTM query MNO for UAVs served by the MNO in a specific location.

Editor's note:	For NR to be used for UAVs, "aerial features" as defined in TS 36.300 [9] for E-UTRA, must be enabled and RAN work is needed.