3GPP TSG-WG SA2 Meeting #141E e-meeting	S2-2007401

Elbonia, October 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	NTT DOCOMO, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Verizon, AT&T, T-Mobile USA, Spirent

Title:	Key Issue #11 Interim conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposed an interim conclusion for KI#11.

# Discussion

Based on the KI evaluation in S2-2007519, it is agree on a list of principles for normative work.

# Proposal

It is proposed to include the following as evaluation in the eNA TR.

********************************  START OF CHANGES **************************************

# 8	Conclusions

## 8.1 	KI#11 Interim conclusion

For KI#11 for Data collection aspects, it is proposed

list of principles are agreed for normative work as follows:

    - Data Collection Coordination is separated from Data Collection

    - Data Collection Coordination Function (DCCF) and Data Repository Function (DRF) are to be standardized

    - Data Collection is supported via a Messaging Framework. Adaptors supporting 3GPP services allow NFs to interact with the Messaging Framework

NOTE 1: The Messaging Framework is outside the scope of 3GPP. Adaptors are not expected to be standardized by 3GPP, only the interface between 3GPP entities and the adaptors is under 3GPP scope. This includes 3GPP services offered by adaptors to allow NFs to interact with the Messaging Framework.

    - The NWDAF analytics function interacts directly with the DCCF to request the collection of data. The NFs interact directly with the DCCF to request analytics to an NWDAF.

NOTE 2: Based on the outcome of KI#1, decomposed NWDAF functions can also use the above new functions.

    - Data is collected in a standardized manner from one or more NF(s) (including NWDAF) and OAM system.

    - The collected data and/or produced data can be stored in the DRF, which exposes the standardized interface for storage. In this case, DRF act as data source for the stored data.

    - Consumers NF(s) (e.g. NWDAF) access the data from DRF either directly or via DCCF.

    - Based on local policy, the DCCF can provide data to NWDAF using the data stored in DRF instead of forwarding the request to NF.

    - It is possible to use the NRF for discovery of new DCCF and DRF.

NOTE 3:	Additional authorization for Consumers to access data from a Data Source via the DCCF and to access data from DRF (directly or via DCCF) needs to be coordinated with SA3.

******************************** END OF CHANGES **************************************