3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007410r01

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1,2,3,7, Sol#5 Update: Simplification of PDU Session establishment between USS/UTM and C2 PDU Sessions

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS_SA2 / Rel-17

Abstract: This contribution proposes some simplification of the procedures for Solution #5.

# 1. Introduction/Discussion

Solution #5, Part 2 proposes the same procedure to be used for establishing a PDU Session to the USS/UTM, followed again by the same procedure to establish a PDU Session for C2 flight control. The majority of the details for the procedure is described in steps 11-14.

This update to Solution #5 proposes to describe steps 11-14 initially as PDU Session establishment for USS/UTM communication and for C2 communication. If a separate PDU Session is required for C2 communication then a second PDU Session can be established.  For user plane / QoS flow configurations, the contribution proposes reusing the existing service exposure mechanisms for controlling the C2 communication on the PDU Session(s).

This allows the USS/UTM to reuse the existing 3GPP mechanisms to control the communication between the UAV and the UAVc, based on the information received from the 5GS during PDU Session establishment or subsequently on an already established PDU Session.

In addition, the procedure for Revocation of C2 Connectivity in case of pairing revocation has been modified based on Sol#18. Moreover, procedure for QoS Revocation of flight-related communication has been added.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.754.

# * * * * First change * * * *

##### 6.5.3.1.4	Procedure For Update/Revocation of flight-related communication

This procedure is used to stop or update flight related communication between the UAV and UAVC, e.g. when a flight path is revoked or completed.

Figure 6.5.3.1.4-1: Procedure for Update/Revocation of flight-related communication in 5GS

1.	USS/UTM determines to update or revoke QoS requirements for flight related communication.

2.	USS/UTM initiates the AF session with required QoS update procedure as described in clause 4.15.6.6a of TS 23.502 or AF session with required QoS revoke procedure by sending the Nnef_AFsessionWithQoS_Update message or the Nnef_AFsessionWithQoS_Revoke message, respectively.

3.-8.	When using AF session with required QoS update procedure, steps 2-8 are the same as steps 2-7 described in clause 4.15.6.6a of TS 23.502.

When using AF session with required QoS revoke procedure, steps 2-8 are the same as steps 2-7 described in clause 4.15.6.6a of TS 23.502 with the following difference:

- Instead of updating the request QoS, the UFES/NEF revokes the request QoS by using Npcf_PolicyAuthorization_Delete service operation.

This results in signalling to RAN and the UE to update or remove the QoS Flows used for flight related communication.

# * * * * The End * * * *