3GPP TSG-WG SA2 Meeting #141E e-meeting  	S2-2007415r02

# Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0133 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of service continuity for deferred location request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Deferred location request is supported in rel-16 for commercial location service. In particular, for Periodic Location, and LMF determines to use uplink positioning method, multiple NR-RAN/TRPs are configued by LMF for UE measurement reporting. The configuration of NG-RAN/TRP is determined by LMF, based on the camping cell of UE. When UE moves from one cell to another, it is likely that LMF may re-configue the NG-RAN/TRP. Therefore it is necessary for LMF to beawared of the latest camping cell of UE. It is specified that when AMF receives the LDR from GMLC, it will forwards the LDR to LMF, together with the cell ID of the serving cell of UE. But there is no update of the cell ID to LMF when UE changes its serving cell. It needs to clarify how LMF gets notified the latest serving cell of UE when it changes. On the other hand, RAN3 has defined RAN reporting SRS configuration update when the UE SRS configuration changes, in TS 38.455. However, RAN will not report the latest UE cell in the udpate message so this procedure is only applicable for the scenario UE does not changes its serving cell. Consequently, LMF only re-configue the TRP with the updated UE SRS configuartion. There are two scenario for SA2 to consider in order to suppport service continuity when UE changes its serving cell: - UE changes its serving cell but not trigger handover; - UE handover; |  |  |  |  |  |  |  |  |
| Summary of change: |  | In general, AMF needs maintain the serving LMF ID in case of the Deferred location request. Scenario 1 requires AMF to subscribe to NG-RAN for the UE location reporting (cell level), as defined in clause 4.8.3, TS 23.502 “N2 Notification procedure”. When AMF receives the RAN reporting of latest cell ID of the UE, AMF notifies to LMF. Scenario 2 will trigger handover procedure, and the completion of the handover procedure allows the AMF to get the cell ID of the UE. AMF will further notift the LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of service continuity for deferred location request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1a.5, 6.3.1A(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.1a.5	Deferred Location Request

With a deferred location request, an LCS client or AF sends a location request to a PLMN for a target UE (or group of target UEs) and expects to receive a response containing the indication of event occurrence and location information if requested for the target UE (or group of target UEs) at some future time (or times), which may be associated with specific events associated with the target UE (or group of target UEs). In this version of the specification, only deferred location requests for an MT-LR are supported. Deferred location request service continuity is supported when UE changes its serving cell within a NG-RAN or handover to a target NG-RAN.

# * * * * Second change * * * *

6.3.1A	Support of service continuity when UE changes serving cell

Figure 6.3.1A-1: Support of service continuity when UE changes serving cell

- Same as step 1-13 in clause 6.3.1.

- LMF sends Namf_EventExposure subscribe (LCS correlation ID, event type=UE location (cell ID)) to AMF.

- AMF sends location reporting control message to NG-RAN, to request for report of change of UE cell ID, as defined in TS 38.413[X].

- NG-RAN detects the UE changes its serving cell.

- NG-RAN reports the latest cell ID of the serving cell to AMF.

- AMF sends Namf_EventExposure Notify message (cell ID, LCS correlation ID) to LMF.

- If UE handovers to a target NG-RAN.

- AMF obtains the cell ID of the serving cell in the target NG-RAN.

- AMF sends Namf_EventExposure Notify (cell ID, LCS correlation ID) to LMF.

- Same as step 15 in clause 6.3.1.

# * * * * End of changes * * * *