3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007458r1

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, Hisilicon

Title:	KI#1: LDNSR placement in solution 22

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes that LDNSR is located in UPF.

# Discussion

There are 2 possible places for the LDNSR in solution 22:

- Option 1: Standalone NF as AF

- Option 2: Collocated with UPF

This contribution analyzes the 2 potential placements for the LDNSR.

For Option 1, the LDNSR is an AF. According to the latest solution update, it is assumed that the LDNSR subscribes to “PDU Session Status” targeted to a group of UE(s), or any UE accessing a DNN and S-NSSAI combination. When the related PDU Session is established, the PCC rule provided to SMF includes the subscription of AF. Per subscription information the SMF sends a PDU Session Status notification to the LDNSR, which includes the UE IP address and SMF ID.

It seems Option 1 assumes there is only one LDNSR serving the whole PLMN for an S-NSSAI+DNN combination considering it subscribes for any UE. Also the LDNSR is located before NAT function.

However, for operators which deploy SMFs/UPFs for different regions, for the above assumption, it means that the NAT function must be deployed at only one place per S-NSSAI+DNN, and traffic from all the regions must go to the single place before it goes to the DN. This causes a lot of detoured traffic and makes the multiple region deployment meaningless. Hence, the single LDNSR per S-NSSAI+DNN assumption is not feasible. Currently, it is not clear how the Option 1 can work with multiple LDNSRs per S-NSSAI+DNN.

Furthermore, for option 1, if the LDNSR is not restricted to one, it is unclear how to guarantee that the LDNSR provided to UE as DNS Server has subscribed to the PDU Session Status. For a PDU Session, the SMF needs to choose a LDNSR to be the DNS Server for the PDU Session, the SMF will provide it to UE via PCO or via DHCP procedure. However, when SMF chooses LDNSR as DNS Server, it does not know whether the selected LDNSR has subscribed to this notification from this SMF. The subscription from LDNSR and the SMF choosing LDNSR are two independent procedures. If the selected LDNSR has not subscribed to the PDU Session Status of the UE’s PDU Session, the procedure will not work. It seems that such a loose coordination cannot work for a more complicated deployment scenario.

Observation 1: LDNSR as an AF is not a feasible solution based on the current description of solution 22.

For Option 2, the LDNSR is collocated with UPF, the N4 interface can be enhanced. The enhancements include: the SMF provides the ECS options/L-DNS address to the UPF, in addition, the PDR may be enhanced to order the UPF to check the DNS response to see if the EAS IP in the response satisfy with the condition set by SMF. In our view, the enhancements are less difficult to be implemented in the specification comparing with a brand-new interface.

In addition, in real deployment, the LDNSR is deployed close to the PSA of a PDU Session, considering that its function is simple, it is reasonable to be collocated with PSA UPF. Hence, it is recommended that the LDNSR is collocated with PSA UPF, and the N4 interface is enhanced to support the procedure defined in solution #22.

Conclusion: LDNSR is collocated with PSA UPF, and N4 interface is enhanced to support the procedure defined in solution #22.

# Proposal

It is proposed to have the following changes in TR 23.748-040:

/************************** 1st Changes *************************/

## 6.22	Solution #22: DNS based EAS discovery supporting session breakout

This solution is for Key Issue #1 on DNS based EAS discovery supporting session breakout.

### 6.22.1	Description

#### 6.22.1.1	Deployment assumption of the solution

Centralized DNS (C-DNS) server is centrally deployed by MNO or 3rd party and responsible for resolving the UE DNS queries into a suitable Edge Application Server (EAS) IP address.

Localized DNS (L-DNS) resolvers/servers may be locally deployed within edge hosting environment, and responsible for resolving the UE DNS queries into a suitable EAS IP address within the Local DN. The L-DNS resolvers/servers may or may not have connectivity with C-DNS server depending on the deployment. The L-DNS resolvers/servers may be deployed by MNO or 3rd parties.

NOTE 1: The C-DNS server and/or L-DNS resolvers/servers may use an anycast address.

NOTE 2: The C-DNS server or L-DNS resolvers/servers may contact any other DNS servers for recursive queries, which is out of control of 5GS and not described in the solution apart that the authoritative DNS server may support the DNS ECS option. The C-DNS server or L-DNS resolvers/servers can be different from the authoritative DNS server for the target domain name.

Figure 6.22.1.1-1: Target deployment scenario of the solution

The solution requires a new Functionality, an enhanced DNS Forwarder here referred to as “LDNSR”. LDNSR supports Edge AS Discovery using DNS using knowledge of the 5GC connectivity of the UE. The LDNSR is part of PSA UPF.

/************************** 2nd Changes *************************/

### 6.22.2	Procedures

L-DNS server/resolver

L-DNS server/resolver

UE

ULCL/

BP

Local PSA-2

PSA-1

SMF

C-DNS server

2. DNS query

6. DNS response

LDNSR

3. Interaction with SMF

Option 3

Option 1

4a. DNS query/response

4b. Notify SMF

5a.  ULCL/BP insertion and local PSA selection

PDU Session Establishment with PSA1

Option 2a

4c. DNS query/response

4d. Notify SMF

Traffic Filter

1. DNS Query Triggered

Option 2b

DNS query/response

7. DNS Query

7. DNS Query

PRE-ESTABLISHED ULCL/L-PSA & FILTER

DYNAMIC ULCL/L-PSA & FILTER

5b. Router Advertisement

L-DNS server/

Resolver

DNS Response

Figure 6.22.2-1 DNS resolving in session breakout scenario

The AF may provide EAS deployment information to UDR via Nnef_TrafficInfluence, this includes FQDNs, IP addresses/prefixes, Local DNS resolver/server IP address per DNAI.

The SMF gets the information from PCF (PCC rule) during PDU session establishment. During PDU Session Establishment procedure, the address of C-DNS server or LDNSR is provided by SMF via PCO to UE (LDNSR is provided if Dynamic UL CL/L-PSA insertion applies). Steps 1 - 6 are processed for each DNS request if UL CL/BP is not inserted or if UL CL/BP is not configured to locally route DNS query to local PSA after insertion.

For the “pre-established” ULCL/BP/L-PSA scenario, the SMF performs UL CL/BP/L-PSA selection and insertion based on UE location and sets up uplink filter rule on UL CL/BP. Steps 1, 7 are processed for each DNS request if UL CL/BP is configured to route DNS query to Local PSA.

The SMF may provide to the LDNSR

Option 1: The ECS option (IP address corresponding to DNAI available to UE’s location).

Option 2a: The address of L-DNS server serving the DNAI available to UE’s location.

Option 2b: Indicates LDNSR to forward DNS query with FQDN(s) corresponding to DNAI which has no direct connectivity to LDNSR.

Editor’s Note: it is FFS how the SMF provides this information to the LDNSR deployed as part of a UPF, i.e. which N4 procedures are involved.

Based on the UE location, at PDU session establishment and during UE mobility the SMF may provide the LDNSR with ECS option and L-DNS server address for the relevant FQDNs.

The SMF may also provide the IP range(s) that shall trigger the LDNSR to notify SMF when the IP address (of EAS) in DNS response is within one of these the IP range(s). As an alternative, LDNSR could be instructed to notify SMF when ECS is in the DNS response.

Editor’s Note: it is FFS whether the alternative is needed where LDNSR could be instructed to notify SMF when ECS is in the DNS response.

At PDU session establishment and during UE mobility, the SMF may provide the LDNSR with the FQDNs that, when received in the DNS query, will trigger the LDNSR to fetch the forwarding parameters from the SMF (option 1 and 2a) or forward the DNS query to the SMF (option 2b) in step 3 of the following procedure.

Editor’s Note: it is FFS whether the LDNSR fetches from SMF information needed to build ECS or whether the SMF pushes this information (e.g. when UE has moved to an area where new traffic offload conditions may take place).

1.	The UE sends a DNS query including the requested FQDN.

2. 	If an ULCL/BP exists and the DNS query matches the uplink filter rule on UL CL/BP to route the DNS query to Local PSA, then step 3 to 6 are skipped. Otherwise the PSA1 UPF forwards the received DNS query to the LDNSR.

3.	Based on the configuration received from the SMF for the FQDN in the DNS query, the LDNSR determines the forwarding parameters:

Option 1:  the IP address to add as ECS DNS option. This IP address may correspond to the DNAI associated by the SMF with the UE location and a target domain.

Option 2a (with direct connectivity between LDNSR and L-DNS): the address of L-DNS server towards which to propagate a DNS request. This L-DNS address may correspond to the DNAI associated by the SMF with the UE location and a target domain..

Option 2b (without direct connectivity between LDNSR and L-DNS):  the LDNSR requests the SMF to forward a DNS query (in step 5).

The LDSNR may fetch these forwarding parameters from the SMF by providing the FQDN to the SMF in this step.

Editor’s Note: it is FFS how the LDNSR deployed as part of a UPF fetches the forwarding parameters from the SMF, i.e. which N4 procedures are involved.

4a-4b:	Option 1: The LDNSR adds the IPv4 subnet or address or IPv6 prefix provisioned by SMF in step 3 as ECS option as specified in RFC 7871 and sends the DNS request to C-DNS server. The C-DNS returns the DNS response including EAS IP address.

4c-4d:	Option 2a (with direct connectivity between LDNSR and L-DNS): the LDNSR sends the DNS query to the L-DNS server provisioned by SMF and get the DNS response including the EAS IP address.

For both 4a-4b and 4c-4d, After receiving the DNS response, the LDNSR may notify SMF with IP address of EAS and FQDN if certain criteria set by SMF are matched, e.g., the IP address of EAS in DNS response is within the IP range(s) indicated by SMF, or the FQDN is matched. Then, SMF decides whether to trigger the insertion of the ULCL/Local PSA.

Editor’s Note: it is FFS how the LDNSR deployed as part of a UPF notifies the SMF, i.e. which N4 procedures are involved.

5a.	The SMF decides DNAI and performs UL CL/L-PSA selection and set up. The DNAI and UL CL/Local PSA2 is selected based on the information provided by LDNSR to ensure the selected local PSA and EAS are corresponding to same DNAI.

Option 2b: If the LDNSR requested the SMF to forward the DNS query in step 3, the SMF sends the DNS query to the address of the local DNS server via the Local PSA2. The SMF configures the Local PSA2 to forward the DNS response to the SMF. When receiving the DNS response, the SMF forwards it to the LDNSR. After receiving the DNS response, the LDNSR may notify the SMF with IP address of EAS as in option 1 and 2a. The SMF may perform update of the ULCL/L-PSA based on this notification as in option 1 and 2a..

Optionally, the SMF may configure the L-DNS address into ULCL to be diverted to L-PSA. In order to redirect subsequent DNS queries directly to the L-DNS, the LDNSR acts as an iterative DNS server and redirects the DNS Query to the L-DNS selected for the UE Location by responding with a referral to the UE. This triggers the UE to send a new DNS query towards the L-DNS.

As an optimization, the SMF may configure uplink forwarding rules on UL CL to route the traffic and (if the SMF is configured to determine that the DNS traffic is not encrypted) subsequent DNS queries for FQDNs corresponding to the DNAI to the Local PSA2 (as option 3a in step 7).

5b. In the case of IPv6 multi-homing, the SMF notifies the UE of the availability of the new IP prefix @ PSA2 using an IPv6 Router Advertisement message (RFC 4861 [x]). Optionally, in this RA message, the SMF may notify UE the new address of DNS server (RFC 8106 [y]) and indicate the UE to contact with DHCP server to get the DNS-related configuration information (RFC 4861 [x]), which is used to select DNS server for a target FQDN (RFC 6731[26]).

Also, the SMF sends IPv6 multi-homed routing rule along with the IPv6 prefix to the UE to influence the selection of the source Prefix for the subsequent DNS queries(RFC 4191 [29]) as described in TS 23.501 [2] clause 5.8.2.2.2. The SMF may re-configure the UE for the Source IP prefix @ PSA1.

6.	The LDNSR sends DNS response to UE via PSA1.

In the case of IPv6 multi-homing, if the DNS-related information is configured on UE, the UE decides the DNS server to use for the subsequent DNS queries. Otherwise, the UE uses the DNS server address provided in the PCO during PDU session establishment. Further, the UE selects the source IP prefix based on the IPv6 multi-homed routing rule provided by SMF.

7.	Option 3a and “pre-established” case: Based on the uplink forwarding rules, if an ULCL exists and the query matches the uplink filter rule on UL CL, the UL CL route the DNS query to local PSA2, and then local N6 routing forwards to Local DN. If no filter is present in ULCL, the DNS request is routed to PSA-1. AF influenced routes with anycast service address may be used to provision PCF which is then used during PDU session establishment to setup traffic filters in ULCL. All application traffic including Do53, DoT and DoH requests are steered using this mechanism.

The DNS query may be routed to a local DNS resolver/L-DNS which then resolves the DNS Query or forwards it for resolution. It may derive an ECS option (using locally configured value or the source address/prefix of the received DNS request), then send the DNS query to Authoritative DNS server using the ECS option. Alternatively, the DNS query may be routed to C-DNS via Local N6 or DN. The deployment should ensure the DNS query packet has topologically correct UE source address when DNS resolver gets it, e.g. via NAT in Local DN. See Figure 6.22.1.4-1.

The local DNS resolver or C-DNS server sends DNS response to the UE by retracing the forward path.

Option 3b:

a. The local DNS resolver modifies the packet’s destination IP address (corresponding to a default DNS server) to that of the L-DNS and stores the original IP address (Default-DNS-IP) and the packet’s source IP address (corresponding to UE’s IP address) to its own (i.e. the local DNS resolver’s) IP address and stores the original source IP address (UE-IP) for later processing.

b. The local DNS resolver then forwards the modified DNS request to the L-DNS and processes as follows:

-	If the L-DNS can resolve the IP address for the requested FQDN of EAS, it responds to the local DNS resolver with the desired IP address of the local EAS.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS and the L-DNS is not connected to the C-DNS, it responds to the local DNS resolver with DNS Response without including the IP address of EAS or indicating that it could not resolve the FQDN.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS but it is connected to a C-DNS, it communicates with the C-DNS to recursively resolve the EAS IP address.

c. The local DNS resolver receives a DNS Response including empty value:

-  The local DNS resolver forwards the original DNS request (the one that had originally arrived from the UL CL/BP UPF) back to the UL CL/BP UPF.

-  The UL CL/BP UPF forwards the original DNS Request from local DNS resolver to the remote PSA UPF via N9.

NOTE 7: Option 1 and 2 can be still applicable in Phase 2 if the requested FQDN is not configured to be local routed on the UL CL or the DNS traffic is encrypted.

/*********************** 3rd change **********************/

### 6.22.3	Impacts on services, entities and interfaces

UE:

-	In case of IPv6 multi-homing, the UE receives the DNS-related configuration information for the PDU Session and selects the DNS server to be used for DNS queries of a specific FQDN.

SMF:

-	Configure UE with the address of LDNSR as the DNS server during PDU session establishment via PCO.

-	the SMF provides the LDNSR with:

-	Option 1: the IP address to add as ECS DNS option.

-	Option 2a (with direct connectivity between LDNSR and L-DNS): the address of L-DNS server towards which to propagate a DNS request.

-	Option 2b (without direct connectivity between LDNSR and L-DNS) the requirement (with relevant filters) to forward a DNS request to the SMF

-	Conditions on the DNS response that request LDNSR to notify the SMF

-	Dynamically (upon LDNSR notifications) inserts ULCL and local PSA and optionally configures the traffic routing rule to ULCL/BP for subsequent DNS queries targeting FQDNs supported by the DNAI.

-	In case of IPv6 multi-homing, the SMF indicates the UE to contact the DHCP server to get the DNS-related configuration information for the PDU Session.

UPF:

UPF as an ULCL:

-	Optionally, the ULCL is configured with uplink forwarding rules to route the traffic and DNS queries for FQDNs ranges to the Local PSA WG2 (assuming DNS traffic is not ciphered).

N4 interface:

Editor’s Note: The impact on the N4 interface to support the communication between the SMF and the LDNSR in the UPF is FFS.

LDNSR:

-	performs the role of a DNS Resolver and performs interactions with the SMF.

-	the LDNSR is configured by the SMF with:

-	Option 1: the IP address to add as ECS DNS option.

-	Option 2a (with direct connectivity between LDNSR and L-DNS): the address of L-DNS server towards which to propagate a DNS request.

-	Option 2b (without direct connectivity between LDNSR and L-DNS) the requirement (with relevant filters) to forward a DNS request to the SMF

-	Conditions on the DNS response that request LDNSR to notify the SMF

-	The handling of DNS message in the LDNSR:

-	Option 1: the LDNSR adds ECS option in DNS query message.

-	Option 2a: the LDNSR redirects the DNS query to the L-DNS.

-	Option 2b: the LDNSR forwards the DNS query to the SMF and receives the DNS response from SMF.

-	The LDNSR may be provided with the IP range(s) and indicated by SMF to notify the IP address of EAS (in DNS response) to SMF if the IP address of EAS in DNS response matches the IP range(s). The LDNSR may also notify the IP address and FQDN in the DNS response to SMF if FQDN is matched.

PCF

-	Optionally allow AF to provide PCF with Traffic Influence routing per service address

N6 Interface:

Impacts introduced by Option 3b is as described in clause 6.20.3 with replacing DNS Inspector with DNS Resolver.

Editor's note:	the impacts above (option 3b/ in clause 6.20.3) need to be clarified

/*********************** End of Changes **********************/