SA WG2 Meeting #141e	S2-2007493r02

Electronic Meeting, October 12 – 23, 2020

Source:	Nokia, Nokia Shanghai Bell, Telecom Italia

Title:	KI #1,2, Evaluation and conclusions

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This p-CR proposes a Key Issue #1 and 2 Evaluation and Conclusion Section.

# 1	Discussion

This paper provides an evaluation and conclusion for the Key Issue #1 and 2 solutions.

# 2	Proposal

This contribution proposes to implement the following updates to TR 23.700-40 v1.0.0.

* * * Start of Change 1 * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

## 7.1	Evaluation on solutions of KI#1

## This clause contains evaluations and also justifies way forward for the KI.

Almost all solutions imply some form of interaction of the AMF with another function which performs the counting (unless a single AMF set only is used for a S-NSSAI and roaming is not supported, in which case there is no need for an external function).

Solutions where only the AMF sets are involved and OAM splits the quota over the AMF sets, suffer from quota fragmentation and it is not clear what the criterion for splitting would be. Also roaming case is not well supported.

It is beneficial that the CHF records the number of UEs registered with a S-NSSAI, so it is beneficial that the CHF is in the loop of counting events hence it is recommended that CHF and AMF interact for the purpose of counting the UE registered with a network slice.

Since the NSQ would be a new function, and this seems not to be required considering the above, NSQ based solutions are not preferred.

The counting with NSSF involvement is not suitable if the per UE information needs to be reported to the CHF, so it should not be pursued in association with a requirement of reporting information to CHF (i.e. the CHF not only records the number of UE per slice but also which UEs were in a lice at a given time).

## 7.2	Evaluation on solutions of KI#2

This clause contains evaluations and also justifies way forward for the KI.

Almost all solutions imply some form of interaction of the SMF with another function which performs the counting (unless a single SMF set only is used for a S-NSSAI and roaming is not supported, in which case there is no need for an external function).

Solutions where only the SMF sets are involved and OAM splits the quota over the SMF sets, suffer from quota fragmentation and it is not clear what the criterion for splitting would be.

It is beneficial that the CHF records the number of PDU Sessions established for a S-NSSAI, so it is beneficial that the CHF is in the loop of counting events. However, the support of LBO roaming use case benefits from introducing a non-existing inter-PLMN SMF-CHF interface. The CHF can then interface directly with the SMF (sets) for counting purposes also in this case.

Since the NSQ would be a new function, and this seems not to be required considering the above, NSQ based solutions are not preferred.

When multiple SMF sets are involved for a network slice, it is assumed that the CHF consolidates the counting across them.

For the interworking with EPS: the solution can support this as long as the counting is reported by the combined PGW-C-SMF.

## 7.3	Evaluation on solutions of KI#3

Editor's note:	This clause will provide some interim evaluation based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

High level aspects of the solutions:

-	Solution 22 has RAN impact. It lets RAN to enforce the SMBR (Slice Maximum Bitrate).. Currently, RAN is able to be aware of the S-NSSAI of the PDU Session. And RAN is able to be enforce the UE AMBR per UE and GFBR/MFBR per QoS Flow.

Editor´s note:	Solution#22 needs to be validated with RAN WG2 and RAN WG3, due to RAN impacts.

-	Solution 13 uses UPF to enforce the DL slice level bitrate. This solution will require to select the same SMF/PCF and UPF for all the PDU Sessions within the slice. It is not necessary to introduce such limitation.

-	Distribution based solutions, i.e. Solution 20&21, let a centralized NF distribute the SMBR into pieces (i.e. Session AMBR and/or MFBR). They have no RAN impact. However, solutions do not explain how to resolve the fact that since the SMBR is distributed into Session-AMBRs, the aggregated SMBR enforced may be smaller than the SMBR, as such the SLA would not be fulfilled, as the UE will be throttled while SMBR is not fully consumed. The situation could be worse when a large amount of PDU Sessions exist as the SMBR is distributed over more Session AMBR.

# 8	Conclusions

Editor's note:	This clause captures interim conclusions from the study until the WG agrees these are stable and this editor's note is removed.

For the counting of UEs registered with a Network Slice, the CHF shall be involved as per solution 38 also for the roaming use cases.

For the counting of PDU Sessions, the CHF shall be involved as per solution 38.

When more than one AMF set/SMF is used per slice, these interact with the CHF which keeps the up to date value of the counter across all the SMFs and AMFs

The counting of PDU Sessions shall also enable the counting the number of PDU Sessions that are in the EPS if it is policy of the operator to request this for a S-NSSAI. The CHF can also infer the number of UEs with at least one PDU Session and perform counting of UEs with at last one PDU Session, if so required. This is useful to count UEs when they are in the EPS, as long as they have at least one PDN Connection that maps to a PDU Session for the Network Slice.

* * * Start of Change 2 * * *

## 8.x	Conclusion for Key Issue #7

For Key Issue #7, Solution 29 "Operating Band Information is Provided with the Configured NSSAI" will be used as the basis for normative work.