SA WG2 Meeting #141E	S2-2007746

12 October – 23 October 2020

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm Incorporated

Title:	KI #1: Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the evaluations for the solutions of KI#1 and proposes the conclusion for KI#1.

# 1		Discussion

This contribution proposes conclusions and way forward for Key Issue #1 for Uplink Time Synchronization.

# 2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 7.1	Key Issue #1: Uplink Time Synchronization

Solution #1 is the only solution addressing KI#1 Uplink Time Synchronization. The solution describes four alternative methods for BMCA procedure for gPTP. In method 1 most of the BMCA logic is centralized to the TSN AF (or NEF for non-TSN sessions), but also DS-TT and NW-TT participate to BMCA operation, e.g. the DS-TT must be able to determine whether the received Announce is worse or better than the current Announce.

Method 2 and Method 4 are very similar, both run the BMCA function in NW-TT. The main difference is whether the Announce messages are generated on port basis from each DS-TT (Method 2) or from NW-TT on behalf of the DS-TT (Method 4).

In methods 2 and 4 the DS-TT does not need to be aware of its port state (except when its grandmaster is activated or deactivated). In both methods, the BMCA result may need to be reported from NW-TT to the NEF (or TSN AF) if the port states in the DS-TT(s) need to be changed (e.g. GM needs to be activated or deactivated in DS-TT).

Method 3 assumes that the DS-TT knows its port state via Announce messages in user plane, but it is unclear how the DS-TT port learns that its state has changed between Master, Slave and Passive states. Also in method 3 the BMCA result may need to be reported from NW-TT to the NEF (or TSN AF) if the GM needs to be activated or deactivated in DS-TT.

## 7.1	Key Issue #1: Uplink Time Synchronization

Solution #1 is the only solution addressing KI#1 Uplink Time Synchronization. The solution describes four alternative methods for BMCA procedure for gPTP. In method 1 most of the BMCA logic is centralized to the TSN AF (or NEF for non-TSN sessions), but also DS-TT and NW-TT participate to BMCA operation, e.g. the DS-TT must be able to determine whether the received Announce is worse or better than the current Announce.

Method 2 and Method 4 are very similar, both run the BMCA function in NW-TT. The main difference is whether the Announce messages are generated on port basis from each DS-TT (Method 2) or from NW-TT on behalf of the DS-TT (Method 4). In both methods, the BMCA result may need to be reported from NW-TT to the NEF (or TSN AF) if the port states in the DS-TT(s) need to be changed (e.g. GM needs to be activated or deactivated in DS-TT).

# * * * * Second change * * * *

## 8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following are some interim principles to be considered for the way forward:

-	The following interim agreement are applied for transmission of the gPTP messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM' gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	In the case of synchronizing TSN end stations behind UPF/NW-TT, the NW-TT performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	In the case of synchronizing TSN end stations behind other UE(s), UPF then forwards the gPTP messages to the UEs via all PDU sessions terminating in this UPF except for the PDU session of the source ("avoids play back to the source DS-TT port"). The other UE(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of (g)PtP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

-	The following agreement is applied for BMCA procedure:

-	Solution #1 Method 2 in clause 6.1.3.3 is recommended as the basis for normative work. This method implies:

-	NW-TT needs to process BMCA, determine port roles within the 5GS, and maintains Master-Slave hierarchy.

-	If the NW-TT determines that due to the result of the BMCA the port states needs to be changed in DS-TT(s), NW-TT reports port states to the TSN AF (or AF or NEF) via BMIC, if the TSN AF (or NEF) has subscribed for the BMCA result reports.

-	DS-TT forwards all received Announce messages to NW-TT.

-	The TSN AF (or NEF) configures the grandmaster in DS-TT port (if supported)  via PMIC; otherwise TSN AF configures the grandmaster in NW-TT for PDU sessions where the DS-TT does not support acting as a (g)PTP GM.