SA WG2 Meeting #141E	S2-2007746

12 October – 23 October 2020

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm Incorporated

Title:	KI #1: Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the evaluations for the solutions of KI#1 and proposes the conclusion for KI#1.

1		Discussion

This contribution proposes conclusions and way forward for Key Issue #1 for Uplink Time Synchronization.

2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following are some interim principles to be considered for the way forward:

-	The following interim agreement are applied for transmission of the gPTP messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM' gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	In the case of synchronizing TSN end stations behind UPF/NW-TT, the NW-TT performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	In the case of synchronizing TSN end stations behind other UE(s), UPF then forwards the gPTP messages to the UEs via all PDU sessions terminating in this UPF except for the PDU session of the source ("avoids play back to the source DS-TT port"). The other UE(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of (g)PtP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

-	The following agreement is applied for BMCA procedure:

-	NW-TT needs to process BMCA, determine port roles within the 5GS, maintains Master-Slave hierarchy and regenerate the Announce message for the Master ports on the NW-TT and the DS-TT.

-	The NW-TT/UPF sends the regenerated Announce message to the PDU sessions related to the Master Port on the DS-TT.

-	If the TSN AF (or NEF) has subscribed for the BMCA result reports, the NW-TT reports BMCA result to the TSN AF (or AF or NEF) via BMIC.

-	DS-TT forwards the received Announce messages to NW-TT.

Editor's note:	Whether and how the DS-TT filters the receive Annouce/Syn messages are FFS.

-	The TSN AF (or NEF) configures the grandmaster in DS-TT port (if supported)  via PMIC; otherwise TSN AF configures the grandmaster in NW-TT for PDU sessions where the DS-TT does not support acting as a (g)PTP GM.