SA WG2 Meeting #141E	S2-2007762

12 October – 23 October 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #3B: Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the evaluations for the solutions of KI#3B and proposes the conclusion for KI#3B.

# 1		Discussion

This contribution proposes conclusions and way forward for Key Issue #3B for Exposure of Time Synchronization.

# 2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 7.2	Key Issue #3B: Exposure of Time Synchronization

Solution #7 supports the AF to be able to request Time Synchronization service via External Parameter Provisioning. The exposure includes service activation/deactivation/modification, it can interwork with IEEE TSN or TSC use cases, and it can target a UE or a group of UEs. The exposure proposed for time synchronization supports the 4 time synchronization methods listed in KI#3B.

Solution #8 supports AF requesting TSN Synchronization activation and deactivation providing new configuration to the UDM via NEF and triggering a PDU Session establishment. The same SMF and UPF control all PDU Sessions involved in the time synchronization service. The time synchronization service is coupled to a TSC PDU Session.

Solution #9 supports the DS-TT acting as a (g)PTP grand master (GM) for devices attached to the DS-TT. The DS-TT indicates to the network inside a PMIC whether DS-TT is capable of acting as (g)PTP GM and which version(s) it supports. The indicated capabilities can be used to support exposure for time synchronization. Additionally, the time information used in (g)PTP at the DS-TT is based on the time information received from the gNB using RRC/SIB signalling.

# * * * * Second change * * * *

## 8.3	Key Issue #3B: Exposure of Time Synchronization

To enable Exposure of Time Synchronization, it is recommended to select solutions 7 and 9 as the basis for normative work. Solution 7 provide the description to extend the exposure framework to support AF requesting time synchronization. Solution 7 considers the exchange of information required between the 5GS and the AF to support the 4 time synchronization methods of KI#3B and the functionality required at the DS/NW-TTs for every method. Solution 9 complements the exposure of time synchronization addressing the subscription and authorization of the AF's request. Additionally, it enables the time synchronization configuration before the PDU Session is established. Solution 9 adds the support of DS-TT capabilities indication towards the 5GS.

Following are principles to be considered as the basis for normative work:

-	Time Synchronization service can be provided for Ethernet PDU sessions and IP PDU sessions.

-	Four time synchronization methods are supported as described in Solution #7.

-	Time synchronization service may support GM residing in DS-TT(s) as described in Solution #9.

Editor's note: Whether 	Time Synchronization service policy and charging control may be applied on a PDU Session or independent of any PDU Session is FFS..