SA WG2 Meeting SA2-141e	S2-2007778

12 – 23 October, 2020, E-meeting	(revision of S2-2007620, S2-2007087, S2-2007576)

Source:	Intel, Charter, Comcast, Sony

Title:	KI #1, Proposed way forward

Document for:	Discussion / Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes to interim conclusions on KI#1.

# 1	Proposal

Based on the discussion in S2-2007085 (refer to Proposals 2-6 and Slide #13) it is proposed to agree an interim conclusion on KI#1 as indicated below. Revision S2-2005685r19 (that could not be agreed during SA2#140E) is used as the basis. Yellow-highlighted text is new.

Relevant discussion part from S2-2007620. For KI#1, the paging cause/not to response to paging/response with BUSY are all depended on RAN and SA3 feedback. However, UE can be in an environment where network-A does not support MUSIM while network-B does. In this case, we suggest the following behaviour: - For UE operating in network-A that does not support MUSIM feature and the UE is being paged by network-A while engaging activity with network-B (which supports MUSIM feature), UE will try to respond to paging by network-A. If UE decides to ignore paging, then it is due to UE internal hardship or implementation and not according to 3GPP. - This above suggestion is to ensure the existing network deployment is not impacted by new feature developed in 3GPP. Furthermore, it is also suggested that even when network-A supports MUSIM operation, UE should not ignore paging from network-A. Hence, we propose BUSY indication (via NAS) to be agreed by SA2. If RAN came back with positive AS based option then SA2 can add this option as deployment choice. In any case, SA2 can decide to have NAS based solution to go forward as basis as this solution would allow common mechanism to be applied for both EPS/5GS system as much as possible.

* * * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

The following interim conclusions are agreed for the baseline functionality:

-	A Paging Cause for both EPS and 5GS, pending feedback from SA3 and RAN WGs.

Editor’s note: Whether Paging Cause will be pursued to final conclusions will be confirmed in SA2#142E taking into account feedback from RAN2, RAN3 and SA3.

Editor’s note: It is FFS whether One or Multiple paging cause values are needed.  Whether a distinct value is needed to allow the UE to discriminate between gNBs supporting the Paging Cause from those that do not will be determined.

Editor’s note: It will be determined whether the Paging Cause is applied 1) only for UEs with the request, i.e. the UEs that have indicated MUSIM capability and have multiple USIMs or 2) to all UEs indiscriminately.

-	A Negotiated Short Absence for 5GS, evaluated together with RRC-based coordinated leaving, pending feedback from RAN WGs.

NOTE:	RAN WGs will determine the details of the procedure including RRC states.

-	A Busy Indication that is used by the Multi-USIM device when it determines to not accept the paging.

Editor's note:	It is FFS whether the Multi-USIM device uses the Busy indication with a “shall”, “should”, “may” or “best effort” condition.

Editor's note:	Whether AS or NAS BUSY indication is used in the interim conclusion on Busy indication is pending RAN and SA3 WGs’ feedback.

–

* * * * End of Change * * * *