3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009023

Elbonia, November 16 – 20, 2020

Source:	Orange

Title:	KI #17, Sol #52: Update to complete the solution

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes an update on solution #52 (related to KI#17).

# Proposal

It is proposed to apply the following changes to the TR 23.700-91.

* * * * Start of changes * * * *

## 6.52	Solution #52: Accuracy levels and options

### 6.52.1	Description

#### 6.52.1.1	General

This is a solution for the KI#17 "Definition of accuracy levels".

Accuracy depends on the subsets of analytics produced (continuous versus discrete values, vectors) per analytics ID. Accuracy constraints, in the scope of the NWDAF, are requirements on the quality of expected outputs. Accuracy levels with finer grain can guide the choice of relevant data collection and calculation models, and the amount of resources (signalling, calculation) to be mobilized.

The control of accuracy options may be modified under the following principles:

-	Add 2 additional values to every "accuracy level" parameter, i.e. allow up to 4 levels;

-	Define a new "subset accuracy level" optional parameter, applicable per subset of analytics;

-	Keep the existing "preferred level of accuracy" global parameter (see TS 23.288 clause 6.1.3), which would act as a default value for the subsets that don't have any "subset accuracy level" parameter.

* * * * End of changes * * * *