3GPP TSG-WG SA2 Meeting #142E	S2-2009351

Nov 16 – Nov 20, 2020, Elbonia	(revision of S2-2008646r02)

Source:	Huawei, HiSilicon

Title:	KI #1, Sol #5: Update to clarify the parameters for ML model discovery

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes to update to clarify the parameters for ML model discovery.

1. Introduction

Based on the discussion in S2-2008645, this contribution proposes to update the Solution #5 for KI#1.

2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

6.5	Solution #5: ML Model sharing architecture

6.5.1	Description

This solution is proposed for Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions and Key Issue #19: Trained data model sharing between multiple NWDAF instances.

AI Function (AIF) trains the ML Model(s) and provides ML Model(s) to 5G NFs (e.g. NWDAF). The following figure 6.5.1-1 shows the network data analytics architecture supporting AIF.

Figure 6.5.1-1: Network data analytics architecture supporting AIF

The AIF is decomposed from a NWDAF, it can be a standalone NF or a NWDAF functionality.

NOTE 1:	The AIF within 3rd party AF is not considered in this release.

The AIF supporting the following functionalities:

-	ML Model Training;

-	ML Model Repository;

-	ML Model Exposure.

In order to support AIF consumer to discover an AIF instance that is able to provide some specific type of ML Model, each AIF instance should provide the list of ML Model ID(s) that it support when registering to the NRF, in addition to other NRF registration elements of the NF profile. Other NFs requiring the discovery of an AIF instance that provides support for some specific type of ML Models may query the NRF and include the ML Model ID(s) that identifies the desired type of ML Model. The ML Model ID identifies an ML Model and correlates with Analytics ID defined in TS 23.288 [5].

NOTE 2:	Discovery, via NRF, of ML models supported by AIF and related parameters are aligned with  Key Issue #19: Trained data model sharing between multiple NWDAF instances.

In order to support a specified ML Model training, the AIF may need to collect data from data source (e.g. 3GPP NF(s), data centre).

The AIF service consumer provides the following input parameters listed below.

-	A list of ML Model ID(s): identifies requested ML Model(s);

-	Filter information: indicates the conditions to be fulfilled (e.g. accuracy, performance).

The AIF provides to the consumer the output information listed below.

-	A list of ML Model ID(s);

-	ML Model.

6.5.2	Procedures

6.5.2.1	ML Model exposure procedure

Figure 6.5.2.1-1: procedure of ML Model exposure

1.	AIF service consumer (e.g. AF, 5GC NF) requests the AIF for an ML Model indicated by ML Model ID(s). The AIF may need to train the ML Model to meet the requirement. The parameters that can be provided by the AIF service consumer are listed in clause 6.5.1.

2.	When a request for an ML Model received, the AIF either responds with the requested ML model if it has, or responds with time when the ML model will be available if ML model is not ready.

3.	If the AIF needs to train the designated ML model, it collect data from data source (e.g. AF, 5GC NFs) for ML Model training.

NOTE:	what the data source is shall comply with the conclusion for KI#11.

4.	The AIF performs ML model training.

5.	The AIF service consumer re-sends the request if needed according to the time indicated by AIF in step 2.

6.	The AIF provides a response including the requested ML model to the consumer.

6.5.2.2	NWDAF analytics service supporting AIF

Figure 6.5.2.2-1: NWDAF analytics service supporting AIF

1.	Consumer NF sends an Analytics request/subscribe including Analytics ID to NWDAF.

2.	In order to provide the requested analytics, the NWDAF may need to determine which ML model is used for the requested analytics. If the NWDAF do not have a proper ML Model for the request analytics, the NWDAF may discover from the NRF the ML Model ID and AIF instance address with requested Analytics ID and send a request to the AIF for the ML Model indicated by the ML Model ID.

3.	The NWDAF may need to collect data from 3GPP NFs for the requested analytics.

4.	The NWDAF derives requested analytics.

5.	If the analytics result does not meet the requirement (e.g. accuracy), the NWDAF may need to request the AIF to train the ML Model indicated by ML Model ID, the NWDAF may also indicate that the analytics result does not meet the requirement, so that the AIF may collect data from the NWDAF to adjust the ML model, where the data collected from the NWDAF was used to determine that the analytics result doesn't meet the requirements.

6.	The NWDAF derives new requested analytics using the updated ML model provided by the AIF.

7.	The NWDAF responses to the consumer with the analytics result.

6.5.3	Impacts on services, entities and interfaces

A new network function AIF is introduced. The AIF provides ML Model training and ML Model exposure service.

No impact on current services, entities and interfaces, except:

-	NWDAF requests ML Model from AIF;

-	NF profile in NRF includes list of ML Model ID(s).

* * * * End of Changes * * * *