3GPP TSG SA2 Meeting #142E (e-meeting) 	S2-2008533

16 – 20 November, 2020, Elbonia	(revision of S2-200xxxx)

- Source:	LG Electronics

- Title:	New WID: Architecture enhancements for 3GPP support of advanced V2X services - Phase 2

- Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Architecture enhancements for 3GPP support of advanced V2X services – Phase 2

## Acronym: eV2XARC_Ph2

## Unique identifier:

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_eV2XARC_Ph2 | SA2 | 850013 | Study on architecture enhancements for 3GPP support of advanced V2X services – Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720003 | Study on Enhancement of 3GPP support for V2X services (FS_eV2X) | Rel-15 Stage 1 study on eV2X service requirements |
| 750002 | Enhancement of 3GPP support for V2X scenarios (eV2X) | Rel-15 Stage 1 work on eV2X service requirements |
| 820024 | Improvement of V2X service Handling (V2XIMP) | Rel-16 Stage 1 work on eV2X service requirements |
| 760043 | Study on architecture enhancements for 3GPP support of advanced V2X services (FS_eV2XARC) | Rel-15/16 Stage 2 study item |
| 820018 | Architecture enhancements for 3GPP support of advanced V2X services (eV2XARC) | Rel-16 Stage 2 work item |
| 800096 | Study on NR Vehicle-to-Everything (FS_NR_V2X) | Rel-16 RAN study item |
| 830078 | 5G V2X with NR sidelink (5G_V2X_NRSL) | Rel-16 RAN work item |
| 860042 | NR Sidelink enhancement (NR_SL_enh) | Rel-17 RAN work item |

## 3	Justification

In Release 16, architecture enhancements to the 5G System were specified in TS 23.287 to facilitate vehicular communications for Vehicle-to-Everything (V2X) services, over the following reference points, based on service requirements defined in TS 22.185 and TS 22.186:

-	PC5 reference point: NR PC5 RAT, LTE PC5 RAT.

-	Uu reference point: NR, E-UTRA.

Interworking with EPS was also specified in Release 16. However, some of the requirements specified in TS 22.186 are not supported in Release 16 work.

Pre-Rel-17 V2X supports both the vehicle type of UE and pedestrian type of UE. However, Pre-Rel-17 V2X does not support functionality specific for the pedestrian type of UEs carried by pedestrian users, cyclists or etc. LTE PC5 only introduced a special resource selection mechanism (i.e. partial sensing or random selection only for transmission) for the pedestrian type of UE. Similar support is expected for NR PC5 for transmission. That is, there was no optimization for the pedestrian type of UEs for transmission and reception, which has power and computation limitations.

Therefore, the SA2 study (FS_eV2XARC_Ph2) has been progressed to investigate enhanced support of V2X operation for pedestrian UEs (i.e. UEs for Vulnerable Road Users), e.g. V2X communication with power efficiency, according to vehicular services requirements defined in TS 22.185 and TS 22.186.

## 4	Objective

The objective of this work item is to specify V2X communication over NR PC5 reference point with power efficiency for pedestrian UEs (i.e. UEs for Vulnerable Road Users) as per conclusions reached within TR 23.776 (clause 7).

The detailed objective is to support PC5 DRX operation.

Coordination with RAN WGs is expected in meeting the objective of this WID.

Editor’s Note: It is FFS whether power efficiency of LTE-V is considered, without the impact to RAN, e.g., support the application input from V2X application layer.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.287 | Add PC5 DRX operation | SA#92E |  |

## 6	Work item Rapporteur(s)

LaeYoung Kim, LG Electronics, laeyoung.kim@lge.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Coordination with RAN WGs.

## 9	Supporting Individual Members

Supporting IM name

LG Electronics

Qualcomm Incorporated

Deutsche Telekom

LG Uplus

SK Telecom

NTT DOCOMO

Telecom Italia

China Unicom

III

Hansung University

ITRI

ITL

ETRI

UANGEL

NEC

Volkswagen AG

ZTE

Fraunhofer HHI

Convida Wireless LLC

vivo Mobile Communication Co. LTD

AT&T

InterDigital

Apple

KT Corp.

China Mobile

KPN

Samsung

Alibaba

KDDI

Verizon UK Ltd

Orange

Intel

OPPO

Continental Automotive GmbH

Volvo Car Corporation

MediaTek Inc.

Huawei

HiSilicon

Tencent

Nokia

Nokia Shanghai Bell

Lenovo

Motorola Mobility