3GPP TSG-WG SA2 Meeting #143E	S2-2100827

Elbonia, November 16 – 20, 2020	(revision of S2-20xxxx)

Source:	Qualcomm Incorporated

Title:	Conclusion for Key issue #3

Document for:	Approval

Agenda Item:	8.10.1

Work Item / Release:	FS_MUSIM / Rel-17

Abstract: This contribution proposes the discussion and conclusion about Busy Indication.

# 1. Discussion

It is proposed to support NAS-triggered leaving procedure for 5GS.

# 2. Proposal

* * * * Start of Changes * * * *

## 8.3	Conclusions for Key Issue #3: Coordinated leaving for Multi-USIM device

Based on the evaluation in clause 7.3 the following interim conclusions are agreed for the baseline functionality:

-	For leaving in E-UTRA/EPS access, the NAS-level leaving MM procedure is recommended to be supported. The UE sends NAS MM message indicating leave request to releases the RRC-Connected state.

-	For leaving in NR/5GS access and E-UTRA/5GS access, the NAS-level leaving 5GMM procedure is recommended to be supported. The UE sends NAS 5GMM message indicating leaving request to release the RRC-Connected state.

-	When the UE connection with the network is resumed, the previous assistance info, if any, to temporarily restrict/filter MT data/signalling handling is revoked.

Editor's note:	It depends on RAN feedback on if and for which case the RRC-level leaving procedure is recommended to be supported for leaving in NG-RAN access

-	In the NAS MM message indicating leave request, the UE provides leaving indication to the CN and the UE may provide assistance information to the network in the leaving procedure regarding MT data/signalling handling.

-	The assistance information may include

-	Information to temporarily restrict/filter MT data/signalling handling:

-	An indication that the UE should only be paged for voice (MMTel voice or CS domain voice (for EPS)), or

-	An indication that the UE should not be paged at all, or

-	PDN connection(s) for MT notification/paging restriction (for EPC).

-	PDU session (s) for MT notification/paging restriction (for 5GC).

-	The assistance information is common for EPC and 5GC.

* * * * End of Changes * * * *