3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102232r01

Elbonia, April 12 – April 16, 2021	(revision of S2-210xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0266 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DCCF usage in non-roaming architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Note 2 in Clause 4.2.2 Network Functions and entities, TS 23,501, DCCF could be either standalone or hosted by NWDAF. It is not necessary to restrict standalone DCCF for non-roaming architecture in clause 4.2, TS 23.288 NOTE 2: The functionalities provided by DCCF and/or ADRF can also be hosted by an NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update “DCCF standalone function” to “DCCF” to allow usage of both standalone DCCF and DCCF hosted by NWDAF for non-roaming architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | only standalone DCCF used for non-roaming architecture and DCCF hosted by NWDAF can not be used non-roaming architecture |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of the changes ***

## 4.2	Non-roaming architecture

### 4.2.0	General

As depicted in Figure 4.2-1, the 5G System architecture allows NWDAF to collect data from any 5GC NF. The NWDAF belongs to the same PLMN as the 5GC NF that provides the data.

Figure 4.2-1: Data Collection architecture from any 5GC NF

The Nnf interface is defined for the NWDAF to request subscription to data delivery for a particular context, to cancel subscription to data delivery and to request a specific report of data for a particular context.

The 5G System architecture allows NWDAF to retrieve the management data from OAM by invoking OAM services.

The 5G System architecture allows NWDAF to collect data from any 5GC NF or OAM using a DCCF.

Figure 4.2-1a: Data Collection architecture using Data Collection Coordination

As depicted in figure 4.2-1a, the Ndccf interface is defined for the NWDAF to support subscription request(s) for data delivery from a DCCF, to cancel subscription to data delivery, and to request a specific report of data. If the data is not already being collected, the DCCF requests the data from the Data Source using Nnf services. The DCCF may collect the data and deliver it to the NWDAF or the DCCF may rely on a messaging framework to collect data from the NF and deliver it to the NWDAF.

As depicted in Figure 4.2-2, the 5G System architecture allows any 5GC NF to request network analytics information from NWDAF with Analytics logical function (AnLF). The NWDAF belongs to the same PLMN as the 5GC NF that consumes the analytics information.

Figure 4.2-2: Network Data Analytics Exposure architecture

The Nnwdaf interface is defined for 5GC NFs, to request subscription to network analytics delivery for a particular context, to cancel subscription to network analytics delivery and to request a specific report of network analytics for a particular context.

NOTE 1:	The 5G System architecture also allows other consumers such as OAM and CEF (Charging Enablement Function) to request network analytics information from NWDAF.

The 5G System architecture allows any NF to obtain Analytics from an NWDAF using a DCCF function and associated Ndccf services, as specified in clause 6.1.4.

As depicted in Figure 4.2-3, the 5G System architecture allows NWDAF to use model training services from another NWDAF containing Model Training logical function (NWDAF(MTLF)).

NOTE 2:	Analytics logical function and Model Training logical function are described in clause 5.1.

Figure 4.2-3: Model Training Exposure architecture

The Nnwdaf interface is used by NWDAF(AnLF) to request and subscribe to model training services.

NOTE 3:	The NWDAF training services are described in clause 7.

Editor's note:	Whether an NWDAF containing only MTLF can consume the model training is FFS.

The 5G System architecture allows any 5GC NF to request network analytics information from an NWDAF using a DCCF.

Figure 4.2-4: Network Data Analytics Exposure architecture using Data Collection Coordination

As depicted in Figure 4.2-4, the Ndccf interface is defined for any NF to support subscription request(s) to network analytics, to cancel subscription for network analytics, and to request a specific report of network analytics. If the analytics is not already being collected, the DCCF requests the analytics from the NWDAF using Nnwdaf services. The DCCF may collect the analytics and deliver it to the NF, or the DCCF may rely on a messaging framework to collect analytics and deliver it to the NF.

### *** End of the changes ***