3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102579r01

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2790 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support GERAN/UTRAN access in SMF+PGW-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Ericsson, China Unicom, Huawei, CATT, Nokia, Nokia Shanghai Bell, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_NIESGU |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current release, GERAN/UTRAN acccess to SMF+PGW-C is not supported. As a result, for policy and charging control, Gx interface is used for GERAN/UTRAN/E-UTRAN and N7 interface for E-UTRAN/NR combination respectively. Operators may want to simplify the network deployment and reduce the operational complexity of PGW-C/ SMF+PGW-C and of the interfaces between PGW-C and PCRF, and between SMF+PGW-C and PCF. With such consideration, the SMF+PGW-C needs to support the functionalities required for GERAN/UTRAN access, e.g. over N7 interfaces, the mapping from 5GS QoS information from PCF to EPS QoS information and then to R99 QoS information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An annex is added to describe the support of GERAN/UTRAN access in SMF+PGW-C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to simplify the network architecture and network management when 5GS is deployed and 2G or 3G still exists in their network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex YY (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR2672, TS23.503 CR 0559 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex YY (normative):
Support of GERAN/UTRAN access

This annex applies when the SMF+PGW-C is enhanced to support UE accessing the network via GERAN/UTRAN over Gn/Gp interface.

SMF+PGW-C selection by SGSN is specified in Annex YY in TS 23.502 [2].

The SMF+PGW-C interacting with PCF for GERAN/UTRAN access is specified in Annex YY in TS 23.502 [2].

The functional description for SMF+PGW-C interacting with PCF to support GERAN/UTRAN access is specified in TS 23.503 [3].

Charging aspects for GERAN/UTRAN access is specified in TS 32.255 [68].

NOTE 1: IP address preservation upon mobility between 5GS and GERAN/UTRAN is not supported, as described in clause 5.17.2.4.

NOTE 2: As described in clause 5.17.2.1, IP address preservation cannot be ensured when UE previously in GERAN/UTRAN moves from EPC/EUTRAN to 5GS or when UE previously in 5GS moves from EPC/EUTRAN to GERAN/UTRAN.

Charging aspects for GERAN/UTRAN access is specified in TS 32.255 [68].

# * * * * End of changes * * * *