3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102746r02

Elbonia, April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0317 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarificaiton about ML model sharing using DCCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2021-04-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 23.288, clause 7.1 and 7.5, it has been defined the ML Model provision service which enables the consumer to receive a notification when a ML model matching the subscription parameters becomes available. This service is defined between NWDAFs and one of the NWDAF contains the MTLF. If multiplie consumers request a same ML model, there are multiple notifications containing the same information due to uncoordinated requests from ML model consumers and it is proposed to allow ML model sharing using Data Collection Coordination to avoid ML model duplication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an EN to address necessity of DCCF feature extension for MTLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If multiplie consumers request a same ML model, there are multiple notifications containing the same information due to uncoordinated requests from ML model consumers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.2.0	General

As depicted in Figure 4.2-1, the 5G System architecture allows NWDAF to collect data from any 5GC NF. The NWDAF belongs to the same PLMN as the 5GC NF that provides the data.

Figure 4.2-1: Data Collection architecture from any 5GC NF

The Nnf interface is defined for the NWDAF to request subscription to data delivery for a particular context, to cancel subscription to data delivery and to request a specific report of data for a particular context.

The 5G System architecture allows NWDAF to retrieve the management data from OAM by invoking OAM services.

The 5G System architecture allows NWDAF to collect data from any 5GC NF or OAM using a DCCF standalone function, as specified in clause 5A.

Figure 4.2-1a: Data Collection architecture using Data Collection Coordination

As depicted in figure 4.2-1a, the Ndccf interface is defined for the NWDAF to support subscription request(s) for data delivery from a DCCF, to cancel subscription to data delivery, and to request a specific report of data. If the data is not already being collected, the DCCF requests the data from the Data Source using Nnf services. The DCCF may collect the data and deliver it to the NWDAF or the DCCF may rely on a messaging framework to collect data from the NF and deliver it to the NWDAF.

As depicted in Figure 4.2-2, the 5G System architecture allows any 5GC NF to request network analytics information from NWDAF with Analytics logical function (AnLF). The NWDAF belongs to the same PLMN as the 5GC NF that consumes the analytics information.

Figure 4.2-2: Network Data Analytics Exposure architecture

The Nnwdaf interface is defined for 5GC NFs, to request subscription to network analytics delivery for a particular context, to cancel subscription to network analytics delivery and to request a specific report of network analytics for a particular context.

NOTE 1:	The 5G System architecture also allows other consumers such as OAM and CEF (Charging Enablement Function) to request network analytics information from NWDAF.

The 5G System architecture allows any NF to obtain Analytics from an NWDAF using a DCCF function and associated Ndccf services, as specified in clause 6.1.4.

As depicted in Figure 4.2-3, the 5G System architecture allows NWDAF to use model training services from another NWDAF containing Model Training logical function (NWDAF(MTLF)).

NOTE 2:	Analytics logical function and Model Training logical function are described in clause 5.1.

Figure 4.2-3: Model Training Exposure architecture

The Nnwdaf interface is used by NWDAF(AnLF) to request and subscribe to model training services.

NOTE 3:	The NWDAF training services are described in clause 7.

Editor's note:	Whether an NWDAF containing only MTLF can consume the model training is FFS.

Editor’s note: It is FFS whether and how DCCF be the consumer of NWDAF (MTLF) services and provide ML model instead of MTLF without introducing MTLF hierarchy and cooperation.

The 5G System architecture allows any 5GC NF to request network analytics information from an NWDAF using a DCCF standalone function.

Figure 4.2-4: Network Data Analytics Exposure architecture using Data Collection Coordination

As depicted in Figure 4.2-5, the Ndccf interface is defined for any NF to support subscription request(s) to network analytics, to cancel subscription for network analytics, and to request a specific report of network analytics. If the analytics is not already being collected, the DCCF requests the analytics from the NWDAF using Nnwdaf services. The DCCF may collect the analytics and deliver it to the NF, or the DCCF may rely on a messaging framework to collect analytics and deliver it to the NF.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

* * * End of Change * * *