3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108477

# Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3247 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502: Time Sync exposure procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Call flow 4.15.9.2 needs correction to fix the right service operations and also updates needed to fix use of UDR for time sync exposure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the figure 4.15.9.2 to correct the service names with the ones in the procedure description and service operation (Nnef_TimeSynchronization_CapsSubscribe" vs "Nnef_TimeSynchronizationCaps_Subscribe") Also, fix the use of UDR Additionally, adding the consideration of time synchronization capabilities to clause 4.16.5.1 for PCF reporting PMIC/UMIC towards the TSCTSF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect service operation remains, also UDR use does not work could render the procedure wrong |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.2, 4.16.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.15.9.2	Exposure of UE availability for Time Synchronization service

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription for UE availability for time synchronization service. Cancelling is done by sending Nnef_TimeSynchronization_CapsUnsubscribe request identifying the subscription to cancel with Subscription Correlation ID.

Figure 4.15.9.2-1: Procedure for exposing 5GS and/or UE availability and capabilities for Time Synchronization services

1.	The PCF determines if existing PDU Sessions are potentially impacted by time synchronization service (based on local configuration or SM Policy Association). For each of these PDU Sessions, the PCF invokes Npcf_PolicyAuthorization service operation to the TSCTSF discovered and selected as described in clause 6.3.24 of TS 23.501 [2]

If time synchronization capability information from the DS-TT or NW-TT is available at the PCF, the PCF reports the capabilities to the TSCTSF invoking Npcf_PolicyAuthorization_Notify.

2.	The AF subscribes to the UE availability for time synchronization service and provides the associated Notification Target Address of the AF by sending Nnef_TimeSynchronization_CapsSubscribe request.

Report Type defines the type of reporting requested (e.g. one-time reporting, periodic reporting or event based reporting).

The request may include DNN and slicing information (S-NSSAI) and shall include an AF-Service-Identifier. If the DNN and S-NSSAI are omitted in the request, the NEF uses the AF-Service-Identifier to determine the target DNN and slicing information (S-NSSAI).

The Event Filter may include a list of UE identifiers (GPSIs) or Groups of UEs identified by an External Group Identifier that further define the subset of the target UEs. If the request does not include UE identifiers, the request is targeted to any UE with a PDU Session using the DNN and S-NSSAI. The NEF uses the UDM service to map the GPSIs to SUPIs and to map the External Group Identifier to Internal Group Identifier.

Additionally, the Event Filter may include one or more of the requested PTP instance type, requested transport protocol for PTP, or requested PTP Profile as described in Table 5.2.6.25.6-1.

When the NEF processes the AF request the AF-Service-Identifier may be used to authorize the AF request.

Depending on the AF-Service-Identifier and/or DNN/S-NSSAI, the NEF may reject the request if the the list of UE identities or External Group Identifier is not included in the request.

To remove an existing time synchronization service configuration, the AF invokes Nnef_TimeSynchronization_CapsUnsubscribe service operation and provides the Subscription Correlation ID.

3.	The NEF discovers the TSCTSF as described in clause 6.3.24 of TS 23.501 [2]. The NEF invokes the Ntsctsf_TimeSynchronization_CapsSubscribe request service operation to the selected TSCTSF.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF as described in clause 6.3.24 of TS 23.501 [2].

4.

(in the case of Ntsctsf_TimeSynchronization_CapsSubscribe): The TSCTSF identifies the affected AF session(s) based on the parameters received in step 3 and interacts with the PCF by triggering a Npcf_PolicyAuthorization_ Update request message.

(in the case of Ntsctsf_TimeSynchronization_CapsUnsubscribe): The TSCTSF uses the Subscription Correlation ID to determine the AF sessions and interacts with the PCF(s) by triggering a Npcf_PolicyAuthorization_Delete request message. Steps 8-13 are skipped.

5.	TSCTSF acknowledges the execution of Ntsctsf_TimeSynchronization_CapsSubscribe to the requester that initiated the request. The acknowledgement contains a Subscription Correlation ID that the requester can use to cancel or modify the subscription.

6.	NEF acknowledges the execution of Nnef_TimeSynchronization_CapsSubscribe to the requester that initiated the request. The acknowledgement contains a Subscription Correlation ID that the AF can use to cancel or modify the subscription.

.	 .

7.	As part of Npcf_PolicyAuthorization_Create/Update request, the TSCTSF uses the procedures as described in clause K.2.1 of TS 23.501 [2] to determine the (g)PTP capabilities from the DS-TT. If the TSCTSF has not determined the (g)PTP capabilities from the NW-TT, the TSCTSF determines the capabilities using the procedures as described in clause K.2.1 of TS 23.501 [2].

The TSCTSF composes the time synchronization capabilities for the DS-TT/UE(s) connected to the NW-TT based on the capability information received from the DS-TT(s) and NW-TT. If the Ntsctsf_TimeSynchronization_CapsSubscribe request include an Event Filter with one or more of the requested PTP instance type, requested transport protocol for PTP, or requested PTP Profile, the TSCTSF considers only the DS-TT(s) and NW-TT(s) with these capabilities as part of the time synchronization capability set that is reported to the NEF (or AF).

The TSCTSF maintains association between the user-plane Node ID, the time synchronization capabilities, the reference to the capabilities (as identified by the Subscription Correlation ID), the Event Filter (if available), the NEF or AF Notification Target Address, and list of the AF sessions with PCFs with this user-plane Node ID. If the Ntsctsf_TimeSynchronization_CapsSubscribe request includes one or more Event Filter(s), the TSCTSF considers only the matching UE identities and the DS-TT(s) and NW-TT(s) with the matching capabilities to be included in the associated AF sessions.

8.	The TSCTSF sends Ntsctsf_TimeSynchronization_CapsNotify (as described in clause 5.2.27.2.8) to the NEF. The message includes the time synchronization capabilities as composed in step 8. The message contains one or more user-plane Node ID(s) and a list of UE identifiers associated to each user-plane Node ID. The user-plane Node ID identifies the NW-TT to where the UE/DS-TT(s) are connected to.

9. The NEF sends Nnef_TimeSynchronization_CapsNotify with Time Synchronization capability event (as described in Table 5.2.6.25.8-1) to the AF.

10-11.	Upon PDU Session Establishment as defined clause 4.3.2.2.1-1, steps 1 and 7 are repeated for the new PDU Session.

12.	If necessary, the TSCTSF may update the time synchronization capabilities for the DS-TT/UE(s) connected to the NW-TT based on the capability information received from the new DS-TT. The TSCTSF sends Ntsctsf_TimeSynchronization_CapsNotify containing the updated capabilities and the UE identifier of the new PDU Session to the NEF.

13.	The NEF sends Nnef_TimeSynchronization_CapsNotify containing the updated capabilities and the UE identifier of the new PDU Session to the AF.

Table 5.2.6.25.6-1 describes the event for Time Synchronization capability. The event is detected when a UE/DS-TT establishes a PDU Session with an NW-TT, where DS-TT and NW-TT support time synchronization service as described in clause 4.4.8.3 in TS 23.501 [2].

# * * * * End of changes * * * *