3GPP TSG-SA2 Meeting # 148-E	S2-2108540r02

# Elbonia, 11 November 2021 - 19 November 2021			    	(Revision of S2-2107343)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3286 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time Synchronization of the 5GS as IEEE 1588 Boundary Clock |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the Release-16, the 5GS can work as a Time-aware system as descripted in IEEE 802.1AS, and the 5GS needs to support the time Synchronization as defined in clause 11.1.3 of IEEE 802.1AS, i.e. the 5GS reuses the same originTimestamp or the preciseOriginTimestamp in the generated Sync/Follow_Up messages and just updates the correction field with the 5GS residence time and link delay, which is defined in clause 5.27.1.2.2.1. However, in the Release-17, the 5GS can work as IEEE 1588 Boundary Clock and participates in best master selection and invokes the BMCA. In this Release-17, the 5GS operated as Boundary Clock needs to be a Master Clock in the Port with Master state and generates the Sync/Follow_Up messages based on its local PTP time based on IEEE 1588. But the 5GS can not synchronize its 5GS time to the external PTP GM clock, in such case, the 5GS can work mathematically and functionally equivalent as a PTP Boundary Clock that has synchonizted to the external PTP GM clock i.e. the 5GS updates the received originTimestamp or the preciseOriginTimestamp (for the two-step operation) with the received correction field and the 5GS residence time and link delay (all are expressed in PTP GM Clock) as the 5GS generates the Sync/Follow_Up message as the PTP Master Clock. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the GM is external, the operation of 5GS as Boundary Clock assumes that profiles that are supported by the 5GS allows the exemption specified in clauses 9.5.9 and 9.5.10 of IEEE Std 1588 [126] where the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is updated to the sum of the received originTimestamp (or preciseOriginTimestamp in case of two-step operation), the received correction field of Sync and Follow_Up (in case of two-step operation), residence time expressed in PTP GM time and link delay expressed in PTP GM time. After the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is updated, the correction field of Sync and Follow_Up (in case of two-step operation) can be set to zero. After the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is updated, the corretionField of Sync is set to zero. As described in clause 5.27.1.2.2, the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is updated with the correction field(s), 5GS residence time and link delay. A mathematically and functionally equivalent operation is specified for the time-aware system in clause 11.1.3 of IEEE Std 802.1AS [104]. If the 5GS operates as a Boundary Clock and the ingress TT is DS-TT, the UPF/NW-TT then regenerates the Sync and Follow_Up (for two-step operation) messages with the updated originTimestamp (or preciseOriginTimestamp) as described by the exemption in clause 5.27.1.1 based on the received Sync and Follow_Up messages for the PTP ports in Master state in NW-TT and DS-TT(s) within this PTP instance. The NW-TT/UPF forwards the regenerated Sync and Follow_Up messages to the PDU session(s) related to the Master ports in the DS-TT(s) within this PTP instance. Due to the exemption described in clause 5.27.1.1, when the PTP instance in 5GS is configured to operate as a Boundary Clock, the 5GS does not need to synchronize its Local PTP Clock to the external PTP grandmaster. The ingress TT in 5GS measures the link delay expressed in PTP GM time and updatethe correction field with the measured link delay expressed in PTP GM time. The egress TT converts the 5GS residence time into the residence time expressed in PTP GM time and updates the originTimestamp (or preciseOriginTimestamp) to the sume of the received originTimestamp (or preciseOriginTimestamp), the corrected field which has been updated by the ingress TT and the residence time expressed in PTP GM time. The egress TT sets the corrected field to zero for the Sync and Follow_Up message (for two-step operation). The externally observable behaviour of 5GS still conforms to the specifications for a Boundary Clock as described in IEEE Std 1588 [126]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS operating as an Boundary Clock can not work well and can not conform to the specifications for a Boundary Clock as described in IEEE Std 1588 [126]. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.1, 5.27.1.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST CHANGE

### 5.27.1	Time Synchronization

#### 5.27.1.1	General

For supporting time synchronization service, the 5GS is configured to operate in one of the following modes (if supported):

1)	as time-aware system as described in IEEE Std 802.1AS [104],

2)	as Boundary Clock as described in IEEE Std 1588 [126], provisioned by the profiles supported by this 3GPP specification including SMPTE Profile for Use of IEEE Std 1588 [126] Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127];

NOTE 1:	Via proper configuration of the IEEE Std 1588 [126] data set members, the 5G internal system clock can become the time source for the PTP grandmaster function for the connected networks in the case of mode 1 and mode 2.

NOTE 2:	In some cases where the 5G internal system clock is the time source for the PTP grandmaster function for the connected networks, it might not be required for the UE to receive gPTP or PTP messages over user plane. The UE and DS-TT uses the 5G timing information and generates the necessary gPTP or PTP message for the end station if needed (this is implementation specific).

3)	as peer-to-peer Transparent Clock as described in IEEE Std 1588 [126], provisioned by the profiles supported by this 3GPP specification including SMPTE Profile for Use of IEEE Std 1588 Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127]; or

4)	as end-to-end Transparent Clock as described in IEEE Std 1588 [126], provisioned by the profiles supported by this 3GPP specification including SMPTE Profile for Use of IEEE Std 1588 Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127].

NOTE 3:	When the GM is external, the operation of 5GS as Boundary Clock assumes that profiles that are supported by the 5GS allows the exemption specified in clauses 9.5.9 and 9.5.10 of IEEE Std 1588 [126] where the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is not required to be updated with the syncEventEgressTimestamp (and a Local PTP Clock locked to the external GM). As described in clause 5.27.1.2.2, the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is updated to the sum of the received originTimestamp (or preciseOriginTimestamp in case of two-step operation), the received correction field of Sync and Follow_Up (in case of two-step operation), residence time expressed in PTP GM time and link delay expressed in PTP GM time. After the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is updated, the correction field of Sync and Follow_Up (in case of two-step operation) is set to zero.

The configuration of the time synchronization service in 5GS for option 1 by TSN AF and CNC is described in clause 5.28.3, and for options 1-4 by AF and NEF or TSCTSF in clause 5.27.1.8 and clause 5.28.3.

The 5GS shall be modelled as an IEEE Std 802.1AS [104] or IEEE Std 1588 [126] compliant entity based on the above configuration.

The DS-TT and NW-TT at the edges of the 5G system may support the IEEE Std 802.1AS [104] or other IEEE Std 1588 [126] profiles' operations respective to the configured mode of operation. The UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil some functions related to IEEE Std 802.1AS [104] and may fulfil some functions related to IEEE Std 1588 [126], e.g. (g)PTP support and timestamping. Figure 5.27.1-1 illustrates the 5G and PTP grandmaster (GM) clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as PTP instance for supporting time synchronization

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the (g)PTP domain synchronization.

-	5G Clock synchronization: Used for NG RAN synchronization and also distributed to the UE. 5G Clock synchronization over the radio interface towards the UE is specified in TS 38.331 [28].

-	(g)PTP domain synchronization: Provides synchronization service to (g)PTP network. This process follows IEEE Std 802.1AS [104] or IEEE Std 1588 [126].

The two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

The 5GS supports two methods for determining the grandmaster PTP Instance and the time-synchronization spanning tree.

-	Method a), BMCA procedure.

-	Method b), local configuration.

This is further described in clause 5.27.1.6.

2ND CHANGE

5.27.1.2.2.2	Distribution of PTP Sync and Follow_Up messages

This clause applies if DS-TT and NW-TT support distribution of PTP Sync and Follow_Up messages. PTP support by DS-TT and NW-TT may be determined as described in clause K.2.1.

The mechanisms for distribution of PTP GM clock and time-stamping described in this clause are according to IEEE Std 1588 [126] for Transparent clock and for the case of Boundary clock when the GM is external, where the originTimestamp (or preciseOriginTimestamp) is updated by the 5GS as described by the exemption in clause 5.27.1.1. If the 5GS acts as the GM with a PTP instance type Boundary clock, then the 5GS updates the originTimestamp (or preciseOriginTimestamp in case of two-step operation) with the 5GS internal clock, as described in clause 5.27.1.7.

NOTE 1:	This means externally-observable behaviour of the PTP instance in 5GS needs to comply with IEEE Std 1588 [126].

Upon reception of a PTP event message from the upstream PTP instance, the ingress TT (i.e. NW-TT or DS-TT) makes an ingress timestamping (TSi) for each PTP event (i.e. Sync) message.

The PTP port in the ingress TT measures the link delay from the upstream PTP instance as described in clause H.4.

The PTP port in the ingress TT modifies the PTP message payload (carried within Sync message for one-step operation or Follow_Up message for two-step operation) as follows:

-	(if the PTP port in the ingress TT has measured the link delay) Adds the measured link delay from the upstream PTP instance in PTP GM time to the correction field.

-	(if the PTP port in the ingress TT has measured the link delay and rateRatio is used) Replaces the cumulative rateRatio received from the upstream PTP instance with the new cumulative rateRatio.

-	Adds TSi in the Suffix field of the PTP message as described in clause H.2.

NOTE 2:	If the 5GS is configured to use the Cumulative frequency transfer method for synchronizing clocks as described in clause 16.10 in IEEE Std 1588 [126], i.e. when the cumulative rateRatio is measured, then the PTP port in the ingress TT uses the cumulative rateRatio received inside the PTP message payload (carried within Sync message for one-step operation or Follow_Up message for two-step operation) to correct the measured link delay to be expressed in PTP GM time as specified in IEEE Std 1588 [126]. The PTP port in the ingress TT then calculates the new cumulative rateRatio (i.e. the cumulative rateRatio of the 5GS) as specified in IEEE Std 1588 [126].

NOTE 3:	If 5GS acts as an end-to-end Transparent Clock, since the end-to-end Transparent Clock does not support peer-to-peer delay mechanism, only the residence time spent within the 5GS in 5G GM time is used to update the correction field of the received PTP Sync or Follow_Up message.

The PTP port in the ingress TT then forwards the PTP message to the UPF/NW-TT. The UPF/NW-TT further distributes the PTP message as follows:

-	If the 5GS is configured to operate as Boundary Clock as described in IEEE Std 1588 [126], the UPF/NW-TT uses the port number of the receiving DS-TT (if the ingress TT is DS-TT), and domainNumber and sdoId in the received PTP message to assign the PTP message to a PTP instance in the NW-TT. The UPF/NW-TT then regenerates the Sync and Follow_Up (for two-step operation) messages with the updated originTimestamp (or preciseOriginTimestamp) as described by the exemption in clause 5.27.1.1 based on the received Sync and Follow_Up messages for the PTP ports in Master state in NW-TT and DS-TT(s) within this PTP instance. The NW-TT/UPF forwards the regenerated Sync and Follow_Up messages to the PDU session(s) related to the Master ports in the DS-TT(s) within this PTP instance.

-	If the 5GS is configured to operate as a Transparent Clock as described in IEEE Std 1588 [126], the UPF/NW-TT uses the port number of the receiving DS-TT (if the ingress TT is DS-TT), and domainNumber and sdoId in the received PTP message to assign the PTP message to a PTP instance in the NW-TT. The UPF/NW-TT then forwards the received Sync messages to PTP ports in DS-TT(s) within this PTP instance via all PDU Sessions terminating to this UPF, and to NW-TT ports within this PTP instance, except toward the ingress PTP port in the ingress TT.

NOTE 4:	If 5GS acts as a Transparent Clock, the NW-TT or DS-TT needs not to keep track of the PTP GM time. The 5GS does not maintain the PTP port states; the ingress PTP messages received on a PTP Port are retransmitted on all other PTP Ports of the Transparent Clock subject to the rules of the underlying transport protocol.

NOTE 5:	Due to the exemption described in clause 5.27.1.1, when the PTP instance in 5GS is configured to operate as a Boundary Clock, the 5GS does not need to synchronize its Local PTP Clock to the external PTP grandmaster. The ingress TT in 5GS measures the link delay expressed in PTP GM time and updatethe correction field with the measured link delay expressed in PTP GM time. The egress TT converts the 5GS residence time into the residence time expressed in PTP GM time and updates the originTimestamp (or preciseOriginTimestamp) to the sum of the received originTimestamp (or preciseOriginTimestamp), the correction field which has been updated by the ingress TT and the residence time expressed in PTP GM time. The egress TT sets the correction field to zero for the Sync and Follow_Up message (for two-step operation). The externally observable behaviour of 5GS still conforms to the specifications for a Boundary Clock as described in IEEE Std 1588 [126].

The PTP port in the egress TT then creates egress timestamping (TSe) for the PTP event (i.e. Sync) messages for external PTP network. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this PTP message expressed in 5GS time.

The PTP port in the egress TT then uses the rateRatio contained inside the PTP message payload (if available, carried within Sync message for one-step operation or Follow_Up message for two-step operation) to convert the residence time spent within the 5GS in PTP GM time.

The PTP port in the egress TT modifies the payload of the PTP message that it sends towards the downstream PTP instance as follows:

-	Adds the calculated residence time to the correction field (Sync message for one-step operation or Follow_Up message for two-step operation).

-	Removes Suffix field of the PTP message that contains TSi.

NOTE 6:	If 5GS acts as an end-to-end Transparent Clock, since the end-to-end Transparent Clock does not support peer-to-peer delay mechanism, only the residence time spent within the 5GS in 5G GM time is used to update the correction field of the received PTP event (e.g. Sync or Follow_Up) message.

END OF CHANGES