3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202046r04

Elbonia, April 6 – 12, 2022

# Title:	Reply LS on Mapped NSSAI

# Response to:	C1-222095 from CT1

# Release:	Rel-17

# Work Item:	5GProtoc17

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 3583

1. Overall Description:

SA2 would like to thank CT1 for their LS on Clarification on Mapped NSSAI.

Specific to the CT1 questions in C1-222095 / S2-2201936 SA2 would like to provide the following feedback:

Question 1: Could SA2 confirm that it is indeed optional for the VPLMN to provide the mapped S-NSSAI for VPLMN S-NSSAI 2?

Answer 1: Scenarios are identified where signaling could be optimized by not providing a mapped NSSAI however, it is understood the UE behaviors may differ in roaming scenarios should mapping information not be included. It is also understood any UE shall handle mapping information whenever mapping information is received by the UE.

Question 2: If the answer to question 1 is yes, could SA2 further clarify the conditions under which it is optional to provide the mapped S-NSSAI.

Answer 2: There are two cases in roaming scenarios:

- If the HPLMN S-NSSAI value is different comparing to the Serving PLMN S-NSSAI value, the Serving PLMN must provide the HPLMN S-NSSAI mapped to the Serving PLMN S-NSSAI to the UE.

- If the HPLMN S-NSSAI value is the same as the Serving PLMN S-NSSAI value, a signalling optimization could be had where the Serving PLMN could provide the Serving PLMN S-NSSAI to the UE without mapping information i.e. the corresponding mapped HPLMN S-NSSAI value. If the S-NSSAI provided to the UE in the Configured NSSAI or in the Allowed NSSAI does not have a corresponding mapping information (i.e. mapped HPLMN S-NSSAI vallue), the UE in this case could assume the same S-NSSAI value is used in the VPLMN as in the HPLMN.

Question 3: If the answer to question 1 is yes, could SA2 further clarify if it is mandatory for the network to provide the mapped S-NSSAI to the UE if the mapped S-NSSAI is known to the network?

Answer 3:  The Serving PLMN decides the mapping of the Serving PLMN S-NSSAI to HPLMN S-NSSAIs based on Service Level Agreement (SLA). There is no such scenario where the Serving PLMN does not know the mapping information for an S-NSSAI it provides to the UE in the Configured NSSAI or Allowed NSSAI. [Robert: I don't think this level of detail is needed for the reply LS.] Hence for all the S-NSSAIs in the Configured NSSAI and Allowed NSSAI either the Serving PLMN will provide a mapped value or the UE could assume that they have the same value as the S-NSSAI of HPLMN, as indicated in Answer 2.

A rejected S-NSSAI for the entire Serving PLMN is expected to be provided without a mapped HPLMN S-NSSAI value if the reason of rejection is that there is no mapping info available. However, if the reason of rejection is not due to unavailable mapping info, the Serving PLMN can provide the rejected VPLMN S-NSSAI value with the corresponding mapped HPLMN S-NSSAI value. In both cases, a new configured NSSAI is provided to the UE with the correct mapping for the S-NSSAIs the UE is expected to be able to request in the VPLMN.

Question 4: If the answer to question 3 is no, could SA2 further clarify how the UE determines the mapped S-NSSAI in that case?

Answer 4:  See above.

If CT1 determines that the above described SA2 logic is not what is assumed by CT1 and there is a risk of UE implementations behaving differently, then it is acceptable for SA2 to always provide the HPLMN S-NSSAI values i.e. mandate the logic for 5GC NFs in the serving PLMN.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E   	May 16 – 20, 2022 				Elbonia