3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204181

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for KI#5: Collection of nearby GNSS assistance data

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3/ Rel-18

Abstract: This solution enables LMF to get GNSS assistance data nearby 3GPP domain

1. Introduction

This solution is for the KI#5:

- How could the core network (i.e., LMF) get the nearby GNSS assistance data from 3GPP domain to mitigate a lack of implementation of GNSS reference receivers to help reduce power consumption and increase the positioning accuracy?

- Whether and how existing procedures, protocols or interfaces can be re-used or enhanced to make UE get the nearby GNSS assistance data?

2. Discussion

Nowadays, GNSS-based positioning is widely used outdoors. A terminal (e.g., a car or a mobile phone) equipped with a GNSS receiver can locate itself by receiving signals from GNSS satellites.

Usually, satellite-related error (e.g., orbit error, satellite clock bias), ionospheric delay, tropospheric delay, multipath error, electromagnetic noise in environment will cause GNSS positioning inaccuracy (normally an error of at least 5 to 10 meters), which is not acceptable for some industries such as autonomous driving. Besides, due to the limited transmission rate of GNSS signals, it usually takes more than 10 minutes for a terminal to get all related satellite information at the first time.

Figure.1: GNSS reference station providing GNSS assistance data

To improve positioning accuracy and decrease positioning delay, GNSS reference stations are deployed to provide assistance data to GNSS receivers. Based on its own measurements of the satellites and known precise coordinates, the reference station can calibrate the positioning error caused by the satellite, ionosphere and other factors and provide related calibration information to GNSS receivers. Besides, RTK differential technology can be further used to help GNSS receiver reach a centimetre or even millimetre level positioning accuracy. Currently, a UE usually obtains the GNSS assistance data from the Internet via a user plane connection.

Observation 1: In generally, GNSS reference stations are required to help improve GNSS positioning accuracy for outdoor UEs.

To achieve the accuracy of centimetres or even millimetres of GNSS positioning, the maximum distance between a UE and a GNSS reference station is required to not exceed a value (e.g., 30 km), which is usually much larger than the distance between gNBs. Thus, currently some network operators are deploying GNSS reference stations co-located with some gNBs to provide high-accuracy outdoor GNSS positioning service to UEs. Such kind of co-location can guarantee short distance between UE and GNSS reference stations and reduce costs of GNSS reference station deployment, as power source and transport network of RAN can be reused by GNSS reference stations.

Figure.2: GNSS reference station co-located with gNB

Observation 2: GNSS reference station can be co-located with gNB to reduce deployment costs and guarantee enough short distance between UE and GNSS reference station.

Besides deploying a third party GNSS reference station nearby RAN, the RAN network itself may support to provides GNSS assistance data by updating functionality of gNBs.

Another way to provide nearby GNSS assistance data is make GNSS reference stations act as UEs registered to 3GPP network and provide their GNSS assistance data to 5GC network.

To cover different cases, this solution provides three options for how LMF get nearby GNSS assistance data. For the GNSS reference stations co-located with gNBs case, a GNSS reference station can be regarded as a trusted AF or untrusted AF.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

6.x	Solution #x: Collection of nearby GNSS assistance data

6.X.1	Introduction

This solution addresses KI#5 about how LMF to obtain nearby GNSS assistance data.

6.X.2	Functional Description

Normally, a serving area of a GNSS reference station can cover one TA or multiple TAs (i.e., its GNSS assistance data applies to one TA or multiple TAs). When LMF collects GNSS assistance data, it requests corresponding data which can serve its serving area. The collection may be performed on a periodic basis.

With different GNSS reference deployment scenarios, how LMF obtain GNSS assistance data can be different. In this solution, following cases are considered:

- Case 1: GNSS reference stations are third party functions co-located with gNBs and LMF collects data from AFs.

LMF gets GNSS assistance data through AF event exposure service as described in clause 5.2.19.2 of TS 23.502, LMF may also get the precise global coordinates of GNSS reference stations. In this case, LMF is enhanced to support utilize the Naf_EventExposure service. LMFs collected GNSS assistance data may update its NF profile in NRF by indicating the TAs of which corresponding GNSS assistance data exists, which can be discovered and used by other LMFs not having corresponding GNSS assistance data.

- For a trusted AF, AF registers at the NRF with its serving area (e.g., TAs or geographical location). LMF discovers trusted AFs which can cover its serving area through NRF and subscribes to GNSS assistance data exposure from discovered trusted AFs.

- For a third-part AF, AF registers at the NEF with its serving area (e.g., geographical location) and NEF registers itself with the serving area information (mapped into TAs) to NRF. LMF discovers corresponding NEFs which can cover its serving area through NRF then subscribes GNSS assistance data through the discovered NEFs.

- Case 2: GNSS reference stations are gNBs.

LMF gets GNSS assistance data through NRPPa as described in clause 6.11.3 of TS 23.273, in this case RAN, LMF and NRPPa are enhanced to provide and transfer GNSS assistance data. LMF could be pre-configured with the information of gNB which supports GNSS assistance data provision. LMFs collected GNSS assistance data collection may update its NF profile in NRF by indicating the TAs of which corresponding GNSS assistance data exists, which can be discovered and used by other LMFs not having corresponding GNSS assistance data.

Editor’s Note: It is TSG RAN to decide whether to provide GNSS assistance data.

- Case 3: GNSS reference stations are UEs

GNSS reference station UE could be fixed, as it is deployed for specific area and mobility is not needed.

Its information consists of following aspects:

-  Capability: indicate the capability that the GNSS reference station UE could works as a GNSS reference station.

-  Location information: absolute location coordinates of the GNSS reference station UE. It could be configured in UE or LMF.

Option B.1 in solution#15 can be re-used for GNSS reference station UE information acquisition and management. After GNSS reference station UEs registered to LMF, LMF starts to collect GNSS assistance data from it via LPP periodically. When other LMFs needs use the GNSS assistance data for target UEs, they quires the GNSS reference station LMF through NRF or local configuration.

In this case, UE, LMF and LPP is enhanced to transfer GNSS assistance data.

6.X.3	Procedures

6.X.3.1a	GNSS assistance data collection from untrusted AF

The procedure in Figure 6.X.3.1a-1 is used by LMF to collect GNSS assistance information from AFs via the NEF, which is similar with DWDAF collecting data from untrusted AF described in clause 6.2.2.3 of TS 23.288.

Fig.6.X.3.1a-1. GNSS assistance data collection

1a-1c. Similar with step 1a-1c in figure 6.2.2.3-1 of TS 23.288. The difference is that AF should also provide its serving area to NEF. NEF should map the serving area into TA(s) if it is presented as geographical location and update the NEF profile in NRF including the serving area information.

1d-1e. Similar with step 1d-1e in figure 6.2.2.3-1 of TS 23.288. The difference is that LMF may additionally provide TAs to discover specific NEFs.

2-5. Similar with step 2-5 in figure 6.2.2.3-1 of TS 23.288. The difference is that besides available GNSS assistance data, AF further provide associated serving area of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data. NEF may map the serving area into a TA or TA list.

6.X.3.1b	GNSS assistance data collection from trusted AF

For trusted AFs, LMF should collect GNSS assistance data through AFs directly. LMF discovers trusted AFs covering LMF serving area through NRF and subscribes to GNSS assistance data exposure from discovered trusted AFs.

Fig.6.X.3.1b-1. GNSS measurements transfer1a-1c. AF registers at the NRF with its serving area (e.g., TAs).

1d-1e. LMF provide TAs to discover AFs.

2-3. LMF subscribes AF for GNSS assistance collection. Besides available GNSS assistance data, AF further provides associated serving area of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data.

6.X.3.2	GNSS assistance data collection from RAN

LMF gets GNSS assistance data through NRPPa as described in clause 6.11.3 of TS 23.273, in this case RAN, LMF and NRPPa are enhanced to provide and transfer GNSS assistance data. LMF could be pre-configured with the information of gNB which supports GNSS assistance data provision.

6.X.3.3	GNSS assistance data collection from GNSS reference station UE

Option B.1 in solution#15 can be re-used for GNSS reference station UE information acquisition and management. After GNSS reference station UEs registered to LMF, LMF starts to collect GNSS assistance data from it via LPP periodically.

6.X.4 Impacts on services, entities, and interfaces

LMF:

-  Support to collect and store GNSS assistance data from AF, RAN or UE.

-  When receive GNSS assistance data, send message to NRF to indicate GNSS assistance data existence in certain area

AF:

-  Support exposure of GNSS assistance data associated with geographical location.

NEF:

-  Support exposure of GNSS assistance data associated with geographical location from AF.

RAN:

-  Support to provide GNSS assistance data to LMF.

UE:

-  Support to provide GNSS assistance data to LMF.

NRF:

-  Support selection of LMF(s) that have registered with GNSS reference station UE/collected GNSS assistance data.

-	Support NF profile update once receive the GNSS reference station UE/GNSS assistance data existence indication.

# * * * * Second change * * * *