3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205552

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI#5, Sol#12: Proposal to remove EN on solution#12

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys/ Rel-18

Abstract: This contribution discussed on the charging aspects of KI#5 discussed in solution#12

# 1. Introduction/Discussion

The following Editor´s Note was added to Solution#12 that proposes a solution on the KI#5 on whether new charging mechanisms are required for identification and charging of Application AI/ML traffic:

Editor's note:	It is FFS whether the parameters that specifies the requested Application AI/ML traffic transmission are included in the Nnef_ChargeableParty_Create directly, or the parameters should be negotiated using a procedure similar to the Background Data Transfer policy negotiation and then a Reference ID can be used in the message.

The procedure to set the Chargeable party for an AF session enables the consumer to provide information about the Sponsor of the traffic between the ASP and the UE. The AI/ML Application Service Provider can sponsor the traffic with a UE to download an AI/ML model or can sponsor the traffic with a UE to generate the AI/ML model prior to its distribution. The Nnef_ChargableParty service can support these sponsoring.

It is also possible that the Background Data Transfer Reference ID is provided by the Sponsor to refer to a previously negotiated policy for background data transfer between the UE and the ASP. Given the performance requirements in terms of latency, bit rates and reliability that characterizes the AI/ML traffic, nothing indicates that a BDT Reference ID is applicable for this case. The proposal is to remove the Editor´s Note with an indication that BDT Reference ID is not provided when an AI/ML ASP sponsors the UE to ASP communication for download or generation of an AI/ML model, not supervision of the performance of the communication.

There is also a description on how the charging key and the measurement method is assigned in the PCC Rule, for example if the ML Model is grows in size, the PCF may change the charging rule from no charging to volume-based charging. That is under a certain model size, there will be no charges for the consumed volumes; they are included in the B2B contract, but if the AI/ML traffic grows in size, the PCF will request to perform measurements of the volume and a charging key will be assigned/changed.

The proposal to allocate a charging key based on the Service Experience or the DN Performance Analytics is removed, given that both Analytics IDs are generated based on AF provided information that most likely takes the user provided information as one of the input, one could think that reporting “low” service experience will trigger a change of the charging key to “no charging” or free of charge, that will in turn cause increase of traffic and really “low” service experience for majority of the users.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-80 v0.3.0.

# * * * * First change * * * *

## 6.12	Solution #12: Enhanced charging for Application AIML traffic transport

### 6.12.1	Description

This solution addresses the following aspect of key issue #5 "5GC Enhancements to enable Application AI/ML Traffic Transport":

-	Whether new 3GPP charging mechanisms are required for identification and charging of Application AI/ML traffic over the user plane that cannot be attributed or charged to the end user?

The Application AI/ML traffic, i.e. data or ML model for AI operations in application layer, can be transmitted as specific QoS flow(s) which is/are different from the QoS flow(s) used for common application data (i.e. non-AI/ML related data over the application layer). In this solution, the PCF may set both the sponsor id and the application service provider id in the PCC rules to provide information for the chargeable party, configures PCC rules and updates SM policy association to provide PCC Rule including how to charge for the Application AI/ML traffic .

NOTE:	Charging aspects will be done in collaboration with SA WG5.

### 6.12.2	Procedures

Figure 6.12.2-1 shows the procedure that the PCF performs to sponsor  Application AI/ML traffic.

Figure 6.12.2-1: Procedure of enhanced charging for Application AI/ML traffic

1.	An PDU Session is established/modified as specified in TS 23.501 [3]. The Application AI/ML traffic, i.e. data or ML model for AI operations in application layer, can be transmitted as specific QoS flow(s) in the PDU Session.

2.	The AF sends a Nnef_ChargeableParty_Create request message (AF Identifier, UE address, Flow description(s) or External Application Identifier, Sponsor Information, Sponsoring Status, DNN, S-NSSAI,) to the NEF, to request to become the chargeable party for transmission of Application AI/ML traffic. Step 1in the procedure of setting a chargeable party at AF session setup (if setted, change of the chargeable party during the session) as specified in clause 4.15.6.4 of TS 23.502 [4] is performed.

3.	The NEF may authorize the AF request based on operator configuration and sends a Npcf_PolicyAuthorization_Create request message to the PCF, which contains IP filter information, sponsored data connectivity information and Sponsoring Status (if received from the AF), as specified in step 2-3 of clause 4.15.6.4 of TS 23.502 [4].

4.

The PCF determines and/or updates the charging rules of Application AI/ML traffic transmission in PCC rules.

5.	The PCF sends a Npcf_PolicyAuthorization_Create response message to the NEF. The message contains the result indicating whether the request to become the chargeable party for Application AI/ML traffic transmission in the PDU Session is granted, and the granted charging rules.

6.	The NEF sends a Nnef_ChargeableParty_Create response message to the AF.

7.	The PCF sends an Npcf_SMPolicyControl_UpdateNotify to the SMF as specified in clause 4.16.5 of TS 23.502 [4], The PCF includes the latest SM policy information which contains the PCC rules and PCRTs to report usage

8.	According to the received PCRT from the PCF, the SMF determines the Usage Reporting Rule (URR) for transmitting the Application AI/ML traffic. The SMF triggers the procedures as described in TS 23.502 [4] to:

-	send the URR to the UPF for reporting the relevant usage information of Application AI/ML traffic. According to the reporting rule, the UPF collects the usage information of Application AI/ML traffic and reports it to the SMF.

-	send the information reported by the UPF with session and subscriber related information to the PCF and other charging function entities to perform enhanced charging for Application AI/ML traffic.

As result of the usage monitoring reporting to PCF, the PCF may decide to change the charging information:

-	Volume based charging.

The existing mechanism is used where different rates are applied according to the volume of ML Model of Application AI/ML traffic.

-	Time based charging.

The existing mechanism is used where different rates are applied according to the ML Model transmission time window of Application AI/ML traffic.

-	Volume and time based charging.

The existing mechanism is used where different rates are applied according to the weighted value of ML Model volume, ML Model transmission time window and network area information.

-	No charging.

### 6.12.3	Impacts on services, entities and interfaces

The mechanisms defined in the current specifications in TS 23.502 [4] and TS 23.503 [5] of charging for sponsored data connectivity with a chargeable party are reused for charging on Application AI/ML traffic with the clarification that BDT Reference ID is not applicable for this type of traffic, and with the following enhancements:

-	The PCF determines and/or updates charging information in the PCC rules using the volume or the time reported for transmission of AI/ML model related traffic.

For example, the ASP sponsors the traffic between the UE and the ASP up to 100 Mbytes, the B2B agreement indicates that the traffic is free of charge to the ASP up to 50 Mbps, the traffic is measured in volume and the charging key is assigned the corresponding rate when the volume exceeds 50 Mbps.

# * * * * End of changes * * * *